3GPP TSG-RAN WG2 Meeting #127	R2-2406701

Maastricht, Netherlands, 19 - 23, Aug, 2024

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	Discussion on LCM for UE-sided model for BM

Document for:	Discussion

Introduction

The following aspects are studied in LCM:

-	Data collection

-	Model training

-	Functionality/model identification

-	Model delivery/transfer

-	Model inference operation

-	Functionality/model selection, activation, deactivation, switching, and fallback operation.

-	Including: Decision by the network (either network initiated or UE-initiated and requested to the network), decision by the UE (event-triggered as configured by the network, UE’s decision reported to the network, or UE-autonomous either with UE’s decision reported to the network or without it)

-	Functionality/model monitoring

-	Model update

-	UE capability

In this contribution, we discuss the LCM for UE-sided model except for data collection and model transfer/delivery.

Discussion

Functionality identification

As first step, UE and NW need to perform functionality identification to reach common understanding on the functionality. During SI, it’s agreed each AI model is associated with meta data. It’s not clear whether functionality is associated with meta data. We understand the meta data is also needed for each functionality. Because NW/UE need to know some basic info about AI functionality to make LCM decision or determine whether AI functionality is applicable.

Observation 1: Meta data of functionality is essential for NW to make LCM decision and for UE to determine the applicable functionality.

Proposal 1: Each functionality is associated with meta data.

We would further discuss the content of meta data.

The basic info of AI functionality is the suitable use case, e.g. BM or Pos. Such info is important, since the input and output of use case is different.

Additional condition is introduced as any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Each functionality may be associated with one or multiple AI models. The AI functionality can achieve best performance in the condition same as training data. Therefore, it’s essential to include expected additional condition of AI functionality in the meta info. LCM can be performed accordingly, e.g. functionality switch/activation/deactivation.

To support the AI functionality, minimum AI capability may be required, e.g. computing capability or storage. However, the RAN node, especially UE, may not always have enough capability regarding computing or storage, especially if there are multiple on-going AI functionalities. It’s essential to know minimum AI capability required by AI functionality.

Proposal 2: Meta info of AI functionality includes following info,

- Expected use case;

- Expected NW additional condition;

- Minimum required computing and storage.

In last meeting, it’s agreed that supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. It’s not clear the granularity and number of functionalities. NW may not activate all supported AI functionalities. Similar issue exists in legacy capability report on band combination. NW can indicate the specific band/band combination and UE only reports the capability on corresponding band/band combination. We understand similar solution can be reused in AI capability report framework. The difference is AI functionality capability request may not per band/band combination. NW may request UE to report the capability of certain use case or additional condition. Existing message can be reused.

Proposal 3: NW can request UE to report supported AI functionality of certain use cases or additional condition to reduce the capability report signalling.

Applicable functionality report

In the email discussion, it’s aimed to merge the proactive and reactive report. Following signalling diagram is proposed,

Fig 1. Applicable functionality report flow

One open issue is whether it’s mandatory to provide NW side additional condition in step 3. We don’t see the reason to make it mandatory. NW may have provided current NW side additional condition to UE during data collection procedure. Even if it’s not provided, nothing is broken without NW side additional condition to support proactive or reactive report.

Proposal 4: it’s optional to provide NW side additional condition in step 3.

We can further discuss how to use NW side additional condition in step 3. We understand NW may provide two types of NW side additional condition for different purpose in step 3,

Type 1: Current NW side additional condition, which reflects current NW status. UE use this to determine whether a functionality is applicable.

Type 2: NW side additional condition for filtering, which may not reflect current NW status. Because NW may only be able to change the NW side additional condition among limited states. Only the functionalities whose expected NW side additional condition is align with the limited states are useful to NW. It’s a waste of signalling for UE to report other functionalities. Because NW would never activate these functionalities, since their expected NW side additional condition would never deployed by NW. Such filtering is useful to avoid UE to report the functionalities which NW would never activate.

Proposal 5: In step 3, NW can provide current NW side additional condition, which reflects current NW status. UE use this to determine whether a functionality is applicable.

Proposal 6: In step 3, NW can provide NW side additional condition for filtering, which may not reflect current NW status. UE only report the functionalities whose expected NW side additional condition is matched with.

Another open issue is whether full inference configuration is provided in step 3.

From procedure point of view, NW is not aware of which functionalities are applicable before step 5. If the full inference configuration provided in step 3, it’s possible the configuration can’t be applied by UE. In legacy, UE would trigger Reconfiguration failure and interruption is expected.

Observation 2: If UE can’t apply the full inference configuration provided in step 3, Reconfiguration failure would occur as in legacy.

If UE behaviour is enhanced to not trigger reconfiguration failure, this message shall be ignored. However, there may be other legacy RRC configuration carried in the same message. Legacy configuration would also be ignored, which may delay other procedures, e.g. measurement report. This is not preferred.

Observation 3: if UE ignore the RRCReconfiguration message instead of reconfiguration failure, other legacy RRC configuration in the same message would be ignored, which result in delay in other procedure.

According to the above observations, we don’t support to provide full inference configuration in step 3, before NW is aware of the applicable functionalities.

Proposal 7: In step 3, full inference configuration is not provided, if NW is not aware of the applicable functionalities.

After step 3, UE would check the functionality applicability. It’s agreed a functionality is applicable if all following conditions are met,

- NW side additional condition is met

- At least one model is available.

- UE side additional condition is met

Any condition not met would result in functionality not applicable. It’s not clear what is reported in step 4. At least UE shall report the applicable functionality. Furthermore, if a functionality is not applicable, it may be beneficial for NW to know the cause. Because NW may be able to enable the applicable functionality based on the cause to non-applicable. If NW side additional condition is not met, NW may change NW side additional condition. If model is not available, NW may transfer the model to UE, if capable, depending on the model delivery solution.

To facilitate the NW handling, UE shall report the cause of the functionality not applicable, i.e. NW side condition not met, UE side additional condition not met or no available model.

Proposal 8: In step 4, UE can report the non-applicable functionalities and the cause of non-applicable, i.e. NW side condition not met, UE side additional condition not met or no available model

In step 5, NW can provide the initial functionality configuration. One open issue is whether L1/L2 signalling for activation/deactivation is needed. The latency requirement of functionality activation/deactivation is not clear. If the latency requirement is not so strict, RRC based activation/deactivation is enough. We can wait for RAN1 progress on the latency requirement to decide whether it’s needed.

Proposal 9: In step 5, wait for RAN1 conclusion on the necessity of L1/L2 signalling for functionality activation/deactivation.

Conclusion

Based on the discussion in section 2, we have following proposals:

Functionality identification

Observation 1: Meta data of functionality is essential for NW to make LCM decision and for UE to determine the applicable functionality.

Proposal 1: Each functionality is associated with meta data.

Proposal 2: Meta info of AI functionality includes following info,

- Expected use case;

- Expected NW additional condition;

- Minimum required computing and storage.

Proposal 3: NW can request UE to report supported AI functionality of certain use cases or additional condition to reduce the capability report signalling.

Applicable functionality report

Proposal 4: it’s optional to provide NW side additional condition in step 3.

Proposal 5: In step 3, NW can provide current NW side additional condition, which reflects current NW status. UE use this to determine whether a functionality is applicable.

Proposal 6: In step 3, NW can provide NW side additional condition for filtering, which may not reflect current NW status. UE only report the functionalities whose expected NW side additional condition is matched with.

Observation 2: If UE can’t apply the full inference configuration provided in step 3, Reconfiguration failure would occur as in legacy.

Observation 3: if UE ignore the RRCReconfiguration message instead of reconfiguration failure, other legacy RRC configuration in the same message would be ignored, which result in delay in other procedure.

Proposal 7: In step 3, full inference configuration is not provided, if NW is not aware of the applicable functionalities.

Proposal 8: In step 4, UE can report the non-applicable functionalities and the cause of non-applicable, i.e. NW side condition not met, UE side additional condition not met or no available model

Proposal 9: In step 5, wait for RAN1 conclusion on the necessity of L1/L2 signalling for functionality activation/deactivation.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions
[2] TR 38.843