3GPP TSG-RAN WG2 Meeting #127	R2-2406702

Maaschitrict, 19 - 23, Aug, 2024

Agenda Item:	8.1.3

Source:	Xiaomi

Title:	Discussion on NW-side data collection

Document for:	Discussion

Introduction

For NW side model, data collection is essential to collect training data. In this contribution, we discuss NW-side data collection.

Discussion

Common procedure for BM and Pos

If the data is collected and reported by UE, data collection procedure can be divided to two steps. UE first initiates data collection and store data. Then, UE can report the data to NW based on report trigger. In the following sections, we would discuss data collection initiation and data report respectively.

Data collection initiation

Considering training data may be very large, data logging shall be supported, which means UE had started to collect data before report. One important aspect is how to initiate UE to perform data collection. The simplest way is NW can send explicit indication to start or stop data collection.

Proposal 1: Data collection start/stop can be indicated by NW indication.

However, NW may only need data UE collected under certain radio channel condition. Because model may be different under different radio channel condition. Radio channel may change due to UE movement and NW may need to frequent send indication to start or stop data collection. Event-based trigger can also reduce the signalling overhead.

Furthermore, UE may not be able to receive the NW indication when radio channel is poor. In AI mobility, UE may need to collect data at cell edge. The NW indication may not be reliable. In such case, event based data collection trigger may be beneficial. NW can configure the event before radio channel degrades. UE starts to collection when event is fulfilled.

Proposal 2: Data collection start/stop can be based on event fulfilment, which is configured by NW.

It may be difficult to train one model to apply in all NW additional conditions. This means models may be associated with different NW additional condition and require training data collected under different NW additional condition. Therefore, UE should store the NW additional condition associated with the collected data.

Proposal 3: UE shall store the NW additional condition when the data is collected.

Note the UE may have limited storage for training data collection. Logged training data may become very large after some time. UE shall be allowed to stop data collection if storage is exhausted, even if data collection is triggered.

Proposal 4: UE can stop data collection if storage is exhausted, even if data collection is triggered.

The data logging could reduce the number of reports. However, it would also consume UE storage, which increase the complexity and cost. It shall be optional for UE to support data logging.

Proposal 5: It’s optional for UE to support data logging for NW side data collection.

Even if UE supports the data logging, it’s possible that different UE chipset/vendor has different implementation. The supported storage size may be different. As indicated in [1], different type of training data may require different storage size. UE can report the supported data logging storage size to NW. NW can decide which data is to be stored by UE.

Proposal 6: UE can support different data logging storage size, which can be indicated by UE capability.

Data report

When UE reports the training data, UE shall also report the additional condition, under which the training data was collected. Because NW may use the training data under different additional conditions to train different models.

Proposal 7: UE report the collected training data and NW additional condition, under which training data was collected.

It’s FFS whether UE can report the collected data based on on-demand request.

We understand it’s necessary to support on-demand request. Because NW may only want to train/retrain certain AI model, which only requires training data collected in certain additional condition. Therefore, UE doesn’t need to report all collected data, which may cost lots of signalling. To avoid the signalling waste, UE can report collected training data info, e.g. data availability or collected data’s additional condition. NW can request UE to only report training data matched with certain additional condition.

Proposal 8: NW can request UE to only report data collected under certain NW additional condition.

Proposal 9: UE can report the data info, e.g. data availability or data associated NW additional condition to NW.

It’s FFS whether UE can report the collected data based on event. The event can be based on additional condition or other criteria. RAN2 can wait for RAN1 progress on the additional condition design. However, as discussed in previous section, UE may have limited storage. If the collected training data has exhausted the storage, UE shall trigger report. This can be discussed and agreed by RAN2.

Proposal 10: If the collected data has exhausted UE’s storage for data collection, UE shall trigger report.

After data report, UE can release the stored collected data. So that new data can be collected.

Proposal 11: UE releases the stored data after report.

It’s FFS how to handle case when single RRC message is not sufficient. We understand there may be following ways to handle/avoid this case,

Option 1: introduce data segmentation into multiple RRC message. NW has to combine multiple RRC message to derive the data. New layer or new function is needed. Spec impact may be huge.

Option 2: Up to UE implementation to send multiple RRC message to carry data. The difference between from 1 is Each RRC message carries intact data. NW doesn’t need to combine the multiple RRC messages to derive the data.

Option 3: UE trigger report if collected data reaches maximum payload of single RRC message is fulfilled, e.g. collected data reaches 144Kb. This solution can avoid data exceeds single RRC message maximum payload.

Option 1 is not preferred due to complexity. Option 2 can resolve this issue with no spec impact. Option 3 requires NW to configure appropriate threshold, which may be difficult. Therefore, option 2 is preferred.

Proposal 12: Up to UE implementation to send multiple RRC message to carry data, if single RRC message is not sufficient to carry all data.

NW-side data collection for Pos

(2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

(1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

For AI positioning, there are there use cases with the AI model deployed at Network side.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Note:	For gNB- and OAM-centric data collection, there may be a need to consult with RAN3 and SA5 whether/how OAM is to be involved.

Note:	For possible impacts due to positioning use cases, there may be a need to consult with RAN3 whether/how NRPPa is to be involved.

According to the TR 38.843 [1], we had made some conclusions on NW-sided data collection and Model training for positioning as below.

Model Training:

o	For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.

	Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

o	For LMF-side model, the LMF is the termination point for training data.

Based on above conclusion, we have some understandings on the data collection for positioning. For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection. However, if the termination point is the LMF, the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training. Since we haven’t agreed that the LMF is the termination point for training data in Case 3a, we won’t consider the LMF as the termination point for now.

For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

For Case 2b, the LMF is responsible for data collection, and the LPP protocol should be used to report the dataset from the UE to the LMF for model training.

Proposal 13: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 14: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 15: For case 2b, the LMF is responsible for data collection, and the LPP protocol should be used to report the dataset from the UE to the LMF for model training.

Conclusion

Based on the discussion in section 2, we have following proposals:

For both BM and Pos:

Proposal 1: Data collection start/stop can be indicated by NW indication.

Proposal 2: Data collection start/stop can be based on event fulfilment, which is configured by NW.

Proposal 3: UE shall store the NW additional condition when the data is collected.

Proposal 4: UE can stop data collection if storage is exhausted, even if data collection is triggered.

Proposal 5: It’s optional for UE to support data logging for NW side data collection.

Proposal 6: UE can support different data logging storage size, which can be indicated by UE capability.

Proposal 7: UE report the collected training data and NW additional condition, under which training data was collected.

Proposal 8: NW can request UE to only report data collected under certain NW additional condition.

Proposal 9: UE can report the data info, e.g. data availability or data associated NW additional condition to NW.

Proposal 10: If the collected data has exhausted UE’s storage for data collection, UE shall trigger report.

Proposal 11: UE releases the stored data after report.

Proposal 12: Up to UE implementation to send multiple RRC message to carry data, if single RRC message is not sufficient to carry all data.

For Pos:

Proposal 13: For Case 3a, if the termination point for training data is the gNB, there may be no specification impact on data collection.

Proposal 14: For Case 3b, the LMF is responsible for data collection, and the NRPPa protocol should be used to report the dataset from the gNB to the LMF for model training.

Proposal 15: For case 2b, the LMF is responsible for data collection, and the LPP protocol should be used to report the dataset from the UE to the LMF for model training.

Reference

[1] R2-2311720	Reply LS on Data Collection Requirements and Assumptions
[2] TR 38.843