3GPP TSG-RAN WG2 Meeting #127	                                          R2-2406703

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	Xiaomi

Title:  	Discussion on RRM prediction simulation result

Agenda Item:	8.3.2.1

Document for:	Discussion and Decision

Introduction

RAN2 agreed two goals for RRM prediction, i.e. measurement reduction and handover performance enhancement. Following cases are identified to evaluate the prediction accuracy.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we provide simulation result of cell level RRM measurement prediction of case 2 and 4. Some observations are given.

Discussion

FR1 to FR1 intra-frequency temporal domain case B

In this case, we simulate the 50% measurement reduction rate. In FR1, only 1 beam is assumed and beam level result is equal to L1 measurement results. According to the simulation assumption, UE can obtain 5 L1 measurement results during each L3 measurement period. Measurement pattern is depicted in following fig,

Fig 1. Measurement pattern

At each prediction time instance, e.g. time instance 2, measurement results at neighbor time instances, e.g. time instance 1 and 3, are used as AI input to predict.

For cell level measurement prediction model, RAN2 agreed to study the following prediction cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

If the cell results generation is part of the AI, we understand the method 1 is equal to case 2 and 3. So we did simulation for case 2 and 3 to evaluate the accuracy respectively.

The radio simulation parameters are shown as following,

| Parameters | Value |
| --- | --- |
| UE trajectory option (option 1,2,3 in[4]) | 3 |
| UE trajectory boundary processing option (option 1,2,3 in[4]) | rebound |
| UE speed (30,60,90 Km/h) | 30,60,90 km/h |
| Whether LOSsoft is modeled or not | No |
| spatial consistency option (A or B) | A |
| Number of TX beams | 1 |
| Number of RX beams | 1 |

To directly predict cell level results based on cell level results.

We agreed the L3 filter is applied in the simulation. In this case, the key question is whether the cell level results as input are L3 filtered or not.

If UE doesn’t perform actual measurement at prediction time instance, UE can’t obtain the L3 filtered cell level results at measurement time instance, too. Because the L3 filtering requires measurement result of current and previous time instance. The measurement result of prediction time instance is not available. In this case, the cell level results as input can only be non-L3 filtered.

Observation 1: UE can’t obtain the L3 filtered cell level results, if measurement is reduced.

If the model input is still L3 filtered cell level results, UE has to perform actual measurement at prediction time instance. However, the measurement is not reduced. It’s not aligned with the target study goal. Therefore, in our simulation, the cell level results as prediction input is before L3 filtered.

Proposal 1: To directly predict cell level results based on cell level results in temporal prediction case B, the cell level results as prediction input has to be before L3 filtered.

The AI simulation parameters are shown as following,

| Input/Output | Model input/Output data type | L3 Cell Level RSRP of past and next measurement period/L3 Cell Level RSRP |
| --- | --- | --- |
| Data Size (Number of Samples) | Training/validity | 9597/ 1200 |
|  | Testing | 1200 |
| Input size/Output size | (Batch size,Input_num,Feature_num) | (N,2,1)/ (N,1,1) |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM GRU |
|  | Model complexity in a number of parameters(/) | LSTM: 182,529 GRU: 137,745 |
|  | Model complexity in model size (e.g. Kbyte) (Model save format：h5) | LSTM: 2215 GRU: 1688 |

The accuracy simulation results are as following,

|  | RSRP difference (dB) |
| --- | --- |
| 30km/h | GRU: 1.678 LSTM: 1.721 |
| 60km/h | GRU: 1.690 LSTM: 1.677 |
| 90km/h | GRU: 1.769 LSTM: 1.764 |

To directly predict cell level results based on beam level results

The model input are L1 measurement results of past and next measurement period.

The AI simulation parameters are shown as following,

| Input/Output | Model input/Output data type | L1 beam Level RSRP of past and next measurement period /L3 Cell Level RSRP |
| --- | --- | --- |
| Data Size (Number of Samples) | Training/validity | 9597/ 1200 |
|  | Testing | 1200 |
| Input size/Output size | (Batch size,Input_num,Feature_num) | (N,10,1)/ (N,1,1) |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM GRU |
|  | Model complexity in a number of parameters(/) | LSTM: 335,217 GRU : 252,561 |
|  | Model complexity in model size (e.g. Kbyte) (Model save format：h5) | LSTM: 4004 GRU: 3033 |

The accuracy simulation results are as following,

|  | RSRP difference (dB) |
| --- | --- |
| 30km/h | GRU: 1.146 LSTM: 1.107 |
| 60km/h | GRU: 1.198 LSTM: 1.192 |
| 90km/h | GRU: 1.315 LSTM: 1.346 |

According to the results, different model has similar accuracy. In high speed, the accuracy is low compared with low speed. It’s reasonable since the radio channel may change rapidly in high speed.

Observation 2: Prediction accuracy decreases when UE speed increases in temporal domain prediction case B.

Proposal 2: Capture the above simulation results in TR for prediction case 2.

However, there may be another question to consider. The AI would result in delay to obtain the prediction result. Because the prediction requires the measurement result of next measurement period to be model input. If the measurement report is supposed to be triggered at the prediction time instance, the measurement report would also be delayed. If the measurement reduction rate is further improved, e.g. 75%, the delay is even longer. The measurement reduction may delay the measurement report and degrade handover performance. The actual impact can be evaluated during system performance evaluation.

FR2 to FR2 intra-frequency temporal domain case A

In temporal prediction case A, the L3 cell level measurement as model input can be L3 filtered. Because the prediction time instance is at future. Current and previous measurement result is available at measurement time instance.

Proposal 3: To directly predict cell level results based on cell level results in temporal prediction case A, the cell level results as prediction input can be after L3 filtered.

The radio simulation parameters are shown as following,

| Parameters | Value |
| --- | --- |
| UE trajectory option (option 1,2,3 in[4]) | 3 |
| UE trajectory boundary processing option (option 1,2,3 in[4]) | rebound |
| UE speed (30,60,90 Km/h) | 60 km/h |
| Whether LOSsoft is modeled or not | No |
| spatial consistency option (A or B) | A |
| Number of TX beams | 12 |
| Number of RX beams | 1 |

Considering the measurement period is 480 ms, prediction window length can be 1 or 2 measurement periods instances, which is enough to cover the TTT and handover delay. The observation window length can be 2 or 5 measurement periods.

The simulation result for 2-2 prediction is as following,

| Input/Output |  | Model input/output data type |  | L3 Cell Level RSRP of past two measurement period/L3 Cell Level RSRP of next two measurement |  |
| --- | --- | --- | --- | --- | --- |
| Data Size (Number of Samples) |  | Training/validity |  | 2163/ 270 |  |
|  |  | Testing |  | 270 |  |
| Input size/Output size |  | (Batch size,Input_num,Feature_num) |  | (N,2,1)/ (N,2,1) |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  | LSTM/GRU |  |
|  | Model complexity in a number of parameters(/) |  | LSTM:98,533 GRU : 74,461 |  |
|  | Model complexity in model size (e.g. Kbyte) (Model save format：h5) |  | LSTM: 1206 GRU:922 |  |
| Accuracy |  | RSRP difference (dB) |  | LSTM: 3.653 GRU:3.688 |  |

The simulation result for 5-1 prediction is as following,

| Input/Output |  | Model input/output data type |  | L3 Cell Level RSRP of past five measurement period/L3 Cell Level RSRP of next measurement period |  |
| --- | --- | --- | --- | --- | --- |
| Data Size (Number of Samples) |  | Training/validity |  | 1442/ 180 |  |
|  |  | Testing |  | 180 |  |
| Input size/Output size |  | (Batch size,Input_num,Feature_num) |  | (N,5,1)/ (N,1,1) |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  | LSTM/GRU |  |
|  | Model complexity in a number of parameters(/) |  | LSTM: 549,961 GRU : 413,785 |  |
|  | Model complexity in model size (e.g. Kbyte) (Model save format：h5) |  | LSTM: 6493 GRU: 4897 |  |
| Accuracy |  | RSRP difference (dB) |  | LSTM: 2.807 GRU:2.793 |  |

The simulation result for 5-2 prediction is as following,

| Input/Output |  | Model input/output data type |  | L3 Cell Level RSRP of past five measurement period/L3 Cell Level RSRP of next two measurement period |  |
| --- | --- | --- | --- | --- | --- |
| Data Size (Number of Samples) |  | Training/validity |  | 1236/ 154 |  |
|  |  | Testing |  | 154 |  |
| Input size/Output size |  | (Batch size,Input_num,Feature_num) |  | (N,5,1)/ (N,2,1) |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  | LSTM/GRU |  |
|  | Model complexity in a number of parameters(/) |  | LSTM: 549,961 GRU : 413,785 |  |
|  | Model complexity in model size (e.g. Kbyte) (Model save format：h5) |  | LSTM: 6493 GRU: 4897 |  |
| Accuracy |  | RSRP difference (dB) |  | LSTM: 3.866 GRU:3.925 |  |

According to the results, longer observation window would improve the prediction accuracy. However, the overall prediction accuracy should be further improved. Maybe longer observation window can be considered to improve the accuracy.

Observation 3: Prediction accuracy increases if longer observation window is used in temporal domain prediction case A.

Proposal 4: Capture the above simulation results in TR for prediction case 3.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: UE can’t obtain the L3 filtered cell level results, if measurement is reduced.

Proposal 1: To directly predict cell level results based on cell level results in temporal prediction case B, the cell level results as prediction input has to be before L3 filtered.

Observation 2: Prediction accuracy decreases when UE speed increases in temporal domain prediction case B.

Proposal 2: Capture the above simulation results in TR for prediction case 2.

Proposal 3: To directly predict cell level results based on cell level results in temporal prediction case A, the cell level results as prediction input can be after L3 filtered.

Observation 3: Prediction accuracy increases if longer observation window is used in temporal domain prediction case A.

Proposal 4: Capture the above simulation results in TR for prediction case 3.

Reference

[1]