3GPP TSG-RAN WG2 Meeting #127	R2-2406705

Maastricht, Netherlands, 19 - 23, Aug, 2024

Agenda Item:	8.3.4.1

Source:	Xiaomi

Title:	Discussion on RLF prediction assumptions

Document for:	Discussion

Introduction

A SI of AI mobility was agreed in [1]. It’s expected to run simulation to evaluate the benefit of AI mobility solutions. In this contribution, we discuss the RLF simulation and evaluation methodology.

Discussion

Simulation assumption

In last meeting, it’s agreed FR2 is prioritized. Beam management is introduced to enable UE to always serve on the best beam. However, we don’t expect to simulate the beam management procedure due to the complexity. To simplify the procedure, we can assume UE is always served by the best beam from serving cell. The SINR is derived based on serving beam.

Proposal 1: The best beam from serving cell is assumed to be the serving beam. SINR is derived based on the serving beam.

In last meeting, it’s agreed the study should focus on RLF due to T310 expiry. To trigger/stop the T310, wideband CQI shall be monitored. The modelling in [2] is showed as below,

For the purpose of RLF monitoring, the basic L1 processing configurations in non-DRX mode should be: L1 sample rate is once every 10ms (i.e. radio frame), with the L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin, respectively.

We understand UE derive the SINR on serving beam, so UE doesn’t need to sweep the beams. We can reuse the sample rate modelling as in LTE.

Proposal 2: For RLF monitoring, L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin, respectively.

If the SINR of all beams are below Qout threshold, OOC is indicated. If at least one of the beams is above Qin threshold, IC is indicted. Following parameters are used to determine RLF in [2],

Table 5.2.1.3.1: The parameters for determine the RLFs and the PDCCH failures. Items; Description. Qout; -8 dB. Qin; -6 dB. T310; 1s (the default value in 36.331). N310; 1. T311; Not used for calibration (since RLF recovery is not simulated in the calibration). N311; 1.

We understand the value of Qout, Qin, N310, N311 can be reused. But T310 may be set to shorter values in FR2 to adjust fast changing radio environment, e.g. 500ms. The exact value can be left to companies’ implementation.

Proposal 3: Reuse the parameters in table 5.2.1.3.1 in 36.839 to determine RLF. T310 can be set to shorter values.

RLF performance metric is modelled in [2] as following,

Definition 2: The RLF performance metric is defined as: the average number of RLF occurrences per UE per second.

The RLF performance metric can be reused.

Proposal 4: The RLF performance metric is defined as: the average number of RLF occurrences per UE per second.

Another mobility performance metric related to RLF is the interruption time. This metric is not defined in [2]. But it’s assumed the HO command is lost when T310 is running, which means the communication is interrupted. Therefore, we understand a straightforward definition of interruption time is the time when T310 is running. Furthermore, if RLF occurs, interruption would occur due to RRC reestablishment. Since we don’t simulate any RLF enhancement, a fixed interruption time can be assumed due to RLF.

Proposal 5: Interruption time includes the time when T310 is running.

Proposal 6: A fixed interruption time is assumed if RLF occurs.

Evaluation methodology

In last meeting, two prediction methods were agreed,

Direct prediction, AI directly predicts RLF occurrence.

Indirect prediction, AI predicts the SINR and UE determines RLF based on the predicted SINR.

In indirect prediction, AI predicts the SINR of each time instance. The IC or OOC status of each time instance can be determined based on Qin and Qout threshold. Therefore, the T310 running status of each time instance and RLF time instance can be predicted in indirect prediction.

Proposal 7: Indirect prediction can provide the T310 running status of each time instance and RLF time instance.

It’s agreed direct prediction can provide RLF probability within a time window or at time instance. However, if only probability is provided, it’s not clear how UE or NW can use this information. Also, it’s difficult to evaluate the accuracy if probability is provided. We understand AI shall provide the RLF occurrence or not within a time window or at time instance. The output should be a classification, i.e. RLF occurrence or not, not RLF probability.

Proposal 8: Direct prediction can provide the RLF occurrence or not with a time window or at time instance.

Based on the prediction result and ground truth, following cases may occur,

|  | Real RLF | Real No RLF |
| --- | --- | --- |
| Predict RLF | True positive | False positive |
| Predict No RLF | False negative | True negative |

We can use F1 score to evaluate the accuracy. If the output is a time window, the positive and negative can be defined by whether real RLF/no RLF within the time window.

Proposal 9: F1 score is used to evaluate the prediction accuracy.

Proposal 10: If the AI output is RLF/no RLF within a time window, the positive and negative can be defined by whether real RLF/no RLF occurs within the time window.

If the output is RLF/no RLF at a time instance, e.g. indirect prediction, the positive and negative can be defined whether real RLF/no RLF occurs at the time instance.

Proposal 11: If the output is RLF/no RLF at a time instance, e.g. indirect prediction, the positive and negative can be defined whether real RLF/no RLF occurs at the time instance.

According to above proposals, if real RLF and predicted RLF does not occur at the same time instance/window, it’s categorized as false positive. However, if the time difference between real RLF and predict RLF is very small. The prediction result may also be very useful, since NW may not wait to the last second to avoid RLF. Therefore, the time difference between real RLF and predict RLF can also be used to further evaluate the accuracy.

Proposal 12: Time difference between real RLF and predict RLF can also be used to further evaluate the accuracy.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: The best beam from serving cell is assumed to be the serving beam. SINR is derived based on the serving beam.

Proposal 2: For RLF monitoring, L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin, respectively.

Proposal 3: Reuse the parameters in table 5.2.1.3.1 in 36.839 to determine RLF. T310 can be set to shorter values.

Proposal 4: The RLF performance metric is defined as: the average number of RLF occurrences per UE per second.

Proposal 5: Interruption time includes the time when T310 is running.

Proposal 6: A fixed interruption time is assumed if RLF occurs.

Proposal 7: Indirect prediction can provide the T310 running status of each time instance and RLF time instance.

Proposal 8: Direct prediction can provide the RLF occurrence or not with a time window or at time instance.

Proposal 9: F1 score is used to evaluate the prediction accuracy.

Proposal 10: If the AI output is RLF/no RLF within a time window, the positive and negative can be defined by whether real RLF/no RLF occurs within the time window.

Proposal 11: If the output is RLF/no RLF at a time instance, e.g. indirect prediction, the positive and negative can be defined whether real RLF/no RLF occurs at the time instance.

Proposal 12: Time difference between real RLF and predict RLF can also be used to further evaluate the accuracy.

Reference

[1] RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

[2] TR 36.839