3GPP TSG-RAN WG2 Meeting #127											    R2-2406708

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	Xiaomi

Title:  	Event-based measurement reporting procedure for L1 measurement

Agenda Item:	8.6.4

Document for:	Discussion and Decision

# Introduction

According to the discussion on the event-triggered L1 measurement in the RAN2#126 and RAN2#125bis, RAN2 made the following agreements regarding the measurement reporting procedure:

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we provide our understandings on the measurement reporting procedure.

# Discussion

## MAC CE or UCI

According to the RAN2#126 meeting discussion, the following options are to be discussed for the even-triggered L1 measurement reporting:

- Option 1: MAC CE

- Option 2: UCI

According to the RAN1 agreements made for the MIMO discussion in the RAN1#116bis meeting, UCI in PUCCH is used to carry the measurement results of event-triggered L1 event of Rel-19 MIMO. According to the Rel-18 L1 measurement reporting procedure, the UCI is also used for the L1 measurement report of candidate cells. For Rel-18 LTM, the maximum number of reported candidate cell is 4. The maximum number of reported beams per cell is also 4. And the candidate value of M*L can be {1,2,3,4, 6, 8, 9, 12, 16} according to UE capability. It means UCI can support up to 16 reported beams in one report instance. Thus, the extra complexity of using MAC CE to provide the L1 measurement results are not needed.

Proposal 1: UCI is used to contain the measurement result.

## Report content

According to the L3 measurement event, the report quantity can be RSRP/RSRQ/SINR of L3 measurement results. According to 3GPP TS 38.305, the beam measurement quantity can also include RSRP/RSRQ/SINR. According to the RAN1 MIMO discussion for the L1 event-triggered measurement report, the measurement report quantity of L1-RSRP is used. From our understanding, RAN2 can assume that at least L1-RSRP can be used for the beam measurement result reported for the LTM measurement event.

Proposal 2: At least L1-RSRP is used for the beam measurement result reported for the LTM measurement event.

According to Rel-18 LTM, the gNB configures the number of reported candidate cells L and the number of reported beams for each candidate cell M. Then the number of beams in the beam report is fixed. And according to the agreements of Rel-19 MIMO, the number of reported beams is also configured by gNB. As such, we think that for the beam report of L1 LTM event, the gNB can also configure the number of reported beams. Regarding the details of the number of beams reported, we think that the Rel-18 configuration of LTM configuration can be reused. This means that the gNB configures the number of reported candidate cells and the number of reported beams for each candidate cell. On the other hand, we think that due to the blockage for some beam measurement, the UE should be given some freedom to choose the beams which can be reported. This means that the gNB can configure the total number of reported beams and the number of candidate cells, and the UE can decide the number of reported beams for each candidate cell.

Proposal 3: Regarding the number of reported beams triggered by the LTM measurement event, the following two schemes are supported:

- Scheme 1: The gNB configures the number of reported candidate cells and the number of reported beams for each candidate cell.

- Scheme 2: The gNB configures the total number of reported beams and the number of reported candidate cells. But UE decide the number of reported beams for each candidate cell.

With regard to whether the measurement results of beam reported should satisfy the LTM measurement event, we think that there is no need to require that the measurement result of each reported beam satisfies the LTM measurement event.

Proposal 4: At least one reported beam for LTM should satisfy the condition of the LTM measurement vent.

In addition, since the gNB can configure multiple events for the same beam, the UE should report the event ID in the beam report to differentiate the measurement event used for the measurement report.

Proposal 5: The LTM event ID is included in the measurement report.

## Reporting procedure

According to the RAN1 agreements, RAN1 agreed the following two Modes for the reporting procedure:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

  - Step 2: UE transmits the beam report in the second UL channel.

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

The intention of having two-steps reporting (i.e. Request + Report) procedure is to save the uplink resource reserved for the reporting. We think that LTM can also reuse the above two modes.

Proposal 4: For the report transmission procedure, the following two modes as agreed in Rel-19 MIMO are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

  - Step 2: UE transmits the beam report in the second UL channel.

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

## Trigger of new beam measurement

As the LTM procedure is to change the serving cell to another candidate cell, when the quality of the serving SpCell is still good, it may not be necessary for the UE to perform the measurement of the new beam. For the L3 measurement reporting procedure, s-MeasureConfig can be optionally configured to save the UE power, so that the UE only starts to measure the neighbouring cells when the serving cell quality is below the threshold configured by s-MeasureConfig. From our understanding, same mechanism as L3 s-MeasureConfig can be reused for even-triggered L1 measurement report.

Proposal 7: The UE performs the L1 measurement, only when the RSRP of the SpCell is below the threshold configured by RRC, same as the legacy s-MeasureConfig.

## Report on leave

Regarding whether to support the UE report when the leaving condition is satisfied, we think that the legacy procedure as used for the L3 measurement event procedure can be reused. We also consider that the “Report-on-Leave” function can be useful when the gNB wants to know the instant measurement results of the measurement event or wants to adjust the configuration of the measurement event.

Proposal 8: The gNB can configure the UE whether to report the measurement result of a LTM measurement event when the leaving condition is satisfied, same as the legacy reportOnLeave configuration.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: UCI is used to contain the measurement result.

Proposal 2: At least L1-RSRP is used for the beam measurement result reported for the LTM measurement event.

Proposal 3: Regarding the number of reported beams triggered by the LTM measurement event, the following two schemes are supported:

- Scheme 1: The gNB configures the number of reported candidate cells and the number of reported beams for each candidate cell.

- Scheme 2: The gNB configures the total number of reported beams and the number of reported candidate cells. But UE decide the number of reported beams for each candidate cell.

Proposal 4: At least one reported beam for LTM should satisfy the condition of the LTM measurement vent.

Proposal 5: The LTM event ID is included in the measurement report.

Proposal 6: For the report transmission procedure, the following two modes as agreed in Rel-19 MIMO are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

  - Step 2: UE transmits the beam report in the second UL channel.

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

Proposal 7: The UE performs the L1 measurement, only when the RSRP of the SpCell is below the threshold configured by RRC, same as the legacy s-MeasureConfig.

Proposal 8: The gNB can configure the UE whether to report the measurement result of a LTM measurement event when the leaving condition is satisfied, same as the legacy reportOnLeave configuration.

# Reference

[1] RAN2#125 meeting minutes

[1] RAN2#126 meeting minutes

# Annex

## RAN1 MIMO agreements for event-triggered L1 measurement

### RAN1#116 meeting agreement

Whole picture Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources Other procedure(s) as required

Events Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold. - Further study trigger events, including the following example as a starting point - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-3: Quality of a new beam is better than a certain threshold. - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2. - Others are not precluded. - Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Measurement resource Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP.

Signalling contents Agreement On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP - FFS: Study and decide whether additional contents can be supported. - FFS: L1-RSRP format, e.g., absolute and/or differential value. - Note: Above does not imply to preclude discussion on L1-RSRP filtering. - The actual reported content depends on the triggering event - Support of one or multiple events will be discussed separately

### RAN1#116bis meeting agreement

Signalling Medium Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

Event 2 Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Note: SSB or CSI-RS can be configured Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

Other event Agreement On UE-initiated/event-driven beam reporting, further study the following trigger events: - Event-1: Quality of the current beam is worse than a certain threshold. - Event-3: Quality of a new beam is better than a certain threshold. - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2. - Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold. - Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting). - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality. - Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam. - Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Report content Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117 - Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - At least one of N reported beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - FFS: Details on how value of N is determined by the UE - Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - Payload size does not vary as a function of N - FFS: Zero-padding can be provided if N is less than Nmax. - Option-2: Only N=1 beam is reported in the report instance - The reported beam should satisfy the condition of Event-2 - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - Other options are not precluded. - FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC. - FFS: When current beam is reported, whether the current beam is counted in the N reported beams. - The selected option shall satisfy Event-2.

### RAN1#117 meeting agreement

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case.

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH. - The UL-grant DCI format at least comprises DCI format 0_1/0_2. - FFS: DCI format 0_3 - FFS: How to trigger the UEI beam report by the UL-grant DCI format - FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. - The DL-grant DCI format at least comprises DCI format 1_1/1_2. - FFS: DCI format 1_3

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded.