3GPP TSG-RAN WG2 Meeting #127	R2-2406715

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	7.17.3

Source:	Huawei, HiSilicon

Title:	Discussion on configuration for temporary capability restriction

Document for:   Discussion and decision

Introduction

As specified in TS 38.331 [1], MUSIM UAI reporting for temporary capability restriction including reactive UAI and proactive UAI is configured via MUSIM-CapabilityRestrictionConfig-r18 that is included in OtherConfig as shown below:

OtherConfig-v1800 ::=                   SEQUENCE {

idc-AssistanceConfig-v1800              SetupRelease {IDC-AssistanceConfig-v1800}                     OPTIONAL, -- Need M

multiRx-PreferenceReportingConfigFR2-r18 SetupRelease {MultiRx-PreferenceReportingConfigFR2-r18}      OPTIONAL, -- Need M

aerial-FlightPathAvailabilityConfig-r18 ENUMERATED {true}                                             OPTIONAL, -- Need R

ul-TrafficInfoReportingConfig-r18       SetupRelease {UL-TrafficInfoReportingConfig-r18}              OPTIONAL, -- Need M

n3c-RelayUE-InfoReportConfig-r18        ENUMERATED {true}                                             OPTIONAL, -- Need R

successPSCell-Config-r18                SetupRelease {SuccessPSCell-Config-r18}                       OPTIONAL, -- Need M

sn-InitiatedPSCellChange-r18            ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapPriorityAssistanceConfig-r18   ENUMERATED {true}                                          OPTIONAL, -- Cond musimGapConfig

musim-CapabilityRestrictionConfig-r18   SetupRelease {MUSIM-CapabilityRestrictionConfig-r18}          OPTIONAL  -- Need M

}

MUSIM-CapabilityRestrictionConfig-r18 ::=    SEQUENCE {

musim-CandidateBandList-r18                  MUSIM-CandidateBandList-r18                            OPTIONAL, -- Need M

musim-WaitTimer-r18                          ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18                      ENUMERATED {ms0, ms10, ms20, ms40, ms60, ms80, spare2, spare1}

}

MUSIM-CandidateBandList-r18::= SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF FreqBandIndicatorNR

In this contribution, we discuss one issue related to the configuration for temporary capability restriction.

Discussion

The network can setup or release configuration for MUSIM temporary capability restriction via musim-CapabilityRestrictionConfig-r18. Once the configuration is setup, the MUSIM wait timer and MUSIM prohibit timer are mandatory to be configured. But the field musim-CandidateBandList-r18 is optional. If the musim-CandidateBandList-r18 is configured, it means UE is allowed to report MUSIM proactive UAI, i.e. the forbidden/affected combination of bands; otherwise, UE is not allowed to report MUSIM proactive UAI.

However, the need code for musim-CandidateBandList-r18 is “Need M”. This means that  if the network configures musim-CandidateBandList-r18 to enable MUSIM proactive UAI and later if the network intends to change the configuration to only MUSIM reactive UAI reporting (i.e., disable MUSIM proactive UAI but still keep the configuration for MUSIM reactive UAI), the network should first release the musim-CapabilityRestrictionConfig-r18 and then setup musim-CapabilityRestrictionConfig-r18 again with the field musim-CandidateBandList-r18 absent. Such procedure is inefficient and violate the original intention to define the need code of musim-CandidateBandList as “need M”. The root cause for such issue is an IE is specified without using SetupRelease structure but defined as Need M, which result in it cannot be released independently.

Observation 1: If the network has configured musim-CandidateBandList-r18 to enable MUSIM proactive UAI, and the network intends to change the configuration later to only enable MUSIM reactive UAI reporting (i.e., disable MUSIM proactive UAI but still keep the configuration for MUSIM reactive UAI), the network should first release the musim-CapabilityRestrictionConfig-r18 and then setup musim-CapabilityRestrictionConfig-r18 again with the field musim-CandidateBandList-r18 absent. Such procedure is inefficient and violate the original intention to define the need code of musim-CandidateBandList as “need M”.

Generally the network does not change the decision of configuration for MUSIM proactive UAI. But during handover case, it is natural that the configuration of the target node may be different from that of the source node (e.g., source node enables MUSIM proactive UAI but target node enables only MUSIM reactive UAI). For such case, as discussed above, the target node can only release the musim-CapabilityRestricitonConfig in the HandoverCommand message firstly, and after the UE complete the handover procedure, the target node needs to send another RRC reconfiguration message to configure the musim-CapabilityRestrictionConfig again without musim-CandidateBandList-r18.

To overcome the problem of NW implementation based procedure as discussed above, it is suggested to discuss the following options to handle the configuration for musim-CandidateBandList-r18:

- Change the need code for musim-CandidateBandList-r18 from “need M” to “need R”. This is a NBC change impacting ASN.1.

- Dummify the existing musim-CapabilityRestrictionConfig-r18 and add a new field musim-CapabilityRestrictionConfig-v18xy including “SetupRelease {MUSIM-CapabilityRestrictionConfig-v18xy}”. It is BC for ASN.1 decoding but may still be NBC if there is already implementation for “-v20” version.

- Introduce a new field indicating release of musim-CandidateBandList-r18 and an associated new capability. When the field musim-CapabilityRestrictionConfig-r18 is included in an RRC reconfiguration message, and if the musim-CandidateBandList-r18 is absent but new field is present, it implies release of musim-CandidateBandList-r18 rather than maintain it. The network can indicate new field only if the UE reports support of new capability. This is an BC change with introducing more signaling.

All the options above need ASN.1 change and may introduce inter-operability issue but considering this is still early, we think it is acceptable to have such kind of change.

Proposal 1: It is suggested to discuss the following options to handle the configuration for musim-CandidateBandList-r18:

- Change the need code for musim-CandidateBandList-r18 from “need M” to “need R”;

- Dummify the existing musim-CapabilityRestrictionConfig-r18 and add a new field musim-CapabilityRestrictionConfig-v18xy including “SetupRelease {MUSIM-CapabilityRestrictionConfig-r18}”;

- Introduce a new field indicating release of musim-CandidateBandList-r18 and an associated new capability. When the field musim-CapabilityRestrictionConfig-r18 is included in an RRC reconfiguration message, and if the musim-CandidateBandList-r18 is absent but new field is present, it implies release of musim-CandidateBandList-r18 rather than maintain it. The network can indicate new field only if the UE reports support of new capability. This is an BC change with introducing more signaling.

The TP for each option has also been provided in the Annex.

Conclusion

In this contribution, we have the following proposal and provide the TP for each of the option:

Proposal 1: It is suggested to discuss the following options to handle the configuration for musim-CandidateBandList-r18:

- Change the need code for musim-CandidateBandList-r18 from “need M” to “need R”;

- Dummify the existing musim-CapabilityRestrictionConfig-r18 and add a new field musim-CapabilityRestrictionConfig-v18xy including “SetupRelease {MUSIM-CapabilityRestrictionConfig-r18}”;

- Introduce a new field indicating release of musim-CandidateBandList-r18 and an associated new capability. When the field musim-CapabilityRestrictionConfig-r18 is included in an RRC reconfiguration message, and if the musim-CandidateBandList-r18 is absent but new field is present, it implies release of musim-CandidateBandList-r18 rather than maintain it. The network can indicate new field only if the UE reports support of new capability. This is an BC change with introducing more signaling.

References

- 3GPP TS 38.331 V18.2.0. NR; Radio Resource Control (RRC) protocol specification (Release 18).

TP for Options

TP for Option 1)

[Text Omitted]

MUSIM-CapabilityRestrictionConfig-r18 ::= SEQUENCE {

musim-CandidateBandList-r18               MUSIM-CandidateBandList-r18                                           OPTIONAL, -- Need R

musim-WaitTimer-r18                       ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18                   ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8,

s9, s10}

}

[Text Omitted]

TP for Option 2)

[Text Omitted]

RRCReconfiguration-v1800-IEs ::=        SEQUENCE {

needForInterruptionConfigNR-r18         ENUMERATED { disabled, enabled }                                   OPTIONAL, -- Need M

aerial-Config-r18                           SetupRelease { Aerial-Config-r18 }                             OPTIONAL, -- Need M

sl-IndirectPathAddChange-r18                SetupRelease { SL-IndirectPathAddChange-r18 }                  OPTIONAL, -- Need M

n3c-IndirectPathAddChange-r18               SetupRelease { N3C-IndirectPathAddChange-r18 }                 OPTIONAL, -- Need M

n3c-IndirectPathConfigRelay-r18             SetupRelease { N3C-IndirectPathConfigRelay-r18 }               OPTIONAL, -- Need M

otherConfig-v1800                           OtherConfig-v1800                                              OPTIONAL, -- Need M

srs-PosResourceSetAggBW-CombinationList-r18 SetupRelease { SRS-PosResourceSetAggBW-CombinationList-r18 }   OPTIONAL, -- Need M

ltm-Config-r18                              SetupRelease {LTM-Config-r18}                                  OPTIONAL, -- Need M

nonCriticalExtension                        RRCReconfiguration-v18xx-IEs                        OPTIONAL

}

RRCReconfiguration-v18xx-IEs ::=        SEQUENCE {

otherConfig-v18xx                           OtherConfig-v18xx                                              OPTIONAL, -- Need M

nonCriticalExtension                        SEQUENCE {}                                                    OPTIONAL

}

[Text Omitted]

[Text Omitted]

OtherConfig-v1800 ::=                   SEQUENCE {

idc-AssistanceConfig-v1800              SetupRelease {IDC-AssistanceConfig-v1800}                     OPTIONAL, -- Need M

multiRx-PreferenceReportingConfigFR2-r18 SetupRelease {MultiRx-PreferenceReportingConfigFR2-r18}      OPTIONAL, -- Need M

aerial-FlightPathAvailabilityConfig-r18 ENUMERATED {true}                                             OPTIONAL, -- Need R

ul-TrafficInfoReportingConfig-r18       SetupRelease {UL-TrafficInfoReportingConfig-r18}              OPTIONAL, -- Need M

n3c-RelayUE-InfoReportConfig-r18        ENUMERATED {true}                                             OPTIONAL, -- Need R

successPSCell-Config-r18                SetupRelease {SuccessPSCell-Config-r18}                       OPTIONAL, -- Need M

sn-InitiatedPSCellChange-r18            ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapPriorityAssistanceConfig-r18   ENUMERATED {true}                                          OPTIONAL, -- Cond musimGapConfig

dummy   SetupRelease {Dummy-MUSIM-CapabilityRestrictionConfig-r18}          OPTIONAL  -- Need M

}

OtherConfig-v18xy ::=                   SEQUENCE {

musim-CapabilityRestrictionConfig-v18xy    SetupRelease {MUSIM-CapabilityRestrictionConfig-v18xy}          OPTIONAL  -- Need M

}

IDC-AssistanceConfig-v1800 ::=          SEQUENCE {

idc-FDM-AssistanceConfig-r18            SetupRelease {IDC-FDM-AssistanceConfig-r18}                   OPTIONAL, -- Need M

idc-TDM-AssistanceConfig-r18            ENUMERATED {setup}                                            OPTIONAL  -- Cond FDM

}

MultiRx-PreferenceReportingConfigFR2-r18 ::= SEQUENCE {

multiRx-PreferenceReportingConfigFR2ProhibitTimer-r18  ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

MUSIM-GapAssistanceConfig-r17 ::= SEQUENCE {

musim-GapProhibitTimer-r17        ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10}

}

MUSIM-LeaveAssistanceConfig-r17 ::=     SEQUENCE {

musim-LeaveWithoutResponseTimer-r17     ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1}

}

Dummy-MUSIM-CapabilityRestrictionConfig-r18 ::= SEQUENCE {

musim-CandidateBandList-r18               MUSIM-CandidateBandList-r18                                           OPTIONAL, -- Need M

musim-WaitTimer-r18                       ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18                   ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8,

s9, s10}

}

MUSIM-CapabilityRestrictionConfig-v18xy ::= SEQUENCE {

musim-CandidateBandList-v18xy             SetupRelease {MUSIM-CandidateBandList-r18}                 OPTIONAL, -- Need M

musim-WaitTimer-r18                       ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18                   ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8,

s9, s10}

}

MUSIM-CandidateBandList-r18::= SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF FreqBandIndicatorNR

[Text Omitted]

TP for Option 3)

TP for 38.331

[Text Omitted]

RRCReconfiguration-v1800-IEs ::=        SEQUENCE {

needForInterruptionConfigNR-r18         ENUMERATED { disabled, enabled }                                   OPTIONAL, -- Need M

aerial-Config-r18                           SetupRelease { Aerial-Config-r18 }                             OPTIONAL, -- Need M

sl-IndirectPathAddChange-r18                SetupRelease { SL-IndirectPathAddChange-r18 }                  OPTIONAL, -- Need M

n3c-IndirectPathAddChange-r18               SetupRelease { N3C-IndirectPathAddChange-r18 }                 OPTIONAL, -- Need M

n3c-IndirectPathConfigRelay-r18             SetupRelease { N3C-IndirectPathConfigRelay-r18 }               OPTIONAL, -- Need M

otherConfig-v1800                           OtherConfig-v1800                                              OPTIONAL, -- Need M

srs-PosResourceSetAggBW-CombinationList-r18 SetupRelease { SRS-PosResourceSetAggBW-CombinationList-r18 }   OPTIONAL, -- Need M

ltm-Config-r18                              SetupRelease {LTM-Config-r18}                                  OPTIONAL, -- Need M

nonCriticalExtension                        RRCReconfiguration-v18xx-IEs                        OPTIONAL

}

RRCReconfiguration-v18xx-IEs ::=        SEQUENCE {

otherConfig-v18xx                           OtherConfig-v18xx                                              OPTIONAL, -- Need M

nonCriticalExtension                        SEQUENCE {}                                                    OPTIONAL

}

[Text Omitted]

[Text Omitted]

OtherConfig-v1800 ::=                   SEQUENCE {

idc-AssistanceConfig-v1800              SetupRelease {IDC-AssistanceConfig-v1800}                     OPTIONAL, -- Need M

multiRx-PreferenceReportingConfigFR2-r18 SetupRelease {MultiRx-PreferenceReportingConfigFR2-r18}      OPTIONAL, -- Need M

aerial-FlightPathAvailabilityConfig-r18 ENUMERATED {true}                                             OPTIONAL, -- Need R

ul-TrafficInfoReportingConfig-r18       SetupRelease {UL-TrafficInfoReportingConfig-r18}              OPTIONAL, -- Need M

n3c-RelayUE-InfoReportConfig-r18        ENUMERATED {true}                                             OPTIONAL, -- Need R

successPSCell-Config-r18                SetupRelease {SuccessPSCell-Config-r18}                       OPTIONAL, -- Need M

sn-InitiatedPSCellChange-r18            ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapPriorityAssistanceConfig-r18   ENUMERATED {true}                                          OPTIONAL, -- Cond musimGapConfig

musim-CapabilityRestrictionConfig-r18   SetupRelease {MUSIM-CapabilityRestrictionConfig-r18}          OPTIONAL  -- Need M

}

OtherConfig-v18xy ::=                   SEQUENCE {

musim-CandidateBandListReleaseInd-r18         ENUMERATED {true}                                       OPTIONAL, -- Cond musimBandList

}

[Text Omitted]

[Rows Omitted]

musim-CandidateBandList A list of candidate bands that the network intends to use, e.g., for serving cells and for which the UE is requested to provide information on temporary restricted capabilities for MUSIM operation as specified in clause 5.7.4.3.

musim-CandidateBandListReleaseInd If present, the field indicates the UE to release the configured musim-CandidateBandList. This field is not present simultaneously with musim-CandidateBandList.

musim-GapAssistanceConfig Configuration for the UE to report assistance information for gap preference.

[Rows Omitted]

| Conditional Presence | Explanation |
| --- | --- |
| FDM | This field is optionally present, need M, if idc-AssistanceConfig-r16 or idc-FDM-AssistanceConfig is setup. Otherwise, it is absent, need R. |
| maxBW | This field is optionally present, need R, if maxBW-PreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| maxMIMO | This field is optionally present, need R, if maxMIMO-LayerPreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| minOffset | This field is optionally present, need R, if minSchedulingOffsetPreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| musimGapConfig | This field is optionally present, need R, if musim-GapAssistanceConfig-r17 is configured; otherwise it is absent, need R. |
| musimBandList | This field is optionally present, need N, if musim-CandidateBandList-r18 is configured; otherwise it is absent. |
| SCG | This field is optionally present, need M, in an RRCReconfiguration message not within mrdc-SecondaryCellGroup and received, either via SRB3 within DLInformationTransferMRDC or via SRB1. Otherwise, it is absent. |

[Text Omitted]

UE-NR-Capability-v1800 ::=               SEQUENCE {

airToGroundNetwork-r18                   ENUMERATED {supported}                                       OPTIONAL,

eRedCapParameters-r18                    ERedCapParameters-r18                                        OPTIONAL,

ncr-Parameters-r18                       NCR-Parameters-r18                                           OPTIONAL,

softSatelliteSwitchResyncNTN-r18         ENUMERATED {supported}                                       OPTIONAL,

hardSatelliteSwitchResyncNTN-r18         ENUMERATED {supported}                                       OPTIONAL,

mt-SDT-r18                               ENUMERATED {supported}                                       OPTIONAL,

mt-SDT-NTN-r18                           ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndAutonomousDenial-r18      ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndFDM-r18                   ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndTDM-r18                   ENUMERATED {supported}                                       OPTIONAL,

musim-GapPriorityPreference-r18          ENUMERATED {supported}                                       OPTIONAL,

musim-CapabilityRestriction-r18          ENUMERATED {supported}                                       OPTIONAL,

multiRx-FR2-Preference-r18               ENUMERATED {supported}                                       OPTIONAL,

ra-InsteadCG-SDT-r18                     ENUMERATED {supported}                                       OPTIONAL,

resumeAfterSDT-Release-r18               ENUMERATED {supported}                                       OPTIONAL,

ul-TrafficInfo-r18                       ENUMERATED {supported}                                       OPTIONAL,

aerialParameters-r18                     AerialParameters-r18                                         OPTIONAL,

--R4 40-2: beam steering

ntn-VSAT-AntennaType-r18                 ENUMERATED {electronic, mechanical}                          OPTIONAL,

--R4 40-1: VSAT UE type in NTN

ntn-VSAT-MobilityType-r18                ENUMERATED {fixed, mobile}                                   OPTIONAL,

ntn-Parameters-v1820                     NTN-Parameters-v1820                                         OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v18xy                                                   OPTIONAL

}

UE-NR-Capability-v18xy ::=               SEQUENCE {

musim-CapabilityCandidateBandListRelease-r18    ENUMERATED {supported}                                OPTIONAL,

nonCriticalExtension                            SEQUENCE {}                                           OPTIONAL

}

[Text Omitted]

TP for 38.306

4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [Rows Omitted] |  |  |  |  |
| musim-CapabilityRestriction-r18 Indicates whether the UE supports providing MUSIM assistance information with temporary capability restriction and capability restriction indication (i.e., musim-CapRestrictionInd), as defined in TS 38.331 [9]. For a UE supporting nr-NeedForGap-Reporting-r16, this field also indicates UE supports providing musim-NeedForGapsInfoNR-r18 with temporary capability restriction as defined in TS 38.331 [9]. | UE | No | No | No |
| musim-CapabilityCandidateBandListRelease-r18 Indicates whether the UE supports releasing the candidate band list from the current MUSIM capability restriction configuration, as defined in TS 38.331 [9]. | UE | No | No | No |
| [Rows Omitted] |  |  |  |  |