3GPP TSG-RAN WG2 Meeting #127	R2-2406716

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda Item:	8.2.4

Source:	Huawei, HiSilicon

Title:	A-IoT random access procedure

Document for: Discussion and Decision

1	Introduction

In RAN2#126 agreement, the agreements for A-IoT random access were achieved as shown below:

Agreements on “4 step” RA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information. - A-IoT Msg2: the reader echos the ID receved in Msg1. Further information may be included in mgs2 based on RAN1 agreements. - A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request). - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed Agreement From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, the details of A-IoT random access procedure are further discussed.

2	Discussion

2.1	Msg1 time-domain resource

2.1.1	Msg1 time-domain access occasion definition/indication

In RAN1 meeting, RAN1 agreed to study TDMA and FDMA for D2R transmission:

RAN1#116-bis agreement [1]: Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

For FDMA, the detailed configurations of frequency for A-IoT Msg1 are out of RAN2 scope and which are pending to RAN1.

Observation 1:	The frequency-domain access resource definition/indication for A-IoT Msg1 will be discussed by RAN1.

For TDMA, as discussed in [2], the timing accuracy of A-IoT device could be extremely low, which cannot work well in an accurately synchronous system by using legacy methods in 3GPP. Hence, the NR RACH occasion with accurate time boundary cannot be used in A-IoT.

Observation 2:	A-IoT device is not able to support the NR RACH occasion defined by the absolute timing, due to its extremely low timing accuracy.

With the above new design assumptions, RAN1#117 meeting basically agreed on the point that one R2D message will just provide scheduling for one D2R transmission for one device.

Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it:

Proposal 1:	RAN2 agrees that the reader transmits one explicit R2D message to define/indicate the start/boundary of the access occasion (instead of defining the NR RACH occasion by absolute timing).

2.1.2	Msg1 time-domain resource selection

The reader can allocate multiple occasions to distribute in time-domain the devices in case multiple devices are likely to collide in the same occasion. The reader indicates the number of access occasions in one access round by a R2D message. Then devices randomly select an access occasion as their corresponding access occasion to access. For example, in Slotted ALOHA, the parameter Q is sent from the reader to indicate the number of access occasions, and a device randomly selects a number in range [0, 2Q-1] as its access occasion. A Q-like parameter can be included in a R2D message, e.g. Access Round Indication message.

Proposal 2a:	One access round consists of multiple access occasions.

Proposal 2b:	The total number of time-domain access occasions within one access round is indicated by the reader.

Proposal 2c:	A-IoT device randomly selects one access occasion among the multiple time-domain access occasions in an access round.

2.1.3	A-IoT paging and random access relationship

In last RAN2#126[1], there were some discussions on what information can be indicated in the AIoT paging message, and it has been agreed:

- AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

It is clarified in the SID that the traffic types to be studied are DO-DTT and DT, and periodical system information is not supported. Once the A-IoT devices are paged by reader, an R2D configuration for the following A-IoT random access procedure or D2R data transmission is needed. Reader should provide some common parameters in the R2D configuration, from which A-IoT devices can determine the random access occasion and other transmission parameters. The R2D configuration at least includes the number of access occasions of one access round and the detail of the configurations are pending to RAN1 decision.

The R2D configuration can be either sent in a separated message or contained in A-IoT paging message. As one example, for the very first initial trigger message, the R2D configuration can be carried in A-IoT paging message. For the following transmissions, if another access round needs to be triggered for the failed devices in the previous round, the configuration in a separated R2D message can be used without including the whole paging identifier, which has the benefit of less overhead.

An example is given in the figure 2.1-1, the configuration (e.g. number of access occasions) can be indicated not only by the initial paging transmission and retransmitted paging message, but also by separately messages such as the Query message.

Figure 2.1-1:	the relationship between paging and random access

Observation 3:	As one modelling, both A-IoT paging message and access round message (Query) can start/initiate an access round (including multiple access occasions).

Proposal 3:	The R2D configuration, to indicate the number of access occasions of one access round, can be either sent in a separate R2D message or combined into A-IoT paging message.

2.2	Msg1/Msg2 content

2.2.1	Common design part

Terminologies alignment:

|  | Key point/TR terms | RAN2 agenda terms | Previous RAN2 agreement terms |
| --- | --- | --- | --- |
| Solution 1 | Msg1 without data | 3-steps RA | 4-step like RA |
| Solution 2 | Msg1 with data | 2-step RA | 2-step like RA |

In the last RAN2 meeting, the basic procedure for “3-step” CB RA and “2-step” CB RA were discussed, and the following agreement was reached on A-IoT Msg1 and A-IoT Msg2:

Agreements on “4 step” RA 1 A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements Agreements on 2 step CB RA 1 A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed

As analysed in our previous contribution [6], a device randomly selects an access occasion and generates a random ID for A-IoT Msg1 transmission, if reader successfully receives A-IoT Msg1, it replies A-IoT Msg2 which is used for contention resolution. For example, if the total number of access occasions is 64, the total number of candidate frequency resources is 4, the random ID in A-IoT Msg1 is assumed as 16-bits (like RN16 used by RFID) and the total number of device is 250, the probability that two devices select the same access occasion, frequency resource and random ID is 1-(1-164×4×216)250-1 (i.e. almost 0.0015%), which is extremely low. Thus, 16-bit random ID in A-IoT Msg1 is enough for contention resolution in A-IoT Msg2.

Observation 4: 	The failure of contention resolution probability (when 16-bits random ID is used) is already very low (e.g. 0.0015%).

To have a more intuitive understanding, we further discuss the possible number of devices in a collided occasion. As shown in the following table, X is the number of devices that select the same occasion, and P(X) is the probability that X devices select the same occasion. Assuming N devices select among M access occasions, P(X) is:

PX=NX1MXM-1MN-X,

where NX is the combination, calculated as NX=N!X!(N-X)!.

In Slotted ALOHA, many existing papers illustrated that the theoretical optimal efficiency of Slotted ALOHA is about 36.8% [5], and the optimal efficiency can be achieved when the number of slots equals the number of devices. The Table 2.2.1-1a shows the analysis result based on the optimal efficiency criterion, where M=N.

Table 2.2.1-1a	Probability P(X) in the case of N=M

| X | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P(X) (%), M=200, N=200 | 36.70 | 36.88 | 18.44 | 6.12 | 1.51 | 0.30 | 0.049 | 0.0068 | 0.00082 |
| P(X) (%), M=1000, N=1000 | 36.77 | 36.81 | 18.40 | 6.13 | 1.53 | 0.30 | 0.051 | 0.0072 | 0.00089 |

From the above results, it is observed that more than 99.9999% of occasions is selected by less than 8 devices in the case of M=N and obviously, while the percentage is much larger when in the M>N case. That means, the probability that more than 8 devices selecting one access occasion is almost zero/extremely low.

According to the above analyses, even considering the non-ideal factors, a 16-bit random ID in Msg1 should be sufficient for resolving the contention. In most cases, the collided occasion only contains 2 or 3 devices, so an 8-bit random ID may be also acceptable.

Observation 5a:	In the Slotted ALOHA mechanism, about 2~3 devices selecting the same access occasion will be the typical case (normally less than 8 devices). Therefore, 16-bit random ID in Msg1 should be sufficient for resolving the contention (i.e. contention resolution failure probability will be about 1216), among those limited number of devices selecting the same occasion.

Observation 5b:	Reader will allocate suitable/similar number of access occasions according to the total number of devices to be triggered. Therefore, the number of devices selecting the same occasion won’t increase according to the total number of devices.

On the other hand, more bits in Msg1 will increase inventory latency. We compare the inventory latency of 16-bit and 32-bit random ID in Msg1 of 3-step RA procedure, and the detailed description of evaluation is in Appendix. The comparison is shown in the following Table.

Table 2.2.1-2	Performance results on latency of different sizes of Msg1, where M=N

| N | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latency of 3-step RA, 16-bit random ID (seconds) | 69.1 | 62.7 | 55.5 | 48.6 | 41.9 | 34.9 | 27.8 | 20.7 | 13.7 | 6.91 | 5.25 | 3.4 |
| Latency of 3-step RA, 32-bit random ID (seconds) | 88.9 | 79.8 | 71.0 | 62.1 | 53.3 | 44.4 | 35.4 | 26.7 | 17.7 | 8.81 | 6.66 | 4.38 |
| Latency increase (%) | 27.5 | 27.3 | 27.9 | 28.0 | 27.2 | 27.0 | 27.3 | 29.1 | 29.8 | 27.5 | 26.8 | 26.6 |

According to the above result, the inventory latency using a 32-bit random ID is increased 27% compared to using a 16-bit random ID. Hence, from the overhead and latency perspective, a 32-bit random ID in Msg1 are not suggested.

Observation 6: 	Considering the inventory latency, the 32-bit random ID may increase by about 27% the inventory latency, compared to the 16-bit random ID case.

In RFID, a 16-bit random number (i.e.RN16) is used for contention resolution, which works well in a real commercial scenario. It can be a direct and important reference to the design of A-IoT Msg1.

In addition, even though some devices select the same access occasion with the same random ID regardless of low possibility, it can still be handled by the reader. After receiving the A-IoT Msg2 from the reader, the devices will send the A-IoT Msg3, i.e. the device ID. Since the device IDs are unique among all the devices, consequently, the reader may receive a garbled message or multiple device IDs. Then the reader is able to detect there is a collision issue involving same random ID in A-IoT Msg1. The reader can handle it as an Msg3 transmission failure case (refer to Sec 2.4.2 in this paper), and the reader sends a failure indication to trigger the devices to re-access.

Observation 7: 	Similar to NR, there is no need to absolutely avoid the contention resolution failure (like zero probability resolution failure), considering that it is always possible to allow the re-access to handle any rare case failure.

So, based on the above discussion, we have the following proposal:

Proposal 4:	The 16-bit is sufficient for the random ID in A-IoT Msg1 (i.e. not larger than 16bits).

For A-IoT Msg2, the reader echoes the information from A-IoT Msg1, e.g. random ID. In TDMA case, once the reader successfully decodes an A-IoT Msg1, it replies A-IoT Msg2 to the device sending this A-IoT Msg1. If the Msg2 replies to multiple devices, it increases the complexity for device to process the R2D message multiplexing. Moreover, it requires the device to use a relatively large timer (like RAR timer) for the Msg2 reception, which additionally add complexity to the device side. Thus, in TDMA case, A-IoT Msg2 does not include multiple acknowledgements to the A-IoT Msg1 of multiple devices.

On the other hand, when applying FDMA to A-IoT Msg1, multiple devices are allowed to send FDMed Msg1 in the same time-domain access occasion. In this case, it can be further discussed whether A-IoT Msg2 can include multiple acknowledgements to A-IoT Msg1s of multiple devices for one time-domain occasion with multiple frequencies.

Proposal 5: 	As the baseline, in TDMA case, A-IoT Msg2 does not include multiple acknowledgments to multiple devices’ A-IoT Msg1 (FFS for FDMA case).

2.2.1	2-step RA design

In RAN2#126:

Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed

For 2-step CBRA, Msg2 is used for acknowledging the successful Msg1 transmission (and contention resolution). For this purpose, some information from Msg1, such as identification information, can be echoed in Msg2 to identify the specific device. In a 2-step CBRA procedure, a device that sends Msg1 considers the Msg1 transmission/contention resolution as successful, if Msg2 including the same (partial) information in Msg1 is received.

In our understanding, regardless of the above case, the Msg2 at least needs to include the relative unique identification information and to distinguish different devices for contention resolution. Therefore, in the following, we consider the case where Msg1 includes Device ID as an example, and the analysis results will be similar for cases where Msg1 includes any other upper-layer data. Whether Msg1 in 2-step CBRA/Msg3 in 3-step RA includes any other upper layer data is pending to the conclusion of the general AS procedure.

From the perspective of statistics, the randomness of ID in Msg2 to echo the Msg1 depends on the information length. The more bits from device ID included in Msg2, the less collision probability of Msg2 will be (if two or more devices select the same access occasion and FDMA is not considered). For example, the probability that two N-bit binary numbers are exactly the same is 1/2N, if those bits are randomly generated. Therefore, a larger value of N results in a lower collision probability of Msg2 content.

Observation 8:	In 2-step CBRA, as to the identification information in Msg2 used for content resolution:

The more bits it includes, the less efficiency;

The less bits it includes, the more collision it may suffer.

The case that Msg2 includes the whole Device ID is first discussed as follows:

In this case, the total size of Msg1+Msg2 equals {2* size of Device ID} in 2-step CBRA. In 3-step RA, the total size of Msg1+Msg2+Msg3 equals {size of random ID + size of random ID + size of Device ID}. Note that one of the agreements in RAN2#126 is “Msg4 (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access”, so we do not consider the “Msg4” of 3-step RA in the comparison.

As per the above analysis, 16-bit random ID in Msg1 is sufficient because it results in an extremely low probability of failure of contention resolution. Meanwhile, Device ID is typically larger than 32 bits (2*random ID). Taking EPC as an example, the EPC length can range from 96 to 496 bits according to EPC Tag Data standard [4]. This means the signalling overhead would equal 192~992 bits for 2-step CBRA and 128~528 bits for 3-step RA, respectively.

Hence, if the Msg2 includes the entire Device ID, the total signalling overhead of 2-step CBRA would be significantly greater than that of 3-step RA, resulting in a 1.5 to 1.8 times increase.

Observation 9:	For 2-step CBRA, if the Msg2 includes the whole device ID for contention resolution, the total size of Msg1+Msg2 will be even larger than the sum of Msg1+Msg2 +Msg3 of 3-step CBRA.

Observation 10a:	For 2-step CBRA, A-IoT Msg2 should not include the whole device ID.

Another approach of reducing the overhead of 2-step CBRA is to echo the partial Device ID in Msg2. The case that Msg2 includes partial Device ID is then discussed as follows:

Even though the whole Device ID is unique, segments (partial Device ID) of different Device IDs may be the same. Device(s) will mistakenly consider the contention resolution as successful, if two or more devices select the same access occasion (without FDMA) and receive the same acknowledge information from Msg2. To prevent this misunderstanding, Msg2 needs to be unique to a certain extent, reducing conflict probability of Msg2.

The randomness of the partial Device ID cannot depend only on the length of the ID echoed in Msg2. It is also affected by the specific applications, e.g., the design/encoding of Device ID. Some parts in Device ID may be common for different devices with the same attribute, such as those owned by the same company. For instance, in RFID systems, Electronic Product Code (EPC) is a unique identifier for a tag. As defined in EPC Tag Data Standard (TDS) [3], certain types of information in the EPC are the same for different tags owned by the same entity, and there are various EPC encoding schemes. For example, CAGE (Commercial and Government Entity) identifies the company from which the tag is owned, and the same company’s tags have the same CAGE. Hence, such CAGE-like identifiers cannot be used for contention resolution in Msg2, as they provide only a limited degree of randomness (e.g., distinguishing different companies) while introducing additionally invalid overheads.

Hence, increasing the Msg2 size will not always reduce the conflict probability, especially if the common parts of Device ID are included. To avoid adding more unnecessary overheads, Msg2 needs to include the unique portion of Device ID. However, that information cannot be controlled by a reader or visible to a reader, since the encoding schemes of Device ID are typically defined by CN and/or a third party and are transparent to reader.

Based on the above analysis, it is not random enough for contention resolution if Msg2 only includes a short partial device ID. It needs more bits than a purely random number. Compared to the 16-bit requirement of a random ID mentioned above, Msg2 with partial device ID needs to use more bits for unambiguous contention resolution. For example, at least about 32-bit partial device ID needs to be included in Msg2.

Observation 10b:	For 2-step CBRA, if A-IoT Msg2 only includes the partial device ID, it may not be random enough for ambiguous contention resolution, i.e. at least about 32 bits are needed.

In 2-step CBRA, whether random ID in Msg1 is needed is FFS:

Agreements on 2 step CB RA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information

As analysed above, the device ID or partial device ID is not appropriate for contention resolution in A-IoT Msg2. Thus, an additional random ID is needed in Msg1. Like 3-step CBRA, a random ID and the device ID and/or any other upper layer data can be sent in the A-IoT Msg1. Once the reader receives the A-IoT Msg1 from the device, it echoes the random ID in A-IoT Msg2 for contention resolution, which achieves the tradeoff between overhead and randomness.

In addition, a device can be triggered for the later data transmission, e.g., a command procedure after successful transmission of Device ID. Then, the reader needs an AS scheduling ID to address the specific device. Obviously, the random ID in Msg1/Msg2 can be continuously reused as the AS scheduling ID. In this case, there is no need to introduce a new ID for scheduling, which reduces the device implementation complexity and transmission overhead.

Observation 11:	The random ID in Msg1 can be reused as the AS scheduling ID for later data transmission, if any.

Proposal 6:	For 2-step CBRA, the random ID is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.

2.2.3	2-step vs. 3-step RA down-selection

In RAN2#126 agreement, the agreements for “3-step RA” were achieved as shown below:

Agreements on “4 step” RA - IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information. - IoT Msg2: the reader echos the ID receved in Msg1. Further information may be included in mgs2 based on RAN1 agreements. - IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request). - The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. - “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

Compared to 3-step RA, 2-step RA requires a multiplexing processing in Msg1, i.e., multiplexing the random ID (an AS payload) and the data (upper-layer payload, e.g., NAS-like or application-layer payload). It needs a cross-layer processing when generating Msg1, causing additional processing complexity.

Observation 12:	The D2R multiplexing in Msg1 (including both device ID and random ID) will cause some processing complexity.

Though 2-step CBRA can save one step than 3-step RA (Msg4 does not need to be always sent in 4-step RA), the efficiency will not be better than 3-step RA. As described in Sec 2.4.3 (re-access), once Msg1 collides, Msg1 needs to be retransmitted in the later access occasion. In 2-step CBRA, Device ID, e.g., 96 bits, needs to be retransmitted if Msg1 collides. In 3-step RA, a random ID, e.g., 16 bits, needs to be retransmitted if Msg1 collides. By comparison, in 2-step CBRA, retransmitting a larger Msg1 in case of Msg1 collision will result in a larger latency.

On the other hand, allocating more access occasions can reduce Msg1 collision to improve the efficiency. But this causes more idle/empty occasions, which leads to wasted time and increases the total inventory latency.

Furthermore, to compare the performance of 2-step CBRA and 3-step RA procedure, we calculate and compare the total latency of identifying the same amounts of devices by these two methods, and get the following conclusions. The evaluation results are shown in Appendix.

Observation 13:	The 2-step CBRA can save one step of data transmission (to reduce the latency) in case of low probability of Msg1 collision. But, once to reduce the collision by using large number of access occasions, the unused access occasion will then increase the latency.

Observation 14:	The 2-step CBRA is low efficient in case of relatively high probability of Msg1 collision.

Observation 15:	Based on the simulation/calculation in the Appendix, the total latency of 2-step CBRA is larger than 3-step like RA.

Based on the above analyses, it is proposed to at least first support the 3-step like RA and FFS for the 2-step like RA.

Proposal 7:	The solution of “A-IoT Msg1 without data” is supported in random access procedure.

2.3	A-IoT paging transmission failure (re-transmission) handling

RAN#103 meeting concludes that:

- The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.”

In theory, there could be potentially up to several tens of seconds of power charging via energy harvesting. In case of different A-IoT device deployments and architectures/implementations, the charging duration for the A-IoT devices to be ready for communication can vary. For example, devices farther from the reader may take tens of seconds longer to fully charge than those nearby. Then, the issue caused by the different time of device availability occurs. For the devices being available for R2D monitoring relatively early, there could be the chance of running out of power before the network sends the paging message (since the network may need to wait for devices to be available for DL monitoring relatively late).

Besides, there are also some cases that the devices miss the paging reception due to poor channel quality or CRC error. Thus how to ensure the reliability of paging reception is an important issue that needs to be discussed.

Observation 16:	In some cases, some A-IoT devices may fail the paging message reception, e.g. due to the device unavailability (running out of power) after charging or poor channel quality.

To really address device availability and paging reception failure issue, the network can re-transmit the paging message for the same inventory request to handle those devices, which missed the first paging message.

Proposal 8:	To handle the A-IoT paging reception failure, the reader is allowed to transmit the paging message multiple times for the same service request.

If the reader sends the A-IoT paging message multiple times for the same service request, the device that has successfully responded may also receive the subsequent paging re-transmission messages. As a result, the A-IoT device may redundantly perform the random (re) access/data transmission, which leads to a huge waste of air resources, increased collision and a potentially endless cycle. For example, when a device that has already successfully completed the same service responds to a retransmitted paging, there is a possibility of collision with another device that has never responded to the paging for the same service. Consequently, the newly available device may never get the chance to access the reader. Moreover, the reader will be unable to determine the termination of the service, as it continuously receives responses from devices. Therefore, to avoid redundant response, it is necessary for the device, which has successfully responded, to ignore or skip the response to the paging re-transmission of the same service.

Proposal 9:	The device should skip the response (e.g. not to perform the random (re)access and data transmission) to the retransmitted paging message of the same service request, to which the device has successfully responded (i.e. to avoid redundant responses).

In addition, it is also necessary to ensure that the device will not miss the paging message of a new service. In other words, the device must be able to respond to the paging message of a new service, regardless of the status of the previous service. To achieve this, the device needs to differentiate between a new and the current service. Thus, the reader needs to indicate in the paging message if it is for a new service or a re-transmission of the current one.

As one example given in the figure 2.1-1 the device, which failed to access the network after monitoring the initial paging message of inventory 1, can respond to the paging re-transmission messages of the same inventory 1. Upon successful response, it does not respond to additional paging re-transmissions within the inventory 1. However, the device should start responding to the paging message upon indication of a new inventory session, i.e. inventory 2. Furthermore, if the device misses the initial paging message of inventory 3, it can still receive and process the subsequent paging re-transmission indicating for the new inventory 3.

Figure 2.1-1:	Paging reception failure handling

Proposal 10:	The device should response to a paging message of a new service request, which is different from the previous service request that the device has responded (i.e. to avoid missing any request from reader).

Proposal 11:	The A-IoT paging message needs to indicate the device about whether it is for a new service request or a re-transmission for the old service request.

2.4	Transmission failure handling (after A-IoT paging)

2.4.1	Reader side failure handling

From reader side, during a R2D-to-D2R transmission procedure, there are two abnormal conditions:

- R2D failure: A R2D message is transmitted by reader but is not received successfully by device, in which case the device does not reply the corresponding D2R message;

- The corresponding D2R failure: A R2D message is transmitted by reader and received successfully by device, but the reader fails to receive the D2R message replied by the device.

The results of the both above cases are that the reader does not successfully receive the D2R message after sending a R2D message. An effective and direct way to detect/determine such a failure is to define a maximum waiting time between a R2D and the corresponding D2R transmission for reader.

In RAN1#117 agreement, agreement on timing relationship were reached as follows:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

From reader side, TR2D_max limits the maximum waiting time after a reader transmits a R2D message. From device side, TR2D_max is a requirement of maximum time in which a device should process the received R2D message and reply the corresponding D2R message. In our understanding, TR2D_max is to specify the processing requirements of different A-IoT devices.

Hence, based on the RAN1 defined TR2D_max, a reader can detect/determine the failure of R2D transmission or the following D2R transmission. The timer runs once a R2D message is transmitted by reader, the reader considers the R2D/D2R failure, if it does not successfully receive the corresponding D2R message before the timer expires.

Observation 17:	Reader can detect/determine the R2D transmission failure (or the following D2R transmission failure), based on the RAN1 defined timing relationship value TR2D_max.

When abnormal conditions are detected by the reader, it can re-send the R2D to re-trigger the device to send the corresponding D2R message again.

To improve the reliability of the service without introducing additional design complexity, upper layers (e.g., application layer and/or NAS-like layer) can directly retransmit the command or service request, i.e. we can rely on the upper layer implementation. In addition, the reader by implementation can retransmit the command buffered by the reader, if the command or corresponding D2R message is not successfully received, in order to trigger the device to resend those D2R message again. The upper layer retransmission of the command or service request can achieve high(er) reliability.

Proposal 12:	By implementation, the reader can re-send the buffered “command” to trigger the device to re-send the “response”.

As for the failure of Msg3 (including Device ID and/or any other upper layer data) transmission in 3-step RA, one way to re-trigger the device to re-send Msg3 is for the reader to re-send Msg2 again.

Proposal 13:	If the reader fails to receive the Msg3 content, the reader can re-send the A-IoT Msg2 to trigger the device to re-send the Msg3 content (e.g. device ID and/or any other upper layer data).

2.4.2	Device side failure detection

In NR, RAR window and contention resolution timer are defined for random access procedure. If the RAR window ends and Random Access Response that matches the transmitted Preamble has not been received, Random Access Response reception is considered as not successful. If the contention resolution timer expires, the contention resolution is considered as not successful. In A-IoT, accurate timing alignment cannot be supported due to large SFO. Furthermore, the concepts of RAR window and contention resolution timer are not suitable/practical for A-IoT, since they require a large number bits of register to count the timer. Then, as to the handling of the A-IoT Msg1/3 transmission failure and Msg2/”4” reception failure, the following cases are considered.

A-IoT Msg1 is transmitted by device. The device can consider A-IoT Msg1 transmission as successful if A-IoT Msg2 is received (with correct content). There is a case that reader does not receive A-IoT Msg1 successfully, then the reader deems that no device initiated the A-IoT random access in this access occasion, and will send R2D message indicating the (next) access occasion. Even if the reader receives A-IoT Msg1 successfully and sends the A-IoT Msg2, there is the case of A-IoT Msg2 reception failure at the device side. In that case, if the reader does not receive A-IoT Msg3 (after a period of time TR2D_max) after sending the A-IoT Msg2 (for several attempts), it may consider that transmission fails and send R2D message indicating the (next) access occasion (without success indication). From the device perspective, after it transmits the A-IoT Msg1, it expects to receive the A-IoT Msg2 rather than R2D message indicating the (next) access occasion. In this case, the device can consider the A-IoT random access as failed, if the access response (e.g. A-IoT Msg2) is not received successfully and it has received R2D message indicating the (next) access occasion.

Proposal 14:	After the device transmits the A-IoT Msg1, it considers A-IoT random access as failed, if the A-IoT Msg2 is not successfully received and it has received the R2D message indicating start of the next access occasion for another device.

A-IoT Msg3 is transmitted by device. If there is following “actual R2D data”, e.g. read/write command, the device can consider A-IoT Msg3 transmission as successful if following “actual R2D data” is received. If there is no following “actual R2D data”, when the reader cannot receive A-IoT Msg3 successfully (after several attempts), it considers that transmission fails and may decide to send R2D message indicating the (next) access occasion. However, just relying on R2D message indicating the (next) access occasion is not enough for the device to determine whether the A-IoT Msg3 is transmitted successfully or not, since the reader may anyway send R2D message (e.g. reader gives up this device after several attempts), indicating the (next) access occasion, to trigger another device to access even if the reception of the A-IoT Msg3 succeeds. Therefore, from device point of view, it cannot distinguish whether the transmission procedure succeeds or fails only based on R2D message indicating the (next) access occasion. Thus, an R2D message with explicit success/failure indication is necessary.

Following upper layer data is transmitted by device. Similar to A-IoT Msg3 transmission, there is the case of no further following upper layer data to device. Thus, an R2D message with explicit success/failure indication is needed to determine whether current transmission/procedure is failed, before/upon starting the access occasion for the next device.

Observation 18:	After the device transmits the A-IoT Msg3, the R2D message only indicating the (next) access occasion is not enough for the device to determine whether A-IoT random access is failed or not. Therefore, the R2D message with another explicit success/failure indication is necessary.

Therefore, it is proposed:

Proposal 15a:	Reader can send an R2D message to the device, which indicates whether its A-IoT procedure (data transmission for inventory and/or command) is successfully done or not.

Proposal 15b:	After the device transmits the A-IoT Msg3 or the following upper layer data, the failure/success of D2R transmission is determined based on the following R2D message (e.g. according to above Proposal 15a).

According to the analysis above, device can detect/determine “A-IoT procedure failure” based on the R2D message. For example, after sending A-IoT Msg1, the device considers procedure failure if the contention resolution fails, e.g. A-IoT Msg2 is not received successfully and it has received R2D message indicating the (next) access occasion. Or, after sending A-IoT Msg3 or upper layer data, the device considers procedure failure if following R2D message indicating success is not received successfully and it has received R2D message indicating the (next) access occasion. In these cases, the device will perform re-access.

Proposal 16:	After the D2R transmission, the device should perform re-access in case of the “A-IoT procedure failure”, including the contention resolution failure and D2R data transmission failure.

2.4.3	Re-access

A device may fail to complete contention resolution if it does not receive a successful response after it sends Msg1. Referring to other communication systems, e.g., NR/LTE, re-access designs are supported to address contention resolution failure. UE can perform random access again if a random access procedure is not successfully completed.

As discussed above, A-IoT devices also have the contention resolution failure issue due to collision. The devices whose transmissions collided will not get the opportunity to transmit data. For example, in the inventory case these devices will be missed if they do not have another opportunity to access. Besides, if the device fails in A-IoT Msg3 or upper layer data transmission, it also needs to access again.

As described in clause 2.1, one access round consists of multiple access occasions. If the devices failing to access re-select a new access occasion in the same round, the collision probability of the devices selecting the following access occasions will be larger.

Observation 19:	Re-accessing in the same access round will conflict with the reader assigned number of access occasion in this round, which will later cause more collision than the reader expectation.

To solve this problem, re-access in the next round after failure needs to be supported for the A-IoT contention-based random access procedure, i.e., the reader initiate the next round of access occasions for the device(s) failing to complete the access procedure in the last round. The device(s) that previously failed to access can access again in the new/next access round. Besides, a necessary improvement is that reader can adjust/assign the new number of access occasion in the next round according to the number of collided occasions. The collided occasion can help to determine the number of devices which have not been inventoried. This can improve the inventory efficiency, as the efficiency is strongly related to the number of devices and occasion, and the optimal efficiency can be achieved when the number of slots equals the number of devices.

Observation 20:	If re-accessing in the next access round, the reader is able to adjust and assign the new number of access occasion in the next round, to achieve optimized the efficiency.

Indicating/triggering more sub-occasion in one access occasion for the collided devices is another method to allow the collided device(s) can re-access in the same round. However, this will increase the latency of the other devices selecting the following access occasion. Besides, it is hard to determine the number of sub-occasions for re-accessing before the devices attempt to access. If a large number of sub-occasions is allocated, the resources will be wasted. This will introduce additional processing complexity, as the device needs to monitor an additional trigger for sub-occasion after failing to access.

Observation 21:	Re-accessing in the same access round will delay the access for the other devices selecting the later occasions in this round (which may run out of power due to this delay), and also increase the complexity of device behaviour and reader scheduling.

In summary, the first access round is for the initial access, while the following access round is for the re-access of the failed devices in the previous access round.

Proposal 17:	The device performs the re-access in the next access round, rather than in the same access round after detecting a failure, so that the reader is able to adjust the number of access occasions in the next round.

2.5	Contention-free access

R2-2405604 Discussion on random access-like procedure for Ambient IoT NTT DOCOMO INC. discussion Rel-19 Proposal 1. Ambient IoT Reader can determine to conduct CFRA if the number of targeted devices indicated by the CN is one. Discussion - Oppo, Qualcomm, Apple, thinks that we should not limit to a single device. Samsung sees a use case for multiple devices. CATT thinks we should support single device and further study multiple devices. - Intel thinks we should not use CFRA terminology, either contention-free or rach-less. - Vodafone doesn’t want to study multi-device as it would add complexity since reader needs to guarantee that there is no contention. Qualcomm explains that this is from reader perspective. - Mediatek that there may be complexity and we should prioritize single device to start with. - Chair indicates that in the next meeting we will start by discussing single device case. Agreement - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

In RAN2#126 meeting, chair notes indicate that “in the next meeting we will start by discussing single device case”. Based on the discussion and conclusion, the A-IoT paging message including IDs of multiple devices to trigger multiple contention-free accesses depends on RAN1 and SA2 discussion, e.g. multiple resources allocation for multiple devices, use case of multiple contention-free accesses. Thus, the single device case is discussed firstly, while we will discuss multiple devices cases based on more progress from RAN1/SA2.

Observation 22:	For the discussion of multiple devices contention-free access:

Chair notes indicate that in the next meeting we will start by discussing single device case for contention-free access;

Supporting for the use case of multiple devices cases relies on more progress from RAN1 and SA2.

If contention-free access procedure is initiated, the device can skip A-IoT Msg1 and A-IoT Msg2 of contention-based access procedure due to no need of contention resolution. That means that the device can directly send the upper layer data (e.g. device ID) in the very first D2R message after being triggered.

Proposal 18:	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).

After triggered by A-IoT paging message, the very first D2R message for contention-based access and contention-free access is different. Thus, how to determine whether the service is triggered for contention-based access or contention-free access needs to be considered. In the A-IoT paging message, the reader can indicate whether the current random access is for contention-free access and allocate the associated device specific access resources for contention-free access case.

Proposal 19:	For contention-free access, the reader uses the R2D message to indicate whether the current random access is contention-free access type, and if so, indicate the associated device specific access resources.

2.6	FDMA related random access

In last RAN1 meeting, RAN1 agreed to study FDMA for D2R transmission, while no clear agreement on the support of FDM for R2D:

RAN1#116-bis agreement [1]: Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

Observation 23:	RAN1 agree to study the support of TDMA and FDMA for D2R, while there is no clear agreement on the support of FDM for R2D.

The TDMA method distributes devices into multiple occasions to mitigate time-domain collision. In addition, applying FDMA to Msg1 can further mitigate Msg1 collision in the same time-domain occasion, since multiple Msg1s are still possible to be overlapped in time domain. This improves access efficiency and reduce latency compared with TDMA only methods.

Observation 24:	Applying both TDMA and FDMA in A-IoT Msg1 can improve the efficiency, reduce latency and mitigate the A-IoT Msg1 collision.

Proposal 20:	RAN2 confirm both TDMA and FDMA can be applied to A-IoT Msg1.

If RAN2 agrees that FDMA can be applied to A-IoT Msg1, the frequency configuration needs to be provided to the devices prior to sending Msg1. How to indicate the frequency configuration are pending to RAN1 discussion.

Proposal 21a:	If RAN2 confirms to apply the FDMA for A-IoT Msg1, it is assumed that the frequency related information is indicated by the reader (details are up to RAN1).

To improve the efficiency, multiple devices should be distributed into different frequencies, e.g., different frequency shifts in baseband. Then, the device can randomly select one frequency among the indicated frequency related information.

Proposal 21b:	If RAN2 confirms to apply the FDMA for A-IoT Msg1, it is assumed that A-IoT device randomly selects one frequency among the indicated frequency related information.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	The frequency-domain access resource definition/indication for A-IoT Msg1 will be discussed by RAN1.

Observation 2:	A-IoT device is not able to support the NR RACH occasion defined by the absolute timing, due to its extremely low timing accuracy.

Observation 3:	As one modelling, both A-IoT paging message and access round message (Query) can start/initiate an access round (including multiple access occasions).

Observation 4: 	The failure of contention resolution probability (when 16-bits random ID is used) is already very low (e.g. 0.0015%).

Observation 5a:	In the Slotted ALOHA mechanism, about 2~3 devices selecting the same access occasion will be the typical case (normally less than 8 devices). Therefore, 16-bit random ID in Msg1 should be sufficient for resolving the contention (i.e. contention resolution failure probability will be about 1216), among those limited number of devices selecting the same occasion.

Observation 5b:	Reader will allocate suitable/similar number of access occasions according to the total number of devices to be triggered. Therefore, the number of devices selecting the same occasion won’t increase according to the total number of devices.

Observation 6: 	Considering the inventory latency, the 32-bit random ID may increase by about 27% the inventory latency, compared to the 16-bit random ID case.

Observation 7: 	Similar to NR, there is no need to absolutely avoid the contention resolution failure (like zero probability resolution failure), considering that it is always possible to allow the re-access to handle any rare case failure.

Observation 8:	In 2-step CBRA, as to the identification information in Msg2 used for content resolution:

The more bits it includes, the less efficiency;

The less bits it includes, the more collision it may suffer.

Observation 9:	For 2-step CBRA, if the Msg2 includes the whole device ID for contention resolution, the total size of Msg1+Msg2 will be even larger than the sum of Msg1+Msg2 +Msg3 of 3-step CBRA.

Observation 10a: For 2-step CBRA, A-IoT Msg2 should not include the whole device ID.

Observation 10b: For 2-step CBRA, if A-IoT Msg2 only includes the partial device ID, it may not be random enough for ambiguous contention resolution, i.e. at least about 32 bits are needed.

Observation 11:	The random ID in Msg1 can be reused as the AS scheduling ID for later data transmission, if any.

Observation 12:	The D2R multiplexing in Msg1 (including both device ID and random ID) will cause some processing complexity.

Observation 13:	The 2-step CBRA can save one step of data transmission (to reduce the latency) in case of low probability of Msg1 collision. But, once to reduce the collision by using large number of access occasions, the unused access occasion will then increase the latency.

Observation 14:	The 2-step CBRA is low efficient in case of relatively high probability of Msg1 collision.

Observation 15:	Based on the simulation/calculation in the Appendix, the total latency of 2-step CBRA is larger than 3-step like RA.

Observation 16:	In some cases, some A-IoT devices may fail the paging message reception, e.g. due to the device unavailability (running out of power) after charging or poor channel quality.

Observation 17:	Reader can detect/determine the R2D transmission failure (or the following D2R transmission failure), based on the RAN1 defined timing relationship value TR2D_max.

Observation 18:	After the device transmits the A-IoT Msg3, the R2D message only indicating the (next) access occasion is not enough for the device to determine whether A-IoT random access is failed or not. Therefore, the R2D message with another explicit success/failure indication is necessary.

Observation 19:	Re-accessing in the same access round will conflict with the reader assigned number of access occasion in this round, which will later cause more collision than the reader expectation.

Observation 20:	If re-accessing in the next access round, the reader is able to adjust and assign the new number of access occasion in the next round, to achieve optimized the efficiency.

Observation 21:	Re-accessing in the same access round will delay the access for the other devices selecting the later occasions in this round (which may run out of power due to this delay), and also increase the complexity of device behaviour and reader scheduling.

Observation 22:	For the discussion of multiple devices contention-free access:

Chair notes indicate that in the next meeting we will start by discussing single device case for contention-free access;

Supporting for the use case of multiple devices cases relies on more progress from RAN1 and SA2.

Observation 23:	RAN1 agree to study the support of TDMA and FDMA for D2R, while there is no clear agreement on the support of FDM for R2D.

Observation 24:	Applying both TDMA and FDMA in A-IoT Msg1 can improve the efficiency, reduce latency and mitigate the A-IoT Msg1 collision.

Msg1 time-domain access occasion definition/indication

Proposal 1:	RAN2 agrees that the reader transmits one explicit R2D message to define/indicate the start/boundary of the access occasion (instead of defining the NR RACH occasion by absolute timing).

Msg1 time-domain resource selection

Proposal 2a:	One access round consists of multiple access occasions.

Proposal 2b:	The total number of time-domain access occasions within one access round is indicated by the reader.

Proposal 2c:	A-IoT device randomly selects one access occasion among the multiple time-domain access occasions in the access round.

A-IoT paging and random access relationship

Proposal 3:	The R2D configuration, to indicate the number of access occasions of one access round, can be either sent in a R2D separate message or combined into A-IoT paging message.

Msg1/Msg2 content: Common

Proposal 4:	The 16-bit is sufficient for the random ID in A-IoT Msg1 (i.e. not larger than 16bits).

Proposal 5: 	As the baseline, in TDMA case, A-IoT Msg2 does not include multiple acknowledgments to multiple devices’ A-IoT Msg1 (FFS for FDMA case).

Msg1/Msg2 content: 2-step RA design

Proposal 6:	For 2-step CBRA, the random ID is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.

2-step vs. 3-step RA down-selection

Proposal 7:	The solution of “A-IoT Msg1 without data” is supported in random access procedure.

A-IoT paging transmission failure (re-transmission) handling

Proposal 8:	To handle the A-IoT paging reception failure, the reader is allowed to transmit the paging message multiple times for the same service request.

Proposal 9:	The device should skip the response (e.g. not to perform the random (re)access and data transmission) to the retransmitted paging message of the same service request, to which the device has successfully responded (i.e. to avoid redundant responses).

Proposal 10:	The device should response to a paging message of a new service request, which is different from the previous service request that the device has responded (i.e. to avoid missing any request from reader).

Proposal 11:	The A-IoT paging message needs to indicate the device about whether it is for a new service request or a re-transmission for the old service request.

Reader side failure handling

Proposal 12:	By implementation, the reader can re-send the buffered “command” to trigger the device to re-send the “response”.

Proposal 13:	If the reader fails to receive the Msg3 content, the reader can re-send the A-IoT Msg2 to trigger the device to re-send the Msg3 content (e.g. device ID and/or any other upper layer data).

Device side failure detection

Proposal 14:	After the device transmits the A-IoT Msg1, it considers A-IoT random access as failed, if the A-IoT Msg2 is not successfully received and it has received the R2D message indicating start of the next access occasion for another device.

Proposal 15a:	Reader can send an R2D message to the device, which indicates whether its A-IoT procedure (data transmission for inventory and/or command) is successfully done or not.

Proposal 15b:	After the device transmits the A-IoT Msg3 or the following upper layer data, the failure/success of D2R transmission is determined based on the following R2D message (e.g. according to above Proposal 15a).

Proposal 16:	After the D2R transmission, the device should perform re-access in case of the “A-IoT procedure failure”, including the contention resolution failure and D2R data transmission failure.

Re-access

Proposal 17:	The device performs the re-access in the next access round, rather than in the same access round after detecting a failure, so that the reader is able to adjust the number of access occasions in the next round.

Contention-free access

Proposal 18:	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).

Proposal 19:	For contention-free access, the reader uses the R2D message to indicate whether the current random access is contention-free access type, and if so, indicate the associated device specific access resources.

FDMA related random access

Proposal 20:	RAN2 confirm both TDMA and FDMA can be applied to A-IoT Msg1.

Proposal 21a:	If RAN2 confirms to apply the FDMA for A-IoT Msg1, it is assumed that the frequency related information is indicated by the reader (details are up to RAN1).

Proposal 21b:	If RAN2 confirms to apply the FDMA for A-IoT Msg1, it is assumed that A-IoT device randomly selects one frequency among the indicated frequency related information.

# 4	References

- RAN1#116-bis Chair Notes

- R1-2400114, “Ultra low power device architecture for Ambient IoT”, RAN1#116, Athens, Greece, Huawei.

- EPC Tag Data Standard, version 2.0.0, available at https://ref.gs1.org/standards/tds/2.0.0/

- EPC™ Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.

- R2-2404243, A-IoT random access procedure, Huawei, HiSilicon.

- R1-2403954	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

5	Appendix

Considering most transmission parameters and message sizes for A-IoT are still under study, we take the related parameters captured in TR 38.848, TR 22.840 and RFID spec. According to TR 38.848, we set:

- data rate as 500 bps and,

- the size of device ID as 96 bits (in SA1 TR 22.840).

According to RFID spec, we set:

- the size of random ID as 16 bits.

We assume each packet has an 8-bit header (including both AS and upper-layer headers). According to the feature of Slotted ALOHA, we assume each device randomly select an occasion to access in each round, the collided devices can re-access in the next round.

There are three possibilities for each occasion as below, and for each we give the estimated latency by considering the assumed data rate.

- Successful occasion: one access occasion is selected by one device and the access succeeds

  - 3-step RA (Figure 5-1(a)):

    - including one access occasion trigger message (8 bits), one Msg1 (8-bit header + 16-bit random ID = 24 bits), one Msg2 (8-bit header + 16-bit random ID = 24 bits), and Msg3 (8-bit header + 96-bit device ID = 108 bits)

  - 2-step RA (Figure 5-1(b)):

    - including one access occasion trigger message (8 bits), one Msg1 with data (8-bit header + 96-bit device ID + 16-bit random ID = 120 bits) and one Msg2 (8-bit header + 16-bit random ID = 24 bits);

- Collided occasion (Figure 5-1(c)): one access occasion is selected by multiple devices and the access fails

  - including one access occasion trigger message (8 bits) and one Msg1 (24 bits)/Msg1 with data (120 bits);

- Idle occasion (Figure 5-1(b)): one access occasion is not selected by any device

  - including one access occasion trigger message (8 bits) and a minimum waiting time to at least check detect a 8-bit header;

Figure 5-1(a):	Successful occasion of using Msg1 without data and with data

Figure 5-1(b):	Collided and Idle occasion of using without data and with data

Many existing papers illustrated that the theoretical optimal efficiency of Slotted ALOHA is about 36.8% [5], and the optimal efficiency can be achieved when the number of slots equals the number of devices. Hence, we assume the total number of slots to equal the number of unidentified devices in every round.

Table 5-1 and Figure 5-2 illustrate the performance results on latency for identifying different number of devices. It is observed that the 3-step RA can achieve a lower latency, and achieves more significant improvement of efficiency as the number of devices increases.

Table 5-1	Performance results on latency of Msg1 with/without data

| The number of devices | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The overall latency for Msg1 with data (seconds) | 96.9 | 87.1 | 76.5 | 67.0 | 58.4 | 48.8 | 38.6 | 28.5 | 18.6 | 9.4 | 7.3 | 4.7 |
| The overall latency for Msg1 without data (seconds) | 69.1 | 62.7 | 55.5 | 48.6 | 41.9 | 34.9 | 27.8 | 20.7 | 13.7 | 6.9 | 5.2 | 3.4 |
| Average probability of collision = 26.4%, Average probability of success = 36.8%, Average probability of idle occasions = 36.8% |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5-2	Curves of latency changes as the number of devices change

Furthermore, we give the results of allocating more occasions, in which a lower probability of collision (about 9.2%) can be achieved, i.e., in each round, the number of slots is set to twice the number of devices. As shown in Table 5-2 and Figure 5-3, 3-step RA performs better, and achieves more significant improvement of efficiency as the number of devices increases. Though 2-step RA works better with more allocated slots, but still cannot catch up with 3-step RA. The reason is that allocating more slots can achieve a lower collision probability, but the drawback is that there are more idle slots, which leads to wasted time.

According to the above results, though 2-step RA can save one step, the efficiency cannot be greater than 3-step RA because once Msg1 (including Device ID) collide, a longer packet will be wasted and be retransmitted in the next access occasion. On the other hand, allocating much more slots can reduce Msg1 collision. However, this causes more idle/empty slots at the same time, which increases the total latency.

Table 5-2	Performance results on latency of Msg1 with/without data

NOTE: the number of occasions is set to twice the number of devices in each round (i.e. more allocated occasions)

| The number of devices | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The overall latency of Msg1 with data (seconds) | 83.18 | 74.80 | 66.44 | 58.13 | 49.58 | 41.59 | 33.05 | 24.66 | 16.59 | 8.25 | 6.29 | 4.11 |
| The overall latency of Msg1 without data (seconds) | 72.79 | 65.50 | 58.23 | 50.93 | 43.56 | 36.41 | 29.03 | 21.71 | 14.54 | 7.26 | 5.48 | 3.61 |
| Average probability of collision = 9.2%, Average probability of success = 29.9%, Average probability of idle occasions = 60.9% |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5-3	Curves of latency changes as the number of devices change (more allocated occasion)