3GPP TSG-RAN WG2 Meeting #127	R2-2406717

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting [1], the following agreements are reached on LP-WUS for RRC_CONNECTED mode.

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

There were the following agreements on candidate solutions for RRC CONNECTED mode in RAN1.

Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Combination of options in Case 1 should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

  - Basic functionality for LP-WUS

Fig.1. The illustration of candidate solutions

The understanding of each candidate solution and analysis on the higher layer impacts are provided below.

- Option 1-1: based on the conclusions in last RAN2 meeting, the solutions/operations introduced for DCP mechanism are assumed as baseline for Option 1-1, i.e. the LP-WUS monitoring occasion is at a time offset before the start of drx-onDurationTimer and the LP-WUS is used to indicate the start of drx-onDurationTimer.

- Option 1-2: UE can monitor LP-WUS outside C-DRX active time (i.e. UE is not monitoring PDCCH by MR), the LP-WUS is used to trigger the MR PDCCH monitoring as soon as possible (e.g. after LP-WUS reception/processing time, MR transition time, and time for synchronization of MR) without restriction of start of drx-onDurationTimer.

  - Option 1-2-1: UE still should perform PDCCH monitoring during legacy drx-onDurationTimer periodically, and additionally starts PDCCH monitoring as soon as possible after receives its LP-WUS. To achieve power saving gain, Option 1-1 should be combined to use LP-WUS to indicate whether to start the drx-onDurationTimer. So, the updated Option 1-2-1 combined with Option 1-1 is a more reasonable solution with gain on power saving and latency.

  - Option 1-2-2: UE ignores the legacy drx-onDurationTimer when the UE is monitoring LP-WUS, and starts PDCCH monitoring as soon as possible after receiving its LP-WUS.

- Option 1-3: UE can monitor LP-WUS inside C-DRX active time and LP-WUS is used to trigger the MR PDCCH monitoring as soon as possible. This option can be combined with Option 1-2, as Option 1-2 concerns on the LP-WUS monitoring outside C-DRX active time while Option 1-3 concerns on the LP-WUS monitoring inside C-DRX active time. Combination of options in Case 1 were also mentioned in RAN1 conclusion.

Based on TR 38.869 [2], it provided power and latency evaluation on Option 1-1 and Option 1-2. The results in Appendix show that, e.g. for FTP3 traffic, LP-WUS compared with CDRX or CDRX+DCP, the power saving gain provided by Option 1-1 is about average (16.75%~34%) for different CDRX cycle length. However, the power saving gain provided by Option 1-2 can be average 61.3% for the case MR enters deep-sleep state. For latency, showed by UPT gain, the latency of Option 1-1 is much larger than Option 1-2, since for Option 1-1 UE needs to wait until the next onDuration period before it can monitor PDCCH.

Observation 1: For Option 1-1, the power saving gain is less than Option 1-2 and additional latency will be introduced.

For DCP solution, the power saving gain is obtained by sacrificing latency due to duty cycle monitoring for DCP. As the power consumption for LP-WUR is less compared to MR, continuous monitoring of LP-WUS can be supported and the additional power saving gain can be achieved without introducing additional latency. There is no point in introducing a new mechanism to replace an old one without additional gain. So Option 1-1 should be precluded.

Proposal 1: LP-WUS is not a replacement of Rel-16 DCP, i.e. Option 1-1 is precluded.

For Option 1-2, two sub-options are provided. To achieve power saving gain, Option 1-1 should be combined with Option 1-2-1, so the updated Option 1-2-1 is a more complicated solution. Option 1-2-2 can obtain the same gain on power saving and latency with less standard work, so option 1-2-2 is preferred.

Proposal 2: UE starts MR PDCCH monitoring as soon as possible if LP-WUS targeted to itself is received and Option 1-2-2 is supported due to less complexity.

For Option 1-3, it depends on how it works with Option 1-2-2. In our view, MR PDCCH monitoring is only triggered by LP-WUS in Option 1-2-2, then before start of PDCCH monitoring, there are two directions to achieve “PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS”:

- Direction 1: onDurationTimer is not running as legacy operation and MAC layer considers onDuration duration is not within C-DRX active time, so that PDCCH monitoring is not performed as the legacy UE monitoring behavior. Once the LP-WUS is received, it triggers UE to enter C-DRX active time and start PDCCH monitoring.

- Direction 2: onDurationTimer is still running and MAC layer considers onDuration duration is within C-DRX active time, but PDCCH monitoring is not performed. This is similar as PDCCH skipping, PDCCH monitoring is controlled by PHY layer and transparent to MAC layer. In this case, LP-WUS can be received outside or inside the C-DRX active time, and it should trigger PDCCH monitoring regardless of when it is received. So combined Option 1-2-2 and Option 1-3 is a complete solution, this is tightly related to PHY layer and more RAN1 progress is needed, it is too early to determine whether to support or preclude Option 1-3 now.

Fig.2. The illustration of two directions for Option 1-2-2

Observation 2: Option 1-3 can work with Option 1-2-2 and it is more like a PHY solution.

Proposal 3: To further discuss Option 1-3 until RAN2 gets more inputs from RAN1 and has clearer view on basic functionality for LP-WUS.

  - Activation/deactivation procedure of LP-WUS monitoring

There were the following agreements in RAN1.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Firstly, it is meaningful to clarify the terminology. It was agreed in RAN2: The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. This is related to “configure/enable/disable” of the feature, and is different with “activation/deactivation of LP-WUS monitoring”. In the case that UE is configured/enabled with LP-WUS, “activate/deactivate” represents the start/stop of LP-WUS monitoring.

Fig.3. Activation/deactivation of LP-WUS monitoring

To save power, UE can monitor LP-WUS based on gNB’s indication when there is no traffic and switch to monitor PDCCH when traffic arrives. In other words, the activation/deactivation of LP-WUS is highly dependent on the traffic arrival. Therefore, it may be not flexible enough to trigger activation/deactivation of LP-WUS monitoring by RRC signaling, i.e. based on whether the LP-WUS related configuration is provided or not, which takes effect slowly. Instead, a more dynamic triggering of LP-WUS monitoring is a better choice.

Observation 3: A more dynamic and flexible triggering of LP-WUS monitoring than RRC signaling is needed for activation/deactivation of LP-WUS considering DL traffic’s arrival.

As one method, LP-WUS monitoring can be triggered explicitly by L1/L2 signaling, e.g. by DCI or MAC CE. As another method, LP-WUS monitoring can be triggered implicitly by a timer to reduce signaling overhead. For example, a timer can be started if the UE receives LP-WUS to wake up MR and restarted if PDCCH indicates a UL grant or a DL assignment, and when the timer expires, LP-WUS monitoring is activated.

When DL traffic for a particular UE arrives, the gNB will send LP-WUS to trigger MR PDCCH monitoring for data transmission. Or the signal quality for LP-WUS is not good enough, e.g. based on the measurement report from UE, the gNB can send LP-WUS to wake up the MR and deactivate LP-WUS monitoring. Therefore, it is obvious that LP-WUS monitoring can be deactivated by the reception of LP-WUS targeted to the UE. Moreover, as discussed and agreed during the study phase, UL transmission by MR also triggers PDCCH monitoring by MR. Because when UL traffic arrives, it is expected that the UL data can be transmitted in a timely manner.

Proposal 4: In CONNECTED mode, LP-WUS monitoring can be:

- activated by MR L1/L2 signaling and/or timer;

- deactivated by the reception of LP-WUS targeted to the UE and/or UL transmission by MR.

When UE is activated to monitor LP-WUS, if UE starts to apply LP-WUS ahead of the network, uncompleted services from the network may be affected, causing UE’s data loss. On another hand, if the LP-WUS function takes effect on the network before the UE, UE may not enter LP-WUR mode in time, which reduces the power saving gain, and for the network there may be more signaling overhead as well. Thus, it is beneficial that the network/UE can align the start point for monitoring LP-WUS.

Proposal 5: Alignment of the start point for monitoring LP-WUS can be further studied.

  - Other higher layer impacts

    - SPS and CG

LP-WUS indication in CONNECTED mode is to trigger MR PDCCH monitoring, the LP-WUS only impacts MR PDCCH monitoring. For DL configured resources, e.g. DL SPS, as the gNB allocates DL SPS resources despite knowing the LP-WUS is enabled, gNB expects the UE to perform legacy operation on DL SPS resources, and there is no mismatch between UE and gNB on LP-WUR/MR monitoring. From UE perspective, UE should monitor DL SPS resources by MR to avoid missing any DL data.

Proposal 6: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

However, for UL configured resources, e.g. configured grant Type 1 or configured grant Type 2 (UL SPS), sometimes there is a UL configured resource but UE does not have the UL data for transmission. If UL skipping is not enabled, the UE should still transmit padding bits in the UL configured resource which leads to additional power consumption. If the UE is operating with LP-WUR, it is beneficial to skip UL configured resource if no UL data available so the UE does not need to turn on the MR and keeps monitoring LP-WUS.

Proposal 7: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

    - SRS transmission and CSI reporting

In existing DRX and Rel-16 DCP operation, the UE behavior for SRS (periodic SRS and semi-persistent SRS) transmission and CSI reporting (periodic CSI, that is L1-RSRP or not, and semi-persistent CSI) are additionally defined in order to save power. Similarly for LP-WUS, the UE behaviour for SRS transmission and CSI reporting also needs to be considered to reduce the power consumption. For example, as for DCP, when UE is monitoring LP-WUS, the UE does not need to transmit periodic SRS and semi-persistent SRS, report semi-persistent CSI configured on PUSCH and report semi-persistent CSI on PUCCH.

Proposal 8: RAN2 to discuss the UE behavior for SRS transmission and CSI reporting while UE is operating with LP-WUS.

    - LP-WUS with CA

There were the following agreements in RAN1 on CA scenario.

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded

It is unclear whether the LP-WUS will be supported in CA scenario, which needs further justification/discussion in RAN1. If CA is supported, PDCCH monitoring on different serving cells need to be further studied. For example, UE only monitors LP-WUS on PCell, or multiple LP-WUS monitoring occasions on PCell and SCell(s) can be defined to control different serving cell groups. Once the LP-WUS is received targeted to the UE, UE starts MR PDCCH monitoring in all activated serving cells, or only on specific serving cell (e.g. PCell) and relies on other signaling to activate PDCCH monitoring in other serving cell. Anyway, whether/how LP-WUS works when UE is configured with CA needs to be further studied with taking RAN1 progress into account.

Proposal 9: RAN2 waits for more RAN1 conclusion for further study on LP-WUS with CA.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Basic functionality for LP-WUS

Proposal 1: LP-WUS is not a replacement of Rel-16 DCP, i.e. Option 1-1 is precluded.

Proposal 2: UE starts MR PDCCH monitoring as soon as possible if LP-WUS targeted to itself is received and Option 1-2-2 is supported due to less complexity.

Proposal 3: To further discuss Option 1-3 until RAN2 gets more inputs from RAN1 and has clearer view on basic functionality for LP-WUS.

Activation/deactivation procedure of LP-WUS monitoring

Proposal 4: In CONNECTED mode, LP-WUS monitoring can be:

- activated by MR L1/L2 signaling and/or timer;

- deactivated by the reception of LP-WUS targeted to the UE and/or UL transmission by MR.

Proposal 5: Alignment of the start point for monitoring LP-WUS can be further studied.

Other higher layer impacts

Proposal 6: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

Proposal 7: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

Proposal 8: RAN2 to discuss the UE behavior for SRS transmission and CSI reporting while UE is operating with LP-WUS.

Proposal 9: RAN2 waits for more RAN1 conclusion for further study on LP-WUS with CA.

- References

R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

TR 38.869, Study on low-power wake up signal and receiver for NR.

- Appendix (TR 38.869, 8.1.2.2.2)

8	Evaluation results

8.1	Power and latency evaluation

8.1.2	RRC CONNECTED mode

8.1.2.2	FTP 3 and IM traffic model

In this clause, the usage of LP-WUS for FTP3 and IM traffic is studied.

For the evaluated FTP3 traffic, packet size: 0.5Mbyte and average inter-arrival time: 200ms are assumed by companies, unless otherwise noted.

For the evaluated IM traffic, packet size: 0.1 Mbyte and average inter-arrival time: 2seconds are assumed by companies, unless otherwise noted.

The following schemes are compared, and the description of the schemes are as follows,

-	Always on

-	i.e., no DRX , UE is always available for gNB scheduling

-	C-DRX

-	DCP

-	R16 wake-up signal for RRC CONNECTED UE

-	R17 PDCCH Scheme

-	i.e., R17 PDCCH skipping and/or R17 SSSG switching

-	LP-WUS scheme 1

-	LP-WUS to trigger the resumption of MR PDCCH monitoring from micro/light/deep sleep.

-	Note: LP-WUS scheme1 does not require C-DRX configuration.

-	LP-WUS scheme 2

-	LP-WUS to replace DCP, i.e. LP-WUS to trigger the start of the drx_onDurationTimer.

-	LP-WUS to replace DCP outside DRX active time, and LP-WUS to trigger the resumption of MR PDCCH monitoring during DRX active time (including drx_onDurationTimer and drx-inactivityTimer). (evaluated by [8A-1])

-	Note: LP-WUS scheme 2 can only be used with C-DRX configuration.

Note: MR based RRM measurements were not explicitly modelled for power saving evaluation.

8.1.2.2.1	Collection of the results

[Text Omitted]

##### 8.1.2.2.2	Observations

Note: the average value, the min and max value in the range is calculated across all companies' results

For the usage of LP-WUS is to trigger the resumption of PDCCH monitoring (i.e., LP-WUS scheme1),

-	For FTP3 traffic, when WUR ON power setting is NO more than 4units,

-	LP-WUS compared with CDRX or CDRX+DCP provide

-	Results in [8A-11] [8A-7] [8A-13] [8A-14] show that power saving gain 24%~86% (average: 61.3%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-11] [8A-13] [8A-14] show that power saving gain -3%~41% (average: 23.75%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-11] [8A-14] show that power saving gain -74%~-25% (average: -36%) when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

-	Results in [8A-5] [8A-13] show that power saving gain 44%~62% (average: 53%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-5] [8A-13] show that power saving gain -11%~18% (average: 3.5%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-5] shows that power saving gain -26% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	LP-WUS compared with alwayson provide

-	Results in [8A-11] shows that power saving gain 40%~53% (average: 46.5%) when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	For FTP3 traffic, when WUR ON power setting is equal to 10 units,

-	LP-WUS compared with CDRX or CDRX+ DCP provide

-	Results in [8A-7] [8A-11] [8A-13] show that power saving gain 24%~58% (average: 40.25) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-11] [8A-13] show that power saving gain -33%~12% (average: -10.5%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-11] shows that power saving gain -114% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

-	Results in [8A-13] shows that power saving gain 33% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-13] shows that power saving gain-40% when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	For IM traffic, when WUR ON power setting is NO more than 4units,

-	LP-WUS compared with CDRX or CDRX+DCP provide

-	Results in [8A-7] shows that power saving gain 69% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

-	Results in [8A-5] shows that power saving gain 79% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-5] shows that power saving gain -150% when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-5] shows that power saving gain -425% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	For IM traffic, when WUR ON power setting is equal to 10 units,

-	LP-WUS compared with CDRX+ DCP provide

-	Results in [8A-7] shows that power saving gain 56% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	For FTP3 traffic,

-	LP-WUS compared with CDRX or CDRX+DCP provide

-	Results in [8A-11] [8A-7] [8A-13] show that UPT gain -26%~194% (average: 10.75%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-11] [8A-13] show that UPT gain 36%~87% (average: 62.7%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-11] shows that UPT gain 124%~180% (average: 152%) when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

-	Results in [8A-5] [8A-13] show that UPT gain -9%~1% (average: -4%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-5] [8A-13] show that UPT gain 59%~86% (average: 72.5%) when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-5] shows that UPT gain 128% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	LP-WUS compared with always-on provides

-	Results in [8A-11] shows that the same UPT performance as always-on when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

-	For IM traffic,

-	LP-WUS compared with CDRX+ DCP provide

-	Results in [8A-7] show that UPT gain 44%~774% (average: 184%) when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	LP-WUS compared with R17 PDCCH + CDRX+ DCP provide

-	Results in [8A-5] show that UPT gain 94% when MR is allowed to enter in deep-sleep state during LP-WUS monitoring

-	Results in [8A-5] show that UPT gain 453% when MR is allowed to enter in light-sleep state during LP-WUS monitoring

-	Results in [8A-5] show that UPT gain 2260% when MR is allowed to enter in micro-sleep state during LP-WUS monitoring

For the usage of LP-WUS to replace DCP (i.e., LP-WUS scheme2) for FTP3 traffic,

-	The scheme of LP-WUS for DCP replacement compared with CDRX+DCP provide (LP-WUR ON power no more than 1 unit)

-	Results in [8A-2] [8A-8] [8A-11] [8A-15] show that power saving gain (1%~88%, average: 16.75%) when CDRX cycle length is 160ms;

-	Results in [8A-11] [8A-15] show that power saving gain (18%~47%, average: 27%) when CDRX cycle length is 40ms;

-	Results in [8A-2] [8A-8] show that power saving gain (9%~78%, average: 25%) when CDRX cycle length is 320ms;

-	Results in [8A-2] show that power saving gain (10%~63%, average: 34%) when CDRX cycle length is 640ms;

-	The scheme of LP-WUS to replace DCP outside DRX active time and to trigger the resumption of MR PDCCH monitoring during DRX active time (including drx_onDurationTimer and drx-inactivityTimer) compared with R17 PDCCH+CDRX+DCP provide

-	Results in [8A-1] show that power saving gain (8%, 29%, 50%) for the case MR enters (micro sleep, light sleep, deep sleep) during LP-WUS monitoring, respectively;

-	The scheme of LP-WUS for DCP replacement compared with CDRX+DCP provide

-	Results in [8A-2] [8A-11] [8A-15] show that UPT gain (-6%~0%, average: -1.7%) when CDRX cycle length is 160ms;

-	Results in [8A-11] [8A-15] show that UPT gain (-22%~0%, average: -5.5%) when CDRX cycle length is 40ms;

-	Results in [8A-2] show that the same UPT performance when CDRX cycle length is 320ms;

-	Results in [8A-2] show that the same UPT performance when CDRX cycle length is 640ms;

-	The scheme of LP-WUS to replace DCP outside DRX active time and to trigger the resumption of MR PDCCH monitoring during DRX active time (including drx_onDurationTimer and drx-inactivityTimer) compared with R17 PDCCH+CDRX+DCP provide

-	Results in [8A-1] show that UPT gain (34%, -15%, -35%) for the case MR enters (micro sleep, light sleep, deep sleep) during LP-WUS monitoring, respectively;