3GPP TSG-RAN WG2 Meeting #127                                     R2-2406718

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			OPPO

Title: 	Discussion on paging procedure for Ambient IoT

Agenda item:		8.2.3

Document for: 	Discussion and Decision

- Introduction

In the last RAN2 meeting, regarding paging procedure for Ambient IOT, following agreements have been achieved.

Agreements

1 RAN2 will study the following cases for AIOT paging message:

- a message containing an ID of a single AIOT device.

- a message containing a group ID that maps to multiple AIOT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive an AIOT message.

- a message containing multiple IDs of AIOT devices. Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

2 AIOT paging message indicate information from which the device can determine resources to be used for response (D2R    message). FFS how (e.g., implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details

and in the RAN2#125b meeting, following agreements have been achieved.

Agreements:

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

In this contribution, we aim to provide our views on the paging procedure for the AIOT system.

2. Discussion

- 2.1 including the purpose of the paging in the paging message

According to the RAN2 on-line discussion, sending paging message towards AIOT devices could serve for different purposes:

- Inventorying AIOT devices in the proximity, optionally matched with the group/mask/device ID

- Sending commands directly towards the target AIOT device(s)

Considering different UE behaviour can be incurred such as reporting AIOT device ID in the UL message for inventory cases, and reporting confirmation of reception of the commands/ UE buffered data for the commanding cases, the purpose of the paging message may need to be included in the paging message, for the convenience of the AIOT device to know which type of information is needed to be reported towards the reader subsequently.

Proposal 1: RAN2 to agree on including the purpose of the paging message, e.g., inventory, sending commands, in the paging message towards AIOT devices.

- 2.2 Determination of transmission/access occasions

As mentioned in the first part, in the last RAN2 meeting, it was agreed to consider four different cases covering different number of targeted AIOT devices being paged. Subsequently, we will categorize them into two classes: a paging message for a single AIOT device, and a paging message for more than one AIOT devices.

- 2.2.1 a paging message containing an ID of a single AIOT device

In such case, the paged AIOT device could access to the network immediately, after reception of the AIOT paging message, subject to the restriction of any potential message processing delay. Therefore, there is no need to indicate the transmission occasion explicitly in the paging message.

Observation 1: when reception of a AIOT paging message containing only one ID of a single AIOT device, the AIOT device could access to the network immediately, after reception of the AIOT paging message, subject to the restriction of any potential message processing delay.

Proposal 2: RAN2 to agree that indication of the transmission/access occasions related information in the AIOT paging message could be absent.

- 2.2.2 a message for more than one AIOT devices

In such case, (potentially) more than one AIOT devices are paged by the AIOT paging message, each of them should know in which transmission/access occasion they can access to the network. In RFID system, the reader sends a number Q to the target AIOT devices in the inventory message. Subsequently, each targeted AIOT device will generate a random integer number between 0 and 2Q-1. Only when the integer number equals to 0, it will perform access towards the network. Otherwise, it will wait for subsequent QueryRep command to decrease the integer number by 1, or re-roll the integer number, when it receives the QueryAdjust command. In our opinion, such principle of randomizing the AIOT device transmission/access occasion could be re-used for the 5G NR contention-based AIOT procedure. Moreover, presence of such parameter implies the requirement towards the paged AIOT devices to apply the contention-based access procedures.

Observation 2: in the RFID system, the reader sends a number Q to the target AIOT devices in the inventory message. Subsequently, each targeted AIOT device will generate a random integer number between 0 and 2Q-1. Only when the integer number equals to 0, it will perform access towards the network.

Proposal 3: RAN2 to agree on including a parameter in the AIOT paging message to randomize the access occasions for different AIOT devices and indicate the total number of access occasions in the contention-based access procedures.

Proposal 4: RAN2 to agree on the presence of the parameter in the AIOT paging message to randomize the access occasions for different AIOT devices implies the requirement towards the paged AIOT devices to apply the contention-based access procedures.

On the other hand, in our opinion, it is also possible to let the paging message indicate parameters necessary for triggering the multi-device contention-free random-access procedure. For instance, in paging message, the association of different dedicated access/transmission occasions and different AIOT device IDs could be included explicitly, though, this approach will suffer from overwhelmingly high signalling overhead if the AIOT paging message is targeted for a large group of AIOT devices. Hence, we also propose RAN2 to study how to reduce signalling overhead in the paging message to indicate dedicated access/transmission occasions towards more than one AIOT devices applying the contention-free random access procedure.

Proposal 5: RAN2 to agree on indicating the association of consecutive dedicated access/transmission occasions and different AIOT device IDs in the AIOT paging message explicitly for multi-device contention-free based random access procedure.

Observation 3: the approach of indicating the association of consecutive access/transmission occasions and different AIOT device IDs in the AIOT paging message explicitly for contention-free based random access procedure could suffer from overwhelmingly large signalling overhead if the AIOT paging message is targeted for a large group of AIOT devices, eg., more than 100 AIOT devices.

Proposal 6: RAN2 to study how to reduce signalling overhead in the paging message to indicate the dedicated access/transmission occasions towards more than one AIOT devices for the multi-device contention-free random access procedure.

In addition, as mentioned earlier, for the RFID, an asynchronous system, the indication of the current access occasion slot is managed by the network, similar mechanism should also be studied by RAN2. We think this function should be enabled by paging message.

Proposal 7: AIOT Paging message should be able to indicate the current access occasion.

- Conclusion and proposals

In the contribution, following proposals and observations are made:

Proposal 1: RAN2 to agree on including the purpose of the paging message, e.g., inventory, sending commands, in the paging message towards AIOT devices.

Observation 1: when reception of a AIOT paging message containing only one ID of a single AIOT device, the AIOT device could access to the network immediately, after reception of the AIOT paging message, subject to the restriction of any potential message processing delay.

Proposal 2: RAN2 to agree that indication of the transmission/access occasions related information in the AIOT paging message could be absent.

Observation 2: in the RFID system, the reader sends a number Q to the target AIOT devices in the inventory message. Subsequently, each targeted AIOT device will generate a random integer number between 0 and 2Q-1. Only when the integer number equals to 0, it will perform access towards the network.

Proposal 3: RAN2 to agree on including a parameter in the AIOT paging message to randomize the access occasions for different AIOT devices and indicate the total number of access occasions in the contention-based access procedures.

Proposal 4: RAN2 to agree on the presence of the parameter in the AIOT paging message to randomize the access occasions for different AIOT devices implies the requirement towards the paged AIOT devices to apply the contention-based access procedures.

Proposal 5: RAN2 to agree on indicating the association of consecutive dedicated access/transmission occasions and different AIOT device IDs in the AIOT paging message explicitly for multi-device contention-free based random access procedure.

Observation 3: the approach of indicating the association of consecutive access/transmission occasions and different AIOT device IDs in the AIOT paging message explicitly for contention-free based random access procedure could suffer from overwhelmingly large signalling overhead if the AIOT paging message is targeted for a large group of AIOT devices, eg., more than 100 AIOT devices.

Proposal 6: RAN2 to study how to reduce signalling overhead in the paging message to indicate the dedicated access/transmission occasions towards more than one AIOT devices for the multi-device contention-free random access procedure.

Proposal 7: AIOT Paging message should be able to indicate the current access occasion.