3GPP TSG-RAN WG2 Meeting #127                                     R2-2406719

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			OPPO

Title: 	Discussion on providing MBS service area in NTN network

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

In the last RAN2#126 meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area

- MBS broadcast intended service area is provided via system information.

- For MBS broadcast RAN2 considers the following possibilties for including the service area information: SIB20/SIB21/MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS sesssion (FFS the detail)

And in the RAN2#125bis meeting, discussion on the support of Broadcast service has been made, as follows:

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

In this paper, we would like to further present our opinions on the topic.

- Discussion

- 2.1 support of different MBS services/contents in cells covering big area in footprint of the satellite

As mentioned in the last RAN2 meeting agreement, the MBS broadcast intended service area is provided via system information. We aim to further discuss the case that different MBS broadcast is associated with different intended service area, as indicated as follows:

Figure 1: different sub-area belonging to a cell in/corresponding to a satellite footprint could be served with different MBS service

We think the information of different MBS broadcast intended service area for different MBS session could be embedded in the system information for the convenience of the UE to recognize the available MBS services at its current positioning and the areas of broadcasting interested MBS services in the currently camped cell.

Observation 1: if one cell covers a large area, different MBS sessions may be available in different sub-areas.

Proposal 1: RAN2 to agree that different MBS sessions could be associated with different MBS broadcast intended service areas, and such information should be indicated in the system information.

Another thing needs to be highlighted is that, in legacy, for the location dependent broadcast service, different contents for the same MBS session are distributed in different MBS service areas, according to [1]. In [2], the terminology area session ID is employed to identify each distinguished content belonging to a MBS session. In legacy, one MBS service area consists of list of cells or TAIs, so naturally only one content of a certain MBS session is distributed in each cell, and it is reasonable for the MBS configuration to not include the information of the area session ID. On the contrary, in the case one satellite footprint consists of only one cell or multiple cells with each covering a big area, regarding location dependent broadcast service, more than one contents for the same MBS session may be distributed in each cell. In this regard, for different contents, the corresponding area session ID and the associated area range should be provided in the MBS configuration information of one cell.

Observation 2: for the NTN network whose satellite footprint consists of one cell or multiple cells, and each of them covers a big area, regarding location dependent broadcast service, more than one contents for the same MBS session could be distributed in each cell.

Proposal 2: RAN2 to agree that, for the location dependent broadcast service, the area session ID of each distinguished content of the MBS session and the associated area range should be provided in the MBS configuration information.

- 2.2 support of MBS services/contents in cells area of more than one NTN cells (or portions thereof)

In our opinion, there are two possible situations when MBS services/contents are available in more than one NTN cells:

- MBS services are available in multiple cells

- MBS services are available in at least one whole cell and in only portion of at least one cells

If the MBS services are available in multiple cell, in each such cell, the targeting intended broadcast area does not need to be provided to UEs, since the UE camping on such cell can receive MBS services everywhere in the cell. On the other hand, if the MBS services are available in only part of a cell, related enhancements to be considered are mentioned in the last section. Anyway, we should embrace both of two situations: 1. MBS services are available in one whole cell, and 2. MBS services are available in part of one cell. Therefore, we suggest to mark any stage-3 enhancement for indicating intended targeting MBS service area in the NTN cell as optional.

Proposal 3: RAN2 to agree that, to support MBS services/contents in cell area of more than one NTN cells (or portions thereof), only the service area covering portions of cells need to be explicitly indicated.

- 2.3 how to indicate the service area information in the Uu signalling?

Currently, there are three options on table: including the service area information in SIB20, SIB21, or the MCCH consisting of MBSBroadcastConfiguration IE. According to [3], SIB20/21 contains the following information, respectively:

- the information required to acquire the MCCH/MTCH configurations for MBS broadcast

- the information related to service continuity for MBS broadcast reception. Especially, the UE is made aware of which frequency is providing which MBS broadcast services.

Note that different FSAIs included in the SIB21 is also mapped to different broadcast sessions, configured by the upper layer signalling from the network, according to [4]. On the other hand, according to [5], the MCCH carries the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session such as MBS session ID, associated G-RNTI scheduling information, etc. MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset.

Apparently, the current spec only arms the SIB20 with the functionalities of carrying the MCCH and MTCH scheduling information. The size of the SIB20 is relatively small, considering the details of the MBMS service information is further included in the MCCH, rather than itself. It seems that SIB20 informing of the intended service area information does not align with its current functionality. Another choice is to provide MCCH scheduling information per MBS session or, in a further step, per different area session for each location dependent MBS session. In such cases, the UE interested in not only one MBS service needs to monitor more than one MCCH simultaneously. Considering different MCCHs are scheduled at different time, such operation will introduce more complexity to UE.

Observation 3: It seems that SIB20 informing of the intended service area information does not align with its current functionality carrying the both of the MCCH and MTCH scheduling information.

Observation 4: providing a MCCH configurations per MBS session or, in a further step, per different area session for each location dependent MBS session in the SIB20 will let the UE interested in more than one MBS services needs to monitor more than one MCCHs simultaneously, which brings additional UE operation complexity.

In addition, a FSAI included the SIB21 is only able to be mapped to a MBS session, but not an area session belonging to a location dependent MBS session. As a result, for the location dependent broadcast service, we could not indicate the intended area for each area session, without significant change to the current spec.

Observation 5: the granularity of the mapping of FSAIs included the SIB21 is only the MBMS session, but not area session.

On the other hand, indicating the intended service area in the MBSBroadcastConfiguration seems more acceptable to UE. The logic behind is that the information of the intended service area is only viewed as one piece of MBS transmission related information; any UE interested in the MBMS service in the NTN network could find it from the MCCH, as the way how current UEs reads other MBS related detailed information from the MCCH also.

Proposal 4: RAN2 to agree to embed the intended area information in the MBSBroadcastConfiguration IE transmitted on the MCCH.

As for the ETWS, a similar situation with the MBS in the NTN network could occur, wherein earthquake and/or tsunami happens in only certain areas in a NTN cell. In the 5G NR system, following system information blocks are associated with the ETWS:

- SIB 6: contains ETWS primary notification

- SIB 7: contains ETWS secondary notification

According to [6], the Primary notification and the Secondary notification serve for the following purposes:

Primary Notification: is information which is used specifically in Earthquake and Tsunami Warning System in order to notify users about the most urgent event in seconds rather than minutes, such as imminent occurrence of Earthquake.

Secondary Notification: is information which is used specifically in Earthquake and Tsunami Warning System in order to notify users supplementary information that is of lesser urgency such as instructions on what to do / where to get help, for example, map to refuge facilities, time table of food distribution. These information, in general, would be issued by local government.

In our opinion, the area information of the upcoming disaster is vital and should be informed by the Primary Notification, to help people in coverage of the NTN network identify whether or not they need to take urgent actions.

Proposal 5: RAN2 to agree that ETWS in the NTN network could be associated with only certain geographical areas, and such areas could be indicated in SIB6, if the satellite footprint consists of one cell or multiple cells, and each of them covering large area.

- 2.4 representation of the intended area

Currently, in the SIB25, TN coverage information is broadcasted to assist UEs in a NTN cell to perform neighbour cell measurements. Specifically, a piece of CoverageAreaInfo IE composed of an area ID info, a reference Location co-ordinate and a distance Radius is included in the SIB25.

In our opinion, such IE could be reused to indicate the intended areas for the MBS services. Each MBS session could be associated with one or multiple of such areas, and for each such area included in each location dependent MBS session, an independent area session ID is optionally included to indicate the content distributed.

Proposal 6: RAN2 to agree to reuse the coverage area information included in the SIB25 to provide the information of the intended area for the MBS services and/or MBS session content for the NTN network in the SIB.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Observation 1: if one cell covers a large area, different MBS sessions may be available in different sub-areas.

Proposal 1: RAN2 to agree that different MBS sessions could be associated with different MBS broadcast intended service areas, and such information should be indicated in the system information.

Observation 2: for the NTN network whose satellite footprint consists of one cell or multiple cells, and each of them covers a big area, regarding location dependent broadcast service, more than one contents for the same MBS session could be distributed in each cell.

Proposal 2: RAN2 to agree that, for the location dependent broadcast service, the area session ID of each distinguished content of the MBS session and the associated area range should be provided in the MBS configuration information.

Proposal 3: RAN2 to agree that, to support MBS services/contents in cell area of more than one NTN cells (or portions thereof), only the service area covering portions of cells need to be explicitly indicated.

Observation 3: It seems that SIB20 informing of the intended service area information does not align with its current functionality carrying the both of the MCCH and MTCH scheduling information.

Observation 4: providing a MCCH configurations per MBS session or, in a further step, per different area session for each location dependent MBS session in the SIB20 will let the UE interested in more than one MBS services needs to monitor more than one MCCHs simultaneously, which brings additional UE operation complexity.

Observation 5: the granularity of the mapping of FSAIs included the SIB21 is only the MBMS session, but not area session.

Proposal 4: RAN2 to agree to embed the intended area information in the MBSBroadcastConfiguration IE transmitted on the MCCH.

Proposal 5: RAN2 to agree that ETWS in the NTN network could be associated with only certain geographical areas, and such areas could be indicated in SIB6, if the satellite footprint consists of one cell or multiple cells, and each of them covering large area.

Proposal 6: RAN2 to agree to reuse the coverage area information included in the SIB25 to provide the information of the intended area for the MBS services and/or MBS session content for the NTN network in the SIB.

Reference:

[1] TS 23.247 V18.4.0, Architectural enhancements for 5G multicast-broadcast services, Stage 2, SA.

[2] TS 38.413 V17.2.0, NG Application Protocol (NGAP), RAN

[3] TS 38.331 V18.0.0, Radio Resource Control (RRC) protocol specification, 3GPP RAN2.

[4] S2-2108175, Reply LS on MBS broadcast service continuity and MBS session identification, SA2

[5] TS 38.300 V18.0.0, NR and NG-RAN Overall description, RAN2

[6] TS 22.168 V8.3.0, Earthquake and Tsunami Warning System (ETWS)