3GPP TSG-RAN WG2 Meeting #127                                     R2-2406720

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			OPPO

Title: 	Discussion on topology 2 for Ambient IOT

Agenda item:		8.2.5

Document for: 	Discussion and Decision

- Introduction

In the RAN plenary meeting #102, the A-IoT SID has been approved, as descripted in [1]. In particular, Topology 2 with UE as the intermediate node, under network control is one of the deployment scenarios, as indicated as follows:

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

In this paper, we would like to provide our views on the Topology 2 for the A-IOT system

2. Discussion

- 2.1 Fulfil the routing functionality at the intermediate UE

In the topology 2, the destination/source ID of the A-IOT device should be recognized by the intermediate UE/gNB to route the A-IOT data packets (including command & data) to the correct A-IOT device/5GC for R2D/D2R transmission. Next, we will analyse how to achieve this from two perspectives, R2D and D2R, respectively. In addition, the gNB needs to build a routing table associating A-IOT devices with the intermediate UE ID.

Observation 1: in the topology 2, the destination/source ID of the A-IOT device should be recognized by the intermediate UE/gNB to route the A-IOT data packets (including command & data) to the correct A-IOT device/5GC for R2D/D2R transmission.

Observation 2: in the topology 2, the gNB needs to build a routing table associating the A-IOT device to the intermediate UE ID.

- 2.1.1 R2D perspective

For the R2D direction, in our opinion, the A-IOT device destination information should be added by the gNB or the 5GC.

If the destination information is added by the gNB, there exist two options. The first one is to introduce the destination information in a header added by a new layer in the Uu side. Another choice is to let the legacy Uu top protocol layer (e.g., RRC or SDAP) write the destination information. In our opinion, to avoid enforcing impact on the current protocol layer, the first option to introduce a new adapt layer is preferred. Note that since both of the Uu protocol stacks located at the intermediate UE and the gNB have to be enhanced to embrace this new protocol layer, the gNB is able to build the routing table according to the A-IOT information provided in the header of the new layer in the feedback D2R transmission containing the ACK of the R2D transmission, or the D2R data, as indicated in the figure 1.

On the other hand, if the AIOT device destination information is added by the 5GC and could be further interpreted by the intermediate UE Au-side protocol stack for further transmission over the Au interface, then the task of transmission over the Au interface is also accomplished. However, to help the gNB build the routing table to avoid flooding the R2D transmission to all intermediate UEs again in the subsequent message transmission, confirmation of successful transmission of R2D data and/or command towards a particular A-IOT device should be transmitted by the intermediate UE to the gNB. Such requirement may have spec impact, e.g., to let the intermediate UE ‘s RRC encode the A-IOT device ID in the message containing the ACK or the R2D information back to the gNB, after the prior D2R transmission.

The following figure illustrates how the gNB can build the routing table for an A-IOT device and the intermediate UE .

Figure 1: the intermediate UE sends back the ACK of R2D or the corresponding D2R message to help the gNB build the routing table for the A-IOT device and the intermediate UE

Observation 3: there exist three options regarding adding A-IOT device information in the R2D transmission:

- by the gNB: to introduce the destination information in a header added by a new layer in the Uu side

- by the gNB: legacy Uu top protocol layer (e.g., RRC or SDAP) to write the destination information.

- by the 5GC: the destination information could be added in the NAS signalling or by the PDU layer.

Observation 4: the option of letting a legacy Uu protocol layer (e.g., RRC or SDAP) write the destination information in the PDU requires the legacy Uu protocol layer to have new function, which introduces spec impact.

Observation 5: the option of adding the A-IOT device information by the 5GC enforce the intermediate UE to provide the A-IOT device information to the gNB to build the routing table associating A-IOT devices with the intermediate UE to avoid flooding the R2D transmission over all UEs under the coverage.

Observation 6: the option of introducing a new layer for adding A-IOT device information introduces the least spec impact.

- 2.1.2 D2R perspective

For the D2R direction, in our opinion, the A-IOT device source information could be added by the A-IOT device itself or by the intermediate UE. Naturally, the source information of the A-IOT device could be known by intermediate UE due to the performed/on-going A-IOT RACH procedure.

If the source information is added by the A-IOT device, it could be simply added in the AIOT higher layer. In such case, the source information is transparent to the gNB and recognized by the 5GC directly.

On the other hand, if the source information is added by the intermediate UE, similar as the mentioned above, a new layer is suggested to take the task. Note that the information of the A-IOT device ID could be known by the intermediate UE by performing the A-IOT RACH towards the A-IOT device.

Proposal 1: RAN2 to discuss the following two options for adding A-IOT device source/destination information with the D2R/R2D transmission:

- The A-IOT source/destination information is added by the AIOT device/5GC in the AIOT higher layer.

- The A-IOT source/destination information is added by the intermediate UE/gNB in a new adapt layer on top of the legacy Uu protocol stack

- 2.2. Protocol stack impact

As discussed above, only if the A-IOT source/destination information is added by the intermediate UE and the gNB, the current Uu protocol stack is impacted. In addition, since there is no Uu protocol stack resided in the Ambient IOT device, the R2D/D2R data PDU needs to be processed by the full Uu protocol stack at the intermediate UE/gNB, before conveying to the Au-side protocol stack for further processing. If the adapt layer is placed in-between any two protocol layers of the legacy Uu protocol stack, the uncertainty of the processing time of the PDU by the Uu layer(s) located higher than the adapt layer could force the intermediate UE Au-side protocol stack/gNB N2-side protocol stack to wait for an uncertain amount of time of finishing of the processing of the PDU by the remaining Uu protocol stacks, before triggering further transmission over the Au or the N2 interface, which introduces further complexity. An exemplary implementation for R2D transmission at the intermediate UE is shown below. As a result, the newly introduced adapt layer is suggested to be placed on the top of the legacy Uu protocol stack.

Figure 2: an exemplary implementation for the introduced adapt layer resided in-between two legacy Uu protocol layers.

Observation 7: since there is no Uu protocol stack resided in the Ambient IOT device, the R2D/D2R data needs to be processed by the full Uu protocol stack at the intermediate UE/gNB, after receiving it over the Uu interface between gNB and UE.

Observation 8: if the adapt layer is placed in-between any two legacy Uu protocol layers, the processing delay of the PDU by the Uu layer(s) located higher than the adapt layer could force the intermediate UE Au-sided protocol stack/gNB N2-side protocol stack to wait an uncertain amount of time of finishing of the processing of the PDU by the remaining Uu protocol stacks, before triggering further transmission over the Au or the N2 interface.

Proposal 2: RAN2 to agree that the adapt layer, if agreed to be introduced, for processing the source/detonated A-IOT device info is placed on the top of the legacy Uu protocol layer at the intermediate UE and gNB.

For the Au-side protocol stack for the topology 2, we think it is OK to follow the topology 1. Note that in the RAN2#125bis meeting, we have achieved the following assumption:

- RAN2 assumes that RRC layer is not necessary between the gNB and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

As a result, the proposed control-plane based and user-plane based protocol stack are illustrated as follows.

Figure 3-1: the control plane protocol stack for the topology 2, if the new adapt layer needs to be introduced

Figure 3-2: the control plane protocol stack for the topology 2, if the device ID information is added by the 5GC in the UE NAS layer

Figure 3-3: the user plane protocol stack for the topology 2, if the new adapt layer needs to be introduced

Proposal 3: RAN2 to adopt the protocol stack shown in figure 3 as the baseline for topology 2 protocol stack further discussion.

- 2.4 transmission over Au interface for the topology 2

After reception of the A-IOT data/command from the gNB and recognition of the destined A-IOT device information, the intermediate UE needs to perform the transmission towards the A-IOT device over the Au interface. As we have already discussed to apply the A-IOT RACH procedure to accomplish such task for the topology 1, such principle could be reused for the topology 2 also. Similarly, for the R2D transmission, the intermediate UE could also poll A-IOT devices aperiodically/periodically for possible data to be sent towards the network by performing the A-IOT RACH procedure.

Proposal 4: RAN2 to agree to apply the same A-IOT RACH procedure as for the topology 1 for performing A-IOT data transmission over the Au interface between the A-IOT device and the intermediate UE.

- Conclusion and proposals

Observation 1: in the topology 2, the destination/source ID of the A-IOT device should be recognized by the intermediate UE/gNB to route the A-IOT data packets (including command & data) to the correct A-IOT device/5GC for R2D/D2R transmission.

Observation 2: in the topology 2, the gNB needs to build a routing table associating the A-IOT device to the intermediate UE ID.

Observation 3: there exist three options regarding adding A-IOT device information in the R2D transmission:

- by the gNB: to introduce the destination information in a header added by a new layer in the Uu side

- by the gNB: legacy Uu top protocol layer (e.g., RRC or SDAP) to write the destination information.

- by the 5GC: the destination information could be added in the NAS signalling or by the PDU layer.

Observation 4: the option of letting a legacy Uu protocol layer (e.g., RRC or SDAP) write the destination information in the PDU requires the legacy Uu protocol layer to have new function, which introduces spec impact.

Observation 5: the option of adding the A-IOT device information by the 5GC enforce the intermediate UE to provide the A-IOT device information to the gNB to build the routing table associating A-IOT devices with the intermediate UE to avoid flooding the R2D transmission over all UEs under the coverage.

Observation 6: the option of introducing a new layer for adding A-IOT device information introduces the least spec impact.

Proposal 1: RAN2 to discuss the following two options for adding A-IOT device source/destination information with the D2R/R2D transmission:

- The A-IOT source/destination information is added by the AIOT device/5GC in the AIOT higher layer.

- The A-IOT source/destination information is added by the intermediate UE/gNB in a new adapt layer on top of the legacy Uu protocol stack

Observation 7: since there is no Uu protocol stack resided in the Ambient IOT device, the R2D/D2R data needs to be processed by the full Uu protocol stack at the intermediate UE/gNB, after receiving it over the Uu interface between gNB and UE.

Observation 8: if the adapt layer is placed in-between any two legacy Uu protocol layers, the processing delay of the PDU by the Uu layer(s) located higher than the adapt layer could force the intermediate UE Au-sided protocol stack/gNB N2-side protocol stack to wait an uncertain amount of time of finishing of the processing of the PDU by the remaining Uu protocol stacks, before triggering further transmission over the Au or the N2 interface.

Proposal 2: RAN2 to agree that the adapt layer, if agreed to be introduced, for processing the source/detonated A-IOT device info is placed on the top of the legacy Uu protocol layer at the intermediate UE and gNB.

Proposal 3: RAN2 to adopt the protocol stack shown in figure 3 as the baseline for topology 2 protocol stack further discussion.

Proposal 4: RAN2 to agree to apply the same A-IOT RACH procedure as for the topology 1 for performing A-IOT data transmission over the Au interface between the A-IOT device and the intermediate UE.