3GPP TSG-RAN WG2 Meeting #127	R2-2406721

Maastricht, Holland, Aug 19th – Aug 23th, 2024

Agenda Item:	8.5.2

Source: 	vivo

Title:         	Discussion on on-demand SSB SCell operation

Document for: 	Discussion and Decision

- Introduction

In the Rel-19 network energy saving enhancement WI [1], one objective is on-demand SSB (OD-SSB) SCell operation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

RAN1 has made some progress for this objective, which are shown in the Appendix. With this progress, this paper will provide our preference on the scenarios for this objective, and discuss RAN2 spec impacts and high-level solutions for the scenarios agreed in RAN1.

- Discussion

  - Scenarios

In RAN1#116bis meeting, RAN1 has agreed OD-SSB can be indicated by the network at least when the SCell is in deactivated state (i.e., Scenario #2), or when the UE receives SCell activation command (i.e., Scenario #2A). Whether to support Scenario #3A and Scenario #3B are still FFS.

We understand Scenario #3A has the similar gain as the Scenario #2A, i.e., for fast SCell activation. Thus, whether to support Scenario #3A may depend on what solution will be adopted for Scenario #2A. If two separate signaling are used to indicate SCell activation and OD-SSB transmission respectively for Scenario #2A, there is no additional standard effort in RAN2 to also support Scenario #3A, otherwise, we don’t see the need to support Scenario #3A on top of Scenario #2A.

For Scenario #3B, the UE needs to monitor PDCCH periodically based on C-DRX cycle for synchronization when the SCell is in activated, so there should have periodic SSB when the SCell is in activated state. And there is no clear gain of triggering OD-SSB when the SCell is in activated with periodic SSB transmitted with up to 160 ms cycle.

RAN1#116bis Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Given above, we have the following proposals:

- On the basis of Scenario 2A, Scenario #3A (i.e., OD-SSB is indicated after UE receives SCell activation command until SCell activation is completed) can be supported if no additional standard effort is required.

- Scenario #3B (i.e., OD-SSB is indicated after SCell activation completion) is not supported.

Moreover, based on RAN1 agreement, there could be no always-on SSB when the SCell is in deactivated, and as we explained above, we understand there should have periodic SSB when the SCell is in activated. And considering OD-SSB is transmitted during SCell activation period, so it is more reasonable that the UE assumes there is periodic SSB after the SCell activation is completed.

- If there is no always on SSB before SCell activation, the UE assumes there is periodic SSB after the SCell activation complete. FFS the configuration of the periodic SSB (e.g., reuse legacy SSB configuration with a new initial status of legacy SSB configuration, or introduce a new periodic SSB configuration).

  - Indication of OD-SSB

RAN1#117 meeting discussed the signalling design and has agreed to support RRC based signaling and MAC CE based signaling to indicate OD-SSB transmission on the SCell. We will first discuss the below FFS.

RAN1#117 Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable

From our understanding, RRC based OD-SSB transmission indication can be used to indicate the initial activation state of OD-SSB configuration for both Scenarios #2 and #2A.

- RRC based OD-SSB transmission indication is used to indicate the initial activation state of OD-SSB configuration for Scenarios #2 and #2A.

SCell state can be indicated by the SCell A/D MAC CE at any time when the SCell has been configured. So, MAC CE based OD-SSB transmission indication is mainly used to activate the OD-SSB transmission when the SCell is in deactivated for scenario #2 or indicate the activation state of OD-SSB configuration when activating the SCell via MAC CE for scenario #2A.

- MAC CE based OD-SSB transmission indication is used to activate the OD-SSB when the SCell is in deactivated (i.e., Scenarios 2) or indicate the activation state of OD-SSB when activating the SCell via MAC CE (i.e., Scenarios 2A).

Based on the below agreement made in RAN1#116bis, there are two alternatives for MAC CE based OD-SSB transmission indication. From our understanding, pursuing an unified solution for Scenario #2 and #2A is inefficient. We prefer to introduce a new MAC CE to only indicate the OD-SSB transmission for Scenario #2. For Scenario #2A, the coexistence of Rel-17 A-TRS scheme should be considered. In Rel-17, Enhanced SCell Activation/Deactivation MAC CE is introduced for A-TRS based fast known SCell activation, while OD-SSB is used to reduce the SCell activation delay for unknown SCell, and we think both cases can happen at the same time for different SCells being activated.

RAN1#116bis agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

In order to support activating the SCell and indicating the A-TRS for known SCell and the OD-SSB for unknown SCell simultaneously, one way is to extend the Enhanced SCell Activation/Deactivation MAC CE, since this MAC CE has a variable size. There are different ways to extend. For example, the scellActivationRS-Id included in TRS ID field can be linked to one OD-SSB configuration or one A-TRS configuration, but this will constraint the maximum number of A-TRS resources and OD-SSB resources. Another example is to add OD-SSB ID field which is referred to one OD-SSB configuration, and this field is present if the related SCell is activated and the corresponding OD-SSB configuration is configured for this SCell.

Figure 1: Reuse the TRS ID field.

Figure 2: Introduce OD-SSB ID field.

- For Scenario 2, introduce a new MAC CE to indicate the OD-SSB transmission.

- For Scenario 2A, extend the Enhanced SCell Activation/Deactivation MAC CE to support activating the SCell and indicating the A-TRS or OD-SSB of the SCell.

  - Configuration of OD-SSB

In RAN1#117 agreement, RAN1 has agreed the below parameters for OD-SSB.

RAN1#117 Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

Before OD-SSB is indicated, there may have always on SSB. The time offset/periodicity/duration of OD-SSB could be different than that of always on SSB. So, new time offset/periodicity/duration configurations should be introduced for indicating the OD-SSB resources. In addition, if the OD-SSB and the always on SSB on the same frequency, then current SSB frequency parameter can be reused. If the OD-SSB and always on SSB are not on the same frequency, then an additional OD-SSB frequency configuration should be indicated to the UE.

- The below parameters are newly introduced for configuring OD-SSB:

- time offset/periodicity/duration configurations of OD-SSB.

- frequency of OD-SSB, if the OD-SSB and always on SSB are not on the same frequency.

  - L3 measurement and reporting

In RAN1#116bis meeting, L1 and/or L3 measurement based on on-demand SSB is supported for a cell supporting on-demand SSB SCell operation. RAN2 can focus on L3 measurement based on OD-SSB.

Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement

It is clear that the UE performs L3 OD-SSB measurement based on OD-SSB configuration after receiving the OD-SSB indication from the network. However, how to report the measurement results of OD-SSB is unclear and should be discussed. The below listed options can be considered for triggering the report.

- Option 1: report is triggered by measurement report configuration (i.e., periodic, existing/new/separate event);

- Option 2: report is triggered by a new MAC CE sent from the network;

- Option 3: report is triggered after a certain time following the reception of the MAC CE used for indicating OD-SSB.

In option 1, if the OD-SSB configuration is provided under SCell configuration, how to provide the report configuration for this SCell needs to be discussed, for example, the network can explicitly configure a whole set of parameters of report event under SCell configuration, or configure an associated measurement report ID. If the OD-SSB configuration is provided under serving MO configuration, then the existing MR configuration can be reused. Moreover, a separate event can be considered for OD-SSB measurement, if the legacy event is used for the always on SSB measurement. And in option 2/3, network can determine when to collect the available results from the UE based on the arrival of traffic.

Regarding to the signaling of OD-SSB measurement report, we think RRC or MAC CE signaling can be used. MAC CE signaling is more suitable for the CU-DU split case, where the OD-SSB transmission is indicated by the DU via MAC CE, so it would be better to send the measurement report of OD-SSB by the MAC CE to the DU to enable the DU to determine which SCells should be activated. The signaling of OD-SSB measurement report can be discussed after we have concluded how to trigger the OD-SSB measurement report.

- RAN2 to discuss the below options to trigger the OD-SSB measurement report:

- Option 1: report is triggered by measurement report configuration (i.e., periodic, existing/new/separate event);

- Option 2: report is triggered by a new MAC CE sent from the network;

- Option 3: report is triggered after a certain time following the reception of the MAC CE used for indicating OD-SSB.

- Conclusion

In this paper, the following proposals are given:

Scenarios:

- On the basis of Scenario 2A, Scenario #3A (i.e., OD-SSB is indicated after UE receives SCell activation command until SCell activation is completed) can be supported if no additional standard effort is required.

- Scenario #3B (i.e., OD-SSB is indicated after SCell activation completion) is not supported.

- If there is no always on SSB before SCell activation, the UE assumes there is periodic SSB after the SCell activation complete. FFS the configuration of the periodic SSB (e.g., reuse legacy SSB configuration with a new initial status of legacy SSB configuration, or introduce a new periodic SSB configuration).

Indication of OD-SSB:

- RRC based OD-SSB transmission indication is used to indicate the initial activation state of OD-SSB configuration for Scenarios #2 and #2A.

- MAC CE based OD-SSB transmission indication is used to activate the OD-SSB when the SCell is in deactivated (i.e., Scenarios 2) or indicate the activation state of OD-SSB when activating the SCell via MAC CE (i.e., Scenarios 2A).

- For Scenario 2, introduce a new MAC CE to indicate the OD-SSB transmission.

- For Scenario 2A, extend the Enhanced SCell Activation/Deactivation MAC CE to support activating the SCell and indicating the A-TRS or OD-SSB of the SCell.

Configuration of OD-SSB:

- The below parameters are newly introduced for configuring OD-SSB:

- time offset/periodicity/duration configurations of OD-SSB.

- frequency of OD-SSB, if the OD-SSB and always on SSB are not on the same frequency.

L3 measurement and reporting:

- RAN2 to discuss the below options to trigger the OD-SSB measurement report:

- Option 1: report is triggered by measurement report configuration (i.e., periodic, existing/new/separate event);

- Option 2: report is triggered by a new MAC CE sent from the network;

- Option 3: report is triggered after a certain time following the reception of the MAC CE used for indicating OD-SSB.

- References

[1] RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson.

- Appendix (RAN1 agreements)

RAN1#117

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

RAN1#116bis

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement) Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

RAN1#116

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE Agreement - For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)