3GPP TSG-RAN WG2 Meeting #127		R2-2406723

Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Agenda Item:	8.5.4

Source:	vivo

Title:	Discussion on adaptation on common signal transmissions

Document for:	Discussion and Decision

Introduction

In RAN2#125bis meeting, adaptation of paging occasion for network energy saving was further discussed with the below agreements [1]:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. - For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. - For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, we will further provide our consideration on the adaptation of paging occasions in time domain.

Discussion

## 2.1 Adaptation of paging occasions in the time domain

To ensure the paging performance of the legacy UEs, the NW needs to configure at least two legacy PFs in each DRX cycle, i.e., T = 32 frames and N = oneSixteenthT. For Rel-19 UEs, in RAN2#125bis meeting, RAN2 has identified the following two options [1]:

- Option 1: Bundle paging frames;

- Option 2: Extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF; (It is shown in Figure 3)

We think option 1 has the two following sub-options:

- Option 1a: Bundle the PFs of Rel-19 UEs with the PFs of legacy UEs (an example is shown in Figure 1);

- Option 1b: Bundle all PFs of Rel-19 UEs (an example is shown in Figure 2);

From our point of view, Option 1a decreases the RACH collision probability compared to Option 1b since the Rel-19 UEs are divided into two groups with the legacy PFs. Option 1b is similar to option 2.

For the above two options, the pros/cons and examples are summarized in the following Table 1.

| Options | Pros | Cons | Using figures as examples: |
| --- | --- | --- | --- |
| Option1 | It decreases RACH collision | Spec impacts to change PF formula. | Option 1a: Figure 1. Bundle PFs of Rel-19 UEs with the PFs of legacy UEs |
|  |  | - It increases RACH collision. - Spec impacts to change PF formula. | Option 1b: Figure 2. Bundle all PFs of Rel-19 UEs |
| Option2 | Minimum spec impact. | It increases RACH collision. | Figure 3. One paging frame with more paging occasions. |

Table 1: Comparison of options.

Proposal 1: RAN2 to discuss and clarify whether paging adaptation option1 ‘Bundle paging frames’ has the following sub-options and their impact:

- Option 1a: Bundle all PFs of Rel-19 UEs;

- Option 1b: Bundle the PFs of Rel-19 UEs with the PFs of legacy UEs;

PF configuration for Rel-19 UE:

The legacy UEs still monitor the paging transmission on legacy PFs. For the Rel-19 UEs, if the NW does not configure the separate paging configuration, the Rel-19 UEs can monitor the paging transmission on legacy PFs. If the NW configures the separate paging configuration, there are two options can be considered:

- Alt 1: The PF of a Rel-19 UE can only be configured in additional PF that is separate from the legacy PF;

- Alt 2: The PF of a Rel-19 UE can either be configured in legacy PF or additional PF;

For Alt 1, the NW only pages the Rel-19 UEs on Rel-19 PFs and the Rel-19 UEs can only be mapped to Rel-19 PFs. For Alt 2, the NW has the flexibility to adjust the load in PFs. For instance, if the NW estimates that the load of legacy UEs is low, the NW can reduce the number of Rel-19 PFs and a part of Rel-19 UEs can be assigned to legacy PFs. For example, the Rel-19 UEs can report the capability of supporting paging adaptation to AMF (the AMF forwards this capability information to anchor gNB for RAN paging), and the serving gNB can estimate the ratio of the legacy UEs and the Rel-19 UEs based on this UE capability included in the NG/Xn paging messages. Or, the AMF/anchor gNB may take this UE capability into account when configuring tracking/RAN notification areas.

Proposal 2: RAN2 to discuss how the the PF for Rel-19 UEs is configured:

- Alt 1: The PF of a Rel-19 UE can only be configured in additional PF that is separate from the legacy PF;

- Alt 2: The PF of a Rel-19 UE can either be configured in legacy PF or additional PF ;

Option 1a: Bundle the PFs of Rel-19 UEs with the PFs of legacy UEs

In order to bundle Rel-19 PFs with legacy PFs and combine the above two alts, an unified formula can be introduced:

Rel-19 PF = legacy PF +[ (UE_ID mod (N_new/N+factor)) – (factor-1)]

Where N_new is the number of Rel-19 PFs. N is the number of legacy PFs. Factor is a parameter on monitoring behaviors which can be set to 0 or 1 corresponding to alt1 or alt2 respectively. To be specific, the UE firstly  determines the legacy PF according to the legacy formula, and then determines the Rel-19 PFs based on the above new formula.

Figure 5. Bundle the PFs of Rel-19 UEs with the PFs of legacy UEs

For example, as shown in the figure 5. There are two bundle groups due to the number of legacy PFs. Assuming that there are six Rel-19 UEs in the cell, the legacy PF that the UEs (e.g., UE0,UE2,UE4) need to monitor is SFN0 according to the legacy formula. The legacy PF that the UEs (e.g., UE1,UE3,UE5) need to monitor is SFN16. Each group has three UEs respectively. When the load ratio of legacy UEs is relatively high, the factor can be set to 0. Three UEs of each group can be mapped to Rel-19 PFs (e.g., SFN1,2,3 for group1 or SFN17,18,19 for group2) respectively based on the new formula, which is corresponding to Alt 1. When the load of legacy UEs is low, the NW can reduce the number of Rel-19 PFs and a part of Rel-19 UEs can be assigned to legacy PFs. For example, N_new can be reduced to 4 and the factor is set to 1. Three UEs of each group can be mapped to Rel-19 PFs and legacy PFs (e.g., SFN0,1,2 for group1 or SFN16,17,18 for group2), which is corresponding to Alt 2.

Proposal 3：For option 1a, if RAN2 agrees to support both alternatives in Proposal 2, an unified formula can be introduced:

- Rel-19 PF = legacy PF +[ (UE_ID mod (N_new/N+factor)) – (factor-1)]

Option 1b: Bundle all PFs of Rel-19 UEs

For bundle all PF of Rel-19 UEs, the NW can indicate the Rel-19 PFs position (e.g., list of SFNs). For example, as shown in the figure 6, there are six Rel-19 PFs in T, which can be SFNs [1,2,3,4,5,6] indicated by the NW directly.

Figure 6. Bundle all PFs of Rel-19 UEs

To be specific, the Rel-19 PFs are determined by the following potential formula:

(SFN +PF_offset) mod T= SFN [(UE_ID mod N)]

(UE_ID mod N) is equal to the index of entry in the SFN list. For Alt 1, N is corresponding to the number of Rel-19 PFs and the SFN list only includes the Rel-19 PFs. For Alt 2, N is the number of Rel-19 PFs and legacy PFs and the SFN list includes the Rel-19 PFs and legacy PFs.

Proposal 4：For option 1b, RAN2 consider the following formula to determine the PF:

- (SFN +PF_offset) mod T= SFN [(UE_ID mod N)]

Legacy UE handling:

In RAN2#125bis meeting, RAN2 discussed the following options on compatibility of legacy RRC_IDLE and RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs);

Based on the following note for adaptation of common signal/channel transmissions in NES WID [2], the legacy UEs can camp on the cell configured with paging adaptation feature. Therefore, the target scenario in Rel-19 NES is a single cell with the legacy UEs and the Rel-19 UEs supporting paging adaptation. Option 1 is not in the scope of R19 NES WID.

- Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown.

Observation 1: The enhancement on paging adaptation in Rel-19 should not impact the legacy UEs.

Proposal 5: The NW does not bar legacy UEs for the purpose of paging adaptation.

Conclusion

Based on the discussion, we have the following proposals:

Proposal 1: RAN2 to discuss and clarify whether paging adaptation option1 ‘Bundle paging frames’ has the following sub-options and their impact:

- Option 1a: Bundle all PFs of Rel-19 UEs;

- Option 1b: Bundle the PFs of Rel-19 UEs with the PFs of legacy UEs;

Proposal 2: RAN2 to discuss how the the PF for Rel-19 UEs is configured:

- Alt 1: The PF of a Rel-19 UE can only be configured in additional PF that is separate from the legacy PF;

- Alt 2: The PF of a Rel-19 UE can either be configured in legacy PF or additional PF ;

Proposal 3：For option 1a, if RAN2 agrees to support both alternatives in Proposal 2, an unified formula can be introduced:

- Rel-19 PF = legacy PF +[ (UE_ID mod (N_new/N+factor)) – (factor-1)]

Proposal 4：For option 1b, RAN2 consider the following formula to determine the PF:

- (SFN +PF_offset) mod T= SFN [(UE_ID mod N)]

Observation 1: The enhancement on paging adaptation in Rel-19 should not impact the legacy UEs.

Proposal 5: The NW does not bar legacy UEs for the purpose of paging adaptation.

# 4. Reference

- RAN2#126 meeting minutes

- RAN2#125bis meeting minutes.

- 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11th-15th, 2023, New WID: Enhancements of network energy savings for NR, RP-234065.