3GPP TSG-RAN WG2 Meeting #127	R2-2406724

Maastricht, Netherlands, 19th Aug.– 23rd Aug., 2024

Agenda Item:	8.11.2

Source:	vivo

Title:	Discussion on random access in SBFD

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the general RACH issues and random access procedure in SBFD.

Discussion

General issues

RACH triggering events

According to [1], a number of events that can trigger random access procedure are defined. Some of them are listed as the below table:

| RA triggered by events | CFRA | CBRA |
| --- | --- | --- |
| DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised" | √ | √ |
| Explicit request by RRC upon synchronous reconfiguration (e.g. Handover) | √ | √ |
| To establish time alignment for a secondary TAG | √ | √ |
| Request for Other SI | √ | √ |
| Beam failure recovery for SpCell | √ | √ |
| Initial access from RRC_IDLE |  | √ |
| RRC Connection Re-establishment procedure |  | √ |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available |  | √ |
| SR failure |  | √ |

One issue is whether all the RACH triggering events can be supported in SBFD. From our point of view, ROs in SBFD symbols or across SBFD/non-SBFD symbols serve as supplementary RACH resources on top of ROs in non-symbols. One of the characteristics of performing RACH procedure in SBFD is that it may cause certain level of CLI to other UEs performing RACH procedure in SBFD of the same or neighboring cells, which should be minimized as much as possible. In fact, UEs performing RACH procedure for some of the triggering events may cause greater CLI than other triggering events, e.g. mobility related triggering events.

However, we think it is not necessary to forbid some of the RACH triggering events in SBFD if only considering minimization of CLI. Such optimization can be up to NW deployment and implementation. Therefore, we propose that all the current RACH triggering events are supported in SBFD.

Proposal 1: All the current RACH triggering events are supported in SBFD.

Preample partitioning

So far, RedCap, SDT, NSAG and Msg3 Repetition are features that support to be identified by preamble partitioning. For SBFD, the SBFD aware UEs can be identified by transmitting preamble on ROs in SBFD symbols or across SBFD/non-SBFD symbols. Hence, there is no need to support SBFD aware UEs to be identified by preamble partitioning from one aspect. From another aspect, although the NW cannot identify SBFD aware UEs if they transmit preamble in ROs in non-SBFD symbols, there is no proof that the system performance can improve a lot if the NW can know such UEs support SBFD earlier during RACH procedure. Last but not least, preamble partitioning has been over utilized and we should be careful to add new feature to it.

Proposal 2: SBFD is not considered as a new feature for PRACH preamble partitioning, i.e. the NW does not configure SBFD as a feature as ‘true’ for a set of random access resources.

random access procedure

Carrier selection

A serving cell can be configured with supplementary uplink (SUL) for the purpose of coverage enhancement. In the Rel-18 WI Coverage Enhancement (CovEnh), it was discussed and determined that:

- Msg1 repetition can be configured on SUL;

- It is up to the NW whether to configure both NUL with Msg1/3 repetition and SUL for the purpose of CovEnh;

In SBFD, we propose that similar principle can also be agreeable.

Proposal 4: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement.

The next issue is whether SBFD operation has an impact on carrier selection for random access procedure. In legacy procedure, the UE directly determines the UL carrier if it is explicitly indicated by the NW. Otherwise, the UE compares the DL RSRP with the NW configured SUL threshold to determine it [2].

In SBFD, the NW may enable Msg1 repetition in SBFD resources on TDD-NUL (although it has not been agreed yet). The UE compares the DL RSRP with Msg1 repetition threshold to determine the number of repetition. Msg1 repetition enhance the NUL coverage to some extent, which weakens the motivation for the UE to select SUL. Therefore, we propose RAN2 to discuss whether the NW configuring SBFD on NUL has an impact on NUL/SUL selection. For example, if the UE selects to perform SBFD operation on NUL, the UE can rely on Msg1 repetition with SBFD on NUL instead of choosing SUL.

Proposal 5: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

SSB/CSI-RS selection is a step in random access procedure. If not indicated by the NW, the UE compares the RSRP of the SSB/CSI-RS with the threshold and select an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. The UE then determines RO based on the selected SSB/CSI-RS [2].

In SBFD, ROs in SBFD resources and non-SBFD resources can be both mapped to the same SSB/CSI-RS. Considering the NW can evaluate the CLI in SBFD resources based on the UE reports and adjust the SBFD configuration accordingly, the NW may configure different threshold for the UE to select ROs in SBFD resources or non-SBFD resources to minimize the CLI in SBFD resources. However, we may also just rely on the legacy SSB/CSI-RS selection method. We propose RAN2 to discuss whether SBFD operation has an impact on SSB/CSI-RS selection for random access procedure.

Proposal 6: RAN2 to discuss whether SBFD operation has an impact on SSB/CSI-RS selection for random access procedure.

RO selection

For RO selection, the legacy UE selects randomly amongst the consecutive PRACH occasions once the SSB/CSI-RS for RACH is determined, such as [2]:

1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

In SBFD, the key issue for RO selection is whether we need to differentiate SBFD ROs (ROs in SBFD symbols, or across SBFD/non-SBFD symbols per RAN1 agreement [3]) and non-SBFD ROs. As such, there are several options for RO selection in SBFD:

- Option 1: An SBFD aware UE prioritizes selection of SBFD ROs or non-SBFD ROs;

There are motivations for the UE either to prioritize SBFD ROs or to prioritize non-SBFD ROs. For instance, to minimize the CLI in SBFD resources, the UE should prioritize selection of non-SBFD ROs. To increase RACH capacity, the UE should prioritize selection of SBFD ROs.

- Option 2: An SBFD aware UE determine whether to select SBFD ROs based on a threshold, e.g. RSRP threshold, CLI threshold;

This is also for minimizing the CLI in SBFD resources. For instance, if the RSRP is too low compared to a dedicated SBFD threshold, then the UE should not select SBFD ROs to avoid significant interference to SBFD operation on neighbor cells.

- Option 3: The UE randomly selects an RO amongst the consecutive PRACH occasions regardless whether the ROs are SBFD ROs or non-SBFD ROs. Whether the definition of ‘consecutive PRACH occasions’ needs to be revised is up to RAN1.

Actually, the UE may not need to differentiate SBFD ROs or non-SBFD ROs. The UE still randomly selects RO amongst the consecutive ROs. For example, the UE can benefit if the selected RO comes earlier regardless whether it is SBFD RO or non-SBFD RO. Random selection helps the NW to achieve the balance between the said motivations for the UE either to prioritize SBFD ROs or non-SBFD ROs in option 1.

In sum, we propose RAN2 to discuss how an SBFD aware UE to perform RO selection in SBFD.

Proposal 7: RAN2 to discuss how an SBFD aware UE perform RO selection amongst SBFD ROs and non-SBFD ROs:

- Option 1: An SBFD aware UE prioritizes selection of SBFD ROs or non-SBFD ROs;

- Option 2: An SBFD aware UE determine whether to select SBFD ROs based on a threshold, e.g. RSRP threshold, CLI threshold;

- Option 3: The UE randomly selects an RO amongst the consecutive PRACH occasions regardless whether the ROs are SBFD ROs or non-SBFD ROs. Whether the definition of ‘consecutive PRACH occasions’ needs to be revised is up to RAN1.

If RAN2 agrees to go with option 1, another issue is whether/how to allow the UE to switch between SBFD ROs and non-SBFD ROs. For example, if a UE continues to fail in random access procedure in SBFD ROs, the situation may be that the CLI is too high in SBFD resources. The UE may benefit from switching to perform random access in non-SBFD ROs.

Proposal 8: RAN2 to discuss whether/how to allow an SBFD aware UE to switch RO selection between SBFD ROs and non-SBFD ROs if option 1 in P7 is agreed.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: All the current RACH triggering events are supported in SBFD.

Proposal 2: SBFD is not considered as a new feature for PRACH preamble partitioning, i.e. the NW does not configure SBFD as a feature as ‘true’ for a set of random access resources.

Proposal 3: RAN2 to discuss and determine whether to support Msg1/3 repetition in SBFD, or leave it to RAN1.

Proposal 4: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement.

Proposal 5: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

Proposal 6: RAN2 to discuss whether SBFD operation has an impact on SSB/CSI-RS selection for random access procedure.

Proposal 7: RAN2 to discuss how an SBFD aware UE perform RO selection amongst SBFD ROs and non-SBFD ROs:

- Option 1: An SBFD aware UE prioritizes selection of SBFD ROs or non-SBFD ROs;

- Option 2: An SBFD aware UE determine whether to select SBFD ROs based on a threshold, e.g. RSRP threshold, CLI threshold;

- Option 3: The UE randomly selects an RO amongst the consecutive PRACH occasions regardless whether the ROs are SBFD ROs or non-SBFD ROs. Whether the definition of ‘consecutive PRACH occasions’ needs to be revised is up to RAN1.

Proposal 8: RAN2 to discuss whether/how to allow an SBFD aware UE to switch RO selection between SBFD ROs and non-SBFD ROs if option 1 in P7 is agreed.

References

- TS 38.300, V18.2.0

- TS 38.321, V18.2.0

- R1#116 meeting minutes