3GPP TSG-RAN WG2 Meeting #127	R2-2406725

Maastricht, Netherlands, 19th Aug.– 23rd Aug., 2024

Agenda Item:	8.11.3

Source:	vivo

Title:	Discussion on other issues in SBFD

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss some other issues in SBFD from RAN2 perspective.

Discussion

UE capabilities for SBFD RACH configuration

RAN1 agreed on two options for SBFD RACH configuration [1], i.e. use one single RACH configuration or two separate RACH configurations to indicate ROs within UL sub-band in SBFD symbols. RAN1 also agreed on the following principles:

- Enabling both options at the same time for a UE is not supported;

- UE is not required to support both options;

For RRC_CONNECTED UE, the NW can configure the corresponding SBFD RACH configuration according to the UE Capabilities report. Therefore, a new UE capability for SBFD RACH configuration should be defined.

Proposal 1: Define a new UE capability to indicate which SBFD RACH configuration the UE supports, e.g. supportedSBFD-RACH-Config.

One issue is whether UE can report the capability of supporting both options for SBFD RACH. One alternative is that the UE can only supports one of the two options, and another alternative is that the UE can supports either only one option or both of the options. Hence, we propose RAN2 to discuss and clarify this issue.

Proposal 2: RAN2 to discuss and clarify how UE reports the capability of supporting option(s) for SBFD RACH:

For RRC_CONNECTED UE, it should be noted that ‘Enabling both options at the same time for a UE is not supported’ according to RAN1 agreement is to reduce the complexity of UE implementation. For RRC_IDLE/INACTIVE UE, the same principle still applies. In IDLE/INACTIVE state, NW does not have information on UE capability report about which option is supported. One alternative is that the UE assumes the NW will configure only one of the options. Another alternative is that the NW can configure both options, but which option is used and how to define the UE behavior for this case need to be further discussed..

Proposal 3: For RRC_IDLE/INACTIVE UE, RAN2 to discuss and clarify how to configure SBFD RACH configuration and what is corresponding UE behavior.

- Alt 1: only one of the two SBFD RACH configuration options can be configured.

- Alt 2: either one of the options or both of the SBFD RACH configuration options can be configured.

SBFD RACH configuration signalling

Generally speaking, detailed SBFD configuration signalling can be discussed at stage-3 after more agreements are made in RAN1. However, we can also have some initial discussion about it if the schedule allows.

Take the SBFD RACH configuration for example. According to the updated WID [2], we need to specify SBFD operation to support random access in SBFD symbols for both RRC_CONNECTED UE and RRC_IDLE/INACTIVE UE. In R1#116bis, it was agreed to support CBRA and CFRA in SBFD symbols for SBFD aware UEs in RRC CONNECTED state [3]. Therefore, the following conclusions about detailed SBFD RACH configuration can be made:

- The SBFD RACH configuration can be carried in both ServingCellConfigCommonSIB for RRC_IDLE/INACTIVE UE and ServingCellConfigCommon for RRC_CONNECTED UE;

- The SBFD RACH configuration can be carried in both RACH-ConfigCommon for CBRA and RACH-ConfigDedicated for CFRA.

Proposal 4: The SBFD RACH configuration can be carried in both ServingCellConfigCommonSIB for RRC_IDLE/INACTIVE UE and ServingCellConfigCommon for RRC_CONNECTED UE, and in both RACH-ConfigCommon for CBRA and RACH-ConfigDedicated for CFRA.

As for other UL/DL SBFD configurations, e.g. SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, we can leave them to RAN1.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: Define a new UE capability to indicate which SBFD RACH configuration the UE supports, e.g. supportedSBFD-RACH-Config.

Proposal 2: RAN2 to discuss and clarify how UE reports the capability of supporting option(s) for SBFD RACH:

Proposal 3: For RRC_IDLE/INACTIVE UE, RAN2 to discuss and clarify how to configure SBFD RACH configuration and what is corresponding UE behavior.

- Alt 1: only one of the two SBFD RACH configuration options can be configured.

- Alt 2: either one of the options or both of the SBFD RACH configuration options can be configured.

Proposal 4: The SBFD RACH configuration can be carried in both ServingCellConfigCommonSIB for RRC_IDLE/INACTIVE UE and ServingCellConfigCommon for RRC_CONNECTED UE, and in both RACH-ConfigCommon for CBRA and RACH-ConfigDedicated for CFRA.

References

- R1#117 meeting minutes

- RP-241614 Revised WID - Evolution of NR duplex operation Sub-band full duplex (SBFD)

- R1#116bis meeting minutes