3GPP TSG-RAN WG2 #127                                             	    R2-2406733

Maastricht, Netherlands, August 19th –23th, 2024

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	Measurement enhancements for LTM

Document for:	Discussion

# 1	Introduction

RAN2#126 has agreed on the following [1]:

1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This paper discusses aspects related to measurement enhancements for LTM and consequent proposals.

# 2	Issues for LTM event evaluation

## 2.1	Selection of serving beam

This is applicable to Event LTM2, LTM3 and LTM5.

The NW should become aware of the degradation of the “indicated” serving beam so that the NW can take proper action, e.g., indicate another serving beam (if available), or trigger LTM execution of the UE towards a candidate cell. This implies  that a measurement report should be triggered upon degradation of the “indicated” serving beam and thus the  beam used for evaluation of Event LTM1, LTM2 and LTM5 should be the “indicated” beam. This beam should belong to the primary cell of the associated CG for which LTM is configured. Therefore, this can be taken as the baseline. Whether any other additional beams can be used can be discussed in the future.

Proposal 1: The serving beam used for evaluation of Event LTM1, LTM2 and LTM5 is the “indicated” beam on the primary cell of the associated CG for which LTM is configured. FFS if additional serving cell beams (configured/activated) can be used for the evaluation of the Event LTM1, LTM2 and LTM5.

## 2.2	Selection of candidate beam

This is applicable to Event LTM3, LTM4 and LTM5.

RAN2#126 agreed that event-triggered L1 measurements are used to select a candidate beam to trigger early synchronization. In downlink, this implies that event-triggered L1 measurements are used to select a candidate beam to trigger activation of the corresponding TCI state. During event evaluation and prior to event-triggering, the candidate beam used for event evaluation is not necessarily an activated beam. In fact, during event evaluation and prior to event-triggering, a candidate cell may have no such activated beams. Therefore, a candidate beam used for event evaluation simply refers to a configured beam whose associated TCI state is not necessarily activated.

Observation 1: During event evaluation for event-triggered L1 measurements, a candidate cell may have zero activated TCI states.

Proposal 2: A candidate beam used for event evaluation refers to a “configured” beam of the candidate cell that may or may not have a corresponding activated TCI state.

All of Event LTM3, LTM4 and LTM5 refer to a candidate beam becoming a “strong” beam. The network needs to know the index of a candidate beam that triggers an event. Howver, before event-triggering occurs, the network does not need to know which candidate beam will initiate the event. Thus, the event definition must not refer to specific candidate beam indices. This also offers the advantage of reducing the overhead associated with event configuration, especially if the number of configured candidate beams is large.

Observation 2: Prior to event triggering, the network is indifferent to which candidate beam will initiate the event. Many events will need to be defined if event definitions refer to specific beam indices.

Based on the definitions of Event LTM3, LTM4 and LTM5, if a candidate beam fulfils the event condition, then so does the strongest beam of that candidate cell. Therefore, the strongest beam of a candidate cell can always be used for the evaluation of events corresponding to that candidate cell.

Proposal 3: The strongest configured beam of a candidate cell is used for the evaluation of events associated with that cell.

RAN2#126 agreed that event-triggered L1 measurements can be used to select a target cell for LTM switch. It is more robust for a UE to switch to a target cell where it detects multiple strong beams, as opposed to one where it detects only a single strong beam. Consequently, events should be evaluated across a configurable number of candidate beams, namely the strongest ones, so that measurement reports are triggered based on events associated with candidate cells where the UE detects multiple good candidate beams. The consolidation of measurements of the multiple candidate beams is subject to further study. Given that cell-level results are also derived from measurements of multiple beams, there is no need to define separate cell-level results specifically for event evaluation.

Proposal 4: The UE evaluates events across a configurable number of the strongest candidate beams. FFS how the UE consolidates the measurements of multiple beams. No separate cell-level results need to be specifically defined for event evaluation.

When the entering condition for an event is met, the UE sends a measurement report. However, until the leaving condition is subsequently met, the strongest candidate beam (or set of strongest candidate beams) may change. During this time interval, no new measurement report should be triggered based on the same event. Otherwise, the overhead of event-triggered reporting will increase, compromising the benefits of such reporting.

Proposal 5: If the strongest candidate beam (or set of strongest candidate beams) changes before the leaving condition for an event is met, the UE does not need to send another measurement report based on the same event.

## 2.3	Using cell specific offset for event evaluation

RAN2#126 has agreed that beam-specific offset for LTM event evaluation is supported. A beam specific offset can always comprise a cell-specific offset. Therefore, a separate cell-specific offset is not needed.

Observation 3: The beam specific offsets for LTM event evaluation can implicitly comprise a cell specific offset.

Proposal 6: Separate cell-specific offset for LTM event evaluation is not supported.

## 2.4	Alignment of RS type

RAN2#126 made a working assumption that the “same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.”

As discussed above, the serving beam used for event evaluation is the indicated beam. Since the NW may dynamically indicate beams of different RS types (SSB vs. CSI-RS), this creates an issue on how to align the RS type for LTM event evaluation. This can be addressed as follows:

- For CSI-RS-based events, if the indicated serving beam is SSB, the UE uses the CSI-RS of the serving cell associated with the indicated SSB. Definition of association is up to RAN1.

  - How to handle case where multiple CSI-RSs are associated with the indicated SSB is also up to RAN1.

- For SSB-based events, if the indicated serving beam is CSI-RS, the UE uses the SSB of the serving cell associated with the indicated CSI-RS. Definition of association is up to RAN1.

Proposal 7: To align the RS type for event LTM3 and event LTM5:

- For CSI-RS-based events, if the indicated serving beam is SSB, the UE uses CSI-RS of the serving cell associated with the indicated SSB.

- For SSB-based events, if the indicated serving beam is CSI-RS, the UE uses SSB of the serving cell associated with the indicated CSI-RS.

- How to define association b/w SSB and CSI-RS (including 1-to-1 and 1-to-many association) is up to RAN1.

# 3	New triggers for LTM measurement report

## 3.1	Event LTM1

Event LTM1 is not relevant for any of the LTM use cases and therefore does not not be supported.

Proposal 8: Event LTM1 is not supported.

## 3.2	Report on leave

If the UE does not send a measurement report after the leaving condition is met, measurement-related information in the network becomes outdated. Therefore, similar to L3 events, report-on-leave should be supported.

Observation 4: Without reporting measurements on leave, measurement results in the network may become outdated.

Proposal 9: UE may initiate a measurement report when the leaving condition is met.

If the network receives a measurement report both at the entering condition and the leaving condition, additional reporting in the meantime does not provide significant additional value. Therefore, event-triggered periodic reporting can be deprioritized.

Proposal 10: Event-triggered periodic reporting is de-prioritized.

# 4	Signaling for LTM measurement report

The flexibility and reliability of carrying measurement-related information within a MAC-CE offer significant benefits as it can carry more information and can be more easily extended in the future. Therefore, measurement reports via a MAC-CE should be supported.

Proposal 11: A measurement report can be conveyed via a MAC CE.

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals are put forward:

Selection of serving beam

Proposal 1: The serving beam used for evaluation of Event LTM1, LTM2 and LTM5 is the “indicated” beam on the primary cell of the associated serving cell group for which LTM is configured. FFS if additional serving cell beams (configured/activated) can be used for the evaluation of the Event LTM1, LTM2 and LTM5.

Selection of candidate beam

Observation 1: During event evaluation for event-triggered L1 measurements, a candidate cell may have zero activated TCI states.

Observation 2: Prior to event triggering, the network is indifferent to which candidate beam will initiate the event. Many events will need to be defined if event definitions refer to specific beam indices.

Proposal 2: A candidate beam used for event evaluation refers to a “configured” beam of the candidate cell that may or may not have a corresponding activated TCI state.

Proposal 3: The strongest configured beam of a candidate cell is used for the evaluation of events associated with that cell.

Proposal 4: The UE evaluates events across a configurable number of the strongest candidate beams. FFS how the UE consolidates the measurements of multiple beams. No separate cell-level results need to be specifically defined for event evaluation.

Proposal 5: If the strongest candidate beam (or set of strongest candidate beams) changes before the leaving condition for an event is met, the UE refrains from sending another measurement report based on the same event.

Using cell specific offset for event evaluation

Observation 3: The beam specific offsets for LTM event evaluation can implicitly comprise a cell specific offset.

Proposal 6: Separate cell-specific offset for LTM event evaluation is not supported.

Alignment of RS type

Proposal 7: To align the RS type for event LTM3 and event LTM5:

- For CSI-RS-based events, if the indicated serving beam is SSB, the UE uses CSI-RS of the serving cell associated with the indicated SSB.

- For SSB-based events, if the indicated serving beam is CSI-RS, the UE uses SSB of the serving cell associated with the indicated CSI-RS.

- How to define association b/w SSB and CSI-RS (including 1-to-1 and 1-to-many association) is up to RAN1.

Event LTM1

Proposal 8: Event LTM1 is not supported.

Report on leave

Observation 4: Without reporting measurements on leave, measurement results in the network may become outdated.

Proposal 9: UE may initiate a measurement report when the leaving condition is met.

Proposal 10: Event-triggered periodic reporting is deprioritized.

Signaling for LTM measurement report

Proposal 11: A measurement report can be conveyed within a MAC CE.

# References

- Chairman Notes; TSG RAN WG2 Meeting #126