3GPP TSG-RAN WG2 Meeting #127	R2-2406734

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will firstly discuss how to avoid unnecessary RLC retransmission, and then provide our views on the timely RLC retransmission.

# 2	Discussion on avoiding unnecessary transmission

From RAN2#126 agreements [2], RAN2 agreed to further analyse how to avoid unnecessary RLC AM retransmission for the outdated RLC SDU (segment) from the Tx initiated and RX initiated approaches, as given below:

- For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

The working principles of the Rx initiated approach and Tx initiated approach are given in Section 2.1 and 2.2 respectively, while section 2.3 provides a comparison of these two directions and proposes a way forward.

## 2.1	Rx initiated RLC AM retransmission enhancement

According to the agreement of RAN2 #126 [2], the Rx initiated approach should be devised by considering the following factors:

- For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

In the Rx initiated approach, an outdated RLC SDU that has not been received successfully should not trigger NACK, otherwise the outdated RLC SDU will be retransmitted as RAN2 has agreed “Tx side just processes the status report as in legacy”. Specifically, Rx RLC entity does not send NACK to the Tx RLC entity, which can be regarded as a fake ACK in the Tx RLC entity when a status report is received. An outdated RLC SDU can be defined based on a Counter/Timer to be introduced per RLC SDU (segment) in the Rx RLC entity. An RLC SDU is considered as outdated when the maximum number of retransmissions is performed, or when the maximum waiting time transmission has been reached. For the outdated RLC SDU (segment), the Rx RLC entity stops waiting for the RLC retransmission and updates the reassembly window. Meanwhile, the Tx RLC entity will consider the RLC SDU (segment) has been successfully received as there is no NACK reported, so the Tx RLC entity updates its transmitting window accordingly.

Figure 1 gives an example for the Rx initiated RLC AM retransmission enhancement solution. In Figure 1, the RLC SDUs labelled with SN = 1,2,3 are received incompletely at their initial transmission, and hence their introduced retransmission counters, e.g. feedback_Counts, are set up at the Rx RLC entity, respectively. After receiving the status report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1,2,3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity. After the transmission, the RLC SDUs of SN = 2 and 6 are successfully received, the RLC SDUs of SN = 1 and 3 are still received incompletely, and the RLC SDU of SN = 5 is missing. Hence, the Rx RLC entity increases the associated feedback_Counts of SN = 1 and 3 by one, i.e., feedback_Count = 1. Assuming that the maximum retransmission times introduced in the Rx RLC entity are set to 1, that is, the maximum retransmission times of SN = 1 and 3 have been reached. Then the RLC SDUs with SN = 1 and 3 will neither trigger status report nor be reported as NACK in the status report at the Rx RLC entity. Thus, in the RLC status report, only SN = 5 is contained. Therefore, the Tx RLC entity can consider the RLC SDUs with SN 1 and 3 successfully received at the Rx RLC entity, and update the transmitting window and arrange retransmission of RLC SDU of SN = 5 only.

Figure 1 Rx initiated RLC AM retransmission enhancement

## 2.2	Tx initiated RLC AM retransmission enhancement

According to the agreement of RAN2 #126 [2], the Tx initiated approach should be devised by considering the following factors:

- For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side

The Tx RLC entity has the full knowledge of the remaining time of the data, the outdated data will no longer be retransmitted, and a notification can be triggered and sent to the corresponding Rx RLC entity, which is relatively similar to Rel-18 PDCP enhancements aimed for solving SN gap.

To be more specific, once the RLC SDU becomes outdated, e.g. an indication of SDU discarding is sent from the PDCP layer, the Tx RLC entity can stop retransmitting the outdated data even if the associated NACK is received, and then update the transmitting window. Meanwhile, an RLC notification can be triggered by the Tx RLC entity to notify the Rx RLC entity of the SNs that the associated RLC SDUs will no longer be retransmitted. This will trigger the Rx RLC entity to update the reassembly window accordingly, if necessary.

Figure 2 illustrates an instance of Tx initiated RLC AM enhancement solution. First of all, the transmitting side starts recording the delay budget for each RLC SDUs, e.g. RLC SDU of SN = 1,2,3,4, as long as they are obtained, such as starting discardTimer at the PDCP layer. When the RLC SDUs with SN = 1,2,3 are received incompletely at the Rx RLC entity at their initial transmission, an associated status report is triggered and transmitted. The Tx RLC entity then arranges the retransmission of RLC SDUs of SN = 1,2,3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity. Assuming that RLC SDUs of SN = 1,3,5 are not completely received at this transmission, a status report is sent from the Rx RLC entity in order to ask for retransmission. However, at the reception of RLC status report, the delay budgets of RLC SDUs of SN = 1 and 3 have been used up. As a consequence, the Tx RLC entity determines to stop retransmitting the RLC SDUs of SN = 1 and 3, and an RLC notification is triggered at the Tx RLC entity for indicating the SNs of the RLC SDUs no longer retransmitted, which can then be transmitted together with the valid data, e.g. retransmission of RLC SDU of SN = 5. When such notification is received, the Rx RLC entity can stop triggering and sending NACK for the RLC SDUs of indicated SN, and update the reassembly window accordingly.

Figure 2 Tx initiated RLC AM retransmission enhancement

## 2.3	Analysis of Two RLC AM retransmission enhancement solutions

As can be seen above, the Rx initiated option mainly modifies the Rx RLC entity behaviours, where a counter/timer needs to be introduced per RLC SDU (segment) to limit the maximum retransmission times per the maximum waiting duration. However, the outdated data is likely to still be retransmitted, because the Rx RLC entity does not know the precise remaining time of each RLC SDU (segment). For the Rx initiated option, the counter/timer threshold should be carefully set in order to minimize the resources waste.

Observation 1: The Rx initiated approach is inaccurate as the Rx side does not know the exact expiry time of each RLC SDU (segment).

By contrast, since the Tx side has knowledge of the remaining time of the RLC SDU (segment), the retransmission of the outdated data can be terminated timely to avoid resource waste. Additionally, a notification can be triggered and sent from the Tx RLC entity to the corresponding Rx RLC entity, to prevent the Rx RLC entity from triggering and sending unnecessary RLC status report for an outdated RLC SDU, and trigger the Rx RLC entity updates the reassembly window. However, the flaw of the Tx initiated introduced by the signalling overhead approach is also evident.

Observation 2: The Tx initiated approach will introduce some signalling overheads to transmit a notification for outdated data.

As analysed above, both options can reduce unnecessary RLC AM retransmissions, but each has its own problems, i.e. additional overhead in case of the Tx initiated approach and unnecessary retransmissions for the Rx initiated approach. We believe a compromised solution that combines the Tx initiated approach and Rx initiated approach is more effective. To be more specific, at the Tx RLC entity, the outdated data will not be retransmitted even if the associated NACK is received, but the Tx RLC entity will not send any notification to the Rx RLC entity to limit the signalling overhead. Meanwhile, at the Rx RLC entity, the reassembly window can be updated based on the retransmission Times/Time period threshold, while the transmitting window at the Tx RLC entity can be updated as per legacy based on the received RLC status report.

Moreover, in both options, there may be a slight enhancement in the Tx RLC entity, to prevent the RLF once a certain number of retransmissions is reached.

Proposal 1:	RAN2 to adopt a compromised approach where:

- Tx RLC entity does not retransmit the outdated data even if the associated NACK is received. No notification is sent to the Rx RLC entity about the obsolete RLC SDUs.

- Rx RLC entity updates the reassembly window after determining that the SDU is obsolete by itself, e.g. based on the number of retransmissions or based on a timer.

# 3. Discussion on timely RLC retransmission enhancement

An agreement on analysing solutions to ensure timely RLC retransmission has been reached in RAN2 126 [2] meeting, as given below:

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

The major motivation on timely RLC retransmission is to reduce RLC ARQ retransmission latency, there are three possible options:

- Alt1 Autonomous retransmission is to offer the Tx RLC entity a sort of capability to autonomously retransmit an RLC SDU (segment) even if the corresponding NACK has not been received. To be more specific, to enable autonomous RLC retransmission, an RLC SDU (segment) can be considered for retransmission when the specific condition(s) is met. For example, the corresponding remaining time drops below a threshold before the associated feedback is received, as shown in Figure 3 (a). Alternatively, autonomous RLC retransmission can be triggered when the associated feedback is not received at Tx RLC entity within a duration since its last transmission, as illustrated in Figure 3 (b). Moreover, the autonomous retransmission can be limited to be triggered based on the channel quality, e.g. RSRP, where the RLC SDU (segment) can be retransmitted when the channel quality is deteriorated, to ensure the transmission reliability of the associated RLC SDU.

(a) Retransmitting SDU#1 when its remaining time below a threshold and no associated feedback is received



(b)  Retransmitting SDU#1 when no any associated feedback received within a certain period Figure 3 potential methods for autonomous RLC retransmission

Alt2 Enhanced status report based retransmission is to enable more frequent RLC status report. To achieve this, additional status report triggering condition may be needed, e.g. periodical status report. Alternatively, as the status report can be triggered by the expiry of t-reassembly, a shorter value for t-reassembly can also be considered. Moreover, since the triggered status report may be blocked by the t-StatusProhibit, shorter t-reassembly needs to work together with a shorter t-StatusProhibit.

Alt3 Enhanced polling based retransmission tends to reuse the existing status report based RLC retransmission mechanism, but applies a more proactive mechanism to request a frequent status report to enable the RLC retransmission latency reduction. One potential enhancement is to consider the remaining time of the data as an additional factor for polling triggering. For example, when there is delay-critical RLC SDU, or when the data volume of the delay-critical SDUs exceeds a threshold, the polling can be triggered. Upon the reception of the polling, the Rx RLC entity can trigger status report. Similar as above, the frequent polling needs to work together with a shorter t-StatusProhibit .

In our view, Alt2 and Alt3 are quite the same and they have a common purpose to have a fast status report. However, from our analysis, there is no need to enhance status report or introduce new polling trigger. For the status report based enhancement, since the status report will be triggered when t-reassembly timer is expired, and t-reassembly timer will be started when there is missing RLC SDU (segment) detected, the status report will be triggered with the periodicity equal to the length of t-reassembly once there is missing RLC SDU (segment). Therefore, we observe no extra benefit from introducing the periodical status report. From our perspective, fast status report triggering and the associated rapid RLC retransmission can be implemented by simply setting a short t-reassembly, while an appropriate configuration of t-StatusProhibit can effectively prevent frequent status report transmissions. This is similar to the polling based enhancement, where an appropriate configuration of the polling triggering condition, e.g. based on pollByte/pollPDU, is able to ensure that the polling can be triggered and transmitted timely. Moreover, it is worth noting that only the uplink scenario is considered in timely retransmission. In such case, the network can determine whether, when and how to transmit the status report to the UE regardless of reception of the polling or the configuration of the t-reassembly.

Observation 3: For status report based and polling based solutions, the fast status report triggering can be addressed by network configuring appropriate parameter values, e.g. t-reassembly, t-StatusProhibit, and pollByte/pollPDU.

Observation 4: In the uplink scenario, whether, when and how to trigger and transmit the RLC status report is up to the network implementation.

Based on the observations above, there is no need to discuss status report based or polling based enhancements.

Proposal 2: RAN2 does not specify any enhanced status report or enhanced polling based mechanism for the timely RLC retransmission, as network implementation can achieve the same effect.

As discussed above, the RLC retransmission latency can be reduced by configuring an appropriate value for the parameters relevant to the status report triggering. However, the status report based retransmission is not one hundred percent reliable. In an example, it is likely that the retransmission of the RLC SDU has failed and the status report has not been received at the Tx RLC entity, hence imposing latency on the RLC SDU retransmission. Therefore, we believe that autonomous retransmission can be a complementary solution to achieve the timely RLC retransmission even if network has appropriately set the parameter values related to the status report.

Proposal 3:	RAN2 works on the autonomous RLC retransmission.

# 4. Conclusion

This contribution makes the following proposals:

Avoiding unnecessary transmission

Observation 1: The Rx initiated approach is inaccurate as the Rx side does not know the exact expiry time of each RLC SDU (segment).

Observation 2: The Tx initiated approach will introduce some signalling overheads to transmit a notification for outdated data.

Proposal 1:	RAN2 to adopt a compromised approach where:

- Tx RLC entity does not retransmit the outdated data even if the associated NACK is received. No notification is sent to the Rx RLC entity about the obsolete SDUs.

- Rx RLC entity updates the reassembly window after determining that the SDU is obsolete by itself, e.g. based on the number of retransmissions or based on a timer.

Timely RLC retransmission enhancement

Observation 3: For status report based and polling based solutions, the fast status report triggering can be addressed by network configuring appropriate parameter values, e.g. t-reassembly, t-StatusProhibit, or pollByte/pollPDU.

Observation 4: In the uplink scenario, whether, when and how to trigger and transmit the RLC status report is up to the network implementation.

Proposal 2: RAN2 does not specify any enhanced status report or enhanced polling based mechanism for the timely RLC retransmission, as network implementation can achieve the same effect.

Proposal 3:	RAN2 works on the autonomous RLC retransmission.

# 5. Reference

- RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

- Chair note of RAN2#126, https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_126/Inbox/Chair_Notes/R2_1256%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM_rm.docx