3GPP TSG-RAN WG2 Meeting #127	R2-2406737

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.2.2

Source:	China Telecom

Title:	Discussion on functionality aspects of Ambient IoT

Document for:	Discussion and Decision

- Introduction

The A-IoT topic was widely discussed in the previous meetings, and the following agreements were made for functionality aspects [1].

RAN2#125bis Agreements

1	RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

2	RRM L3 measurement reporting is not supported by Ambient IoT devices.

3	RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

4	Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

5	RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

RAN2#126 Agreements

1	Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

In this paper, we will discuss some leftover issues on control plane functionality and provide some proposals.

- Discussion

2.1	AS Security

In SA3#116 meeting, the security of A-IoT service communication was discussed and made the following consensus [2].

As per TS 22.369, Ambient power-enabled IoT (AIoT) services aim to support various use cases, including inventory taking, sensor data collection, asset tracking, and actuator control. These services intended to operate with lower power consumption and complexity than the existing IoT technologies such as eMTC, NB-IoT, and RedCap. To fulfil these requirements, AIoT devices require a communication capability.

From a security perspective, security mechanisms to protect the information transmitted during AIoT service communication need to be supported. Failure to provide such security mechanisms will lead to various attacks such as eavesdropping, manipulation and/or unauthorized transmission of the information during AIoT service communication.

The above agreements imply that the security mechanisms needs to be supported in A-IoT. Without security mechanism, potential security attacks may happen during A-IoT device communication procedure.

Observation: SA3 agreed to support security mechanism to protect the information transmitted during A-IoT service communication.

Based on the above observation, we understand at least the AS security is needed to protect A-IoT devices from various attacks such as eavesdropping, manipulation and/or unauthorized transmission of the information. Without AS security, the device ID transmission between the device and reader during the access procedure may have some security issues. For example, an attack can identify and attack an A-IoT device based on the identifiers associated with the A-IoT devices. Therefore, it is proposed to support the AS security to protect the information transmitted during the A-IoT service communication.

Proposal 1: AS security is supported to protect information transmitted during the A-IoT service communication.

2.2	Resource Allocation/Scheduling

In the RAN1 meeting, the following agreements were reached regarding resource allocation.

RAN1#116bis Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS. The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined RAN1#117 Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

It can be observed that multiple access in time-domain, frequency-domain and code-domain are supported for Ambient-IoT. Therefore, the CP functionality of resource allocation needs to be studied for A-IoT. As for frequency, the network may need to configure the frequency resource, e.g. radio block number, frequency point, frequency shift, to A-IoT to indicate the available frequency resource.

Considering that SFO causes huge timing errors, to reduce A-IoT device complexity, it would be better for the network to control the A-IoT accessing time. Similar to RFID, the A-IoT may only need a counter to record the slot number in the random access. Therefore, the resource allocation can mainly focus on frequency resource allocation.

Proposal 2: Frequency domain resource allocation needs to be considered in initial access procedure.

In fact, the A-IoT energy is limited and storing resource configuration consumes additional energy. Therefore, it would be better to make configuration resource very simple, e.g. only focus on frequency resource allocation. In addition, the resource can be configured as one-shot valid, which means that the network configures the resource at each time the A-IoT needed, e.g. random access procedure. In this way, the power consumption and signalling overhead can be reduced.

Proposal 3: The resource configuration for A-IoT is very simple and only valid in the current service procedure.

2.3	Device Temporary ID Assignment

In RFID, a 16-bit random or pseudo-random number (RN16) is used to identify a Tag uniquely during the whole data transmission. In legacy NR, an AS temporary ID (e.g. C-RNTI) is configured to UE for data transmission. Similarly, as for A-IoT devices, there exists two options.

- Opt 1: No need for temporary ID

- Opt 2: Temporary ID assigned by reader

From the perspective of standard impact, Opt 1 is much simpler than Opt 2. With Opt 1, a 16-bit random number generated during random access can be used to identify the A-IoT device uniquely in the whole transmission procedure. As for Opt 2, the reader needs to allocate a temporary ID for an A-IoT device based on the number of A-IoT devices. For instance, if there are 64 A-IoT devices exist, the reader can allocate an 8-bit temporary ID for a A-IoT device. In this way, the signalling overhead can be reduced when the number of A-IoT devices within reader is small. Both Opt 1 and Opt 2 has its own pros and cons. Therefore, whether to introduce temporary ID to identify A-IoT devices needs further discussion.

Proposal 4: Whether to introduce temporary ID to identify A-IoT devices needs further discussion.

2.3	Protocol Stack for Control Plane

In RAN2#125bis, one leftover issue was which layer is use to implement control plane functionality, i.e. a new AS protocol on top of A-IoT MAC layer or A-IoT MAC.

Based on the current conclusion, the control plane functionality at least includes paging, resource allocation, and interaction with upper layer. Enhancing the A-IoT MAC layer to achieve these functionalities may introduce huge specification complexity. For instance, there may exist many differences between the A-IoT MAC layer and the reader MAC layer, making the interaction extremely difficult. Compared to it, introducing a new AS protocol on top of A-IoT MAC to handle the control plane functionalities would be much simpler, and the protocol stack function is clearer. Therefore, we support to introduce a new AS protocol on top of A-IoT MAC to handle the functionalities of A-IoT paging, resource allocation and interaction with A-IoT upper layer. To reduce the complexity, the new AS layer should be as simple as possible.

Proposal 5: To introduce a new AS protocol on top of A-IoT MAC to handle the control-plane functionalities.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation: SA3 agreed to support security mechanism to protect the information transmitted during A-IoT service communication.

Proposal 1: AS security is supported to protect information transmitted during the A-IoT service communication.

Proposal 2: Frequency domain resource allocation needs to be considered in initial access procedure.

Proposal 3: The resource configuration for A-IoT is very simple and only valid in the current service procedure.

Proposal 4: Whether to introduce temporary ID to identify A-IoT devices needs further discussion.

Proposal 5: To introduce a new AS protocol on top of A-IoT MAC to handle the control-plane functionalities.

- References

- R2-240xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- S3-242535, New Key Issue on the protection of information during AIoT service