3GPP TSG-RAN WG2 Meeting #127                                                                     R2-2406738

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.2.3

Source:	China Telecom

Title:	Discussion on Ambient IoT Paging

Document for:	Discussion

- Introduction

In the RAN#102 meeting, a new SID on solutions for Ambient IoT (Internet of Things) in NR was approved [1].  As a suitable solution for deployment in a 3GPP system, Ambient IoT relies on ultra-low complexity devices with ultra-low power consumption for very-low-end IoT applications. The RAN2-led objectives are listed as follows:

- Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In RAN2#125bis, the following open issue was still need to be discussed.

Agreements

- Unless explicitly stated all agreements apply to all device types and for both topologies.

- From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

- RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

- Baseline procedure:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS

Step C: The device may perform the data communication with the reader as needed,: Details FFS

- We will study the support of both “inventory” and “command” in the same procedure.

- FFS if Initial Trigger Message can also include “command”.

RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

In the RAN2#126 meeting [2], the following agreements on the control plane and paging were made:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Based on the above progress and open issues, the intention of this paper is to further discuss the paging functionality for A-IoT.

- Discussion

As outlined in TS 38.300 [3], the existing paging mechanism supports CN-initiated paging and RAN-initiated paging. At present, the function of the control plane for A-IOT mainly includes the inventory and command signalling transmission.

For inventory use cases, the goal is to determine the identity of all or a subset of AIoT devices within range of the reader. The application inventory procedure is initiated by the third-party A-IoT server (e.g. AF) to discover one or more A-IoT devices in a specific area. The Reader/RAN node can receive an inventory request from CN/AF. The inventory request optionally contains information such as a list of reader IDs or area information to limit the inventory to a specific group of readers. With this inventory request received, the Reader/RAN node initiates a paging message via the air interface to trigger A-IoT devices. Then the A-IoT devices served in the coverage of the reader/RAN node respond to the inventory request from A-RAN and send their device IDs in response message.

More generally, the reader does not know the number of nearby AIoT devices and their types until the reader completes an inventory procedure. For the command use case, the AF can use the inventory data to indicate the nearby reader to send an A-IoT command to the specific A-IoT device. The detailed information of command may be NAS PDU rather than a piece of explicit information that could be decoded by the reader/RAN node.

In addition, the AF may also provide the inventory strategy information (e.g. inventory frequency, inventory period) to both CN and Reader/RAN nodes. With this assistance information, the reader/RAN node can perform periodic inventory and send the paging trigger messages without explicit requests from the AF.

Proposal 1: The Reader/RAN node can initiate the periodical paging trigger messages based on the information received from CN/AF.

Per TS 38.300 [3], the existing NR paging mechanism allows the network to inform one or a group of UEs in RRC_IDLE and RRC_INACTIVE state by paging messages, and to notify UEs of system information change and ETWS/CMAS indications through Short Messages. The legacy paging message is encoded in the RRC layer. According to the agreement in the RAN2#125bis meeting, it was agreed that the RRC layer is not supported between the reader and the device. Regarding the layer to encode A-IoT paging message, there are two potential solutions:

- Solution 1: MAC layer

- Solution 2: a new layer on top of MAC layer if finally supported in the control plane

Theoretically, both solutions are workable. Unlike the legacy system, A-IoT is an asynchronous system which cannot maintain synchronization between UE/device and gNB as legacy UE. Compared to the potential AS layer, the advantage of MAC layer is that it could decide when to send the paging immediately. Therefore, it is proposed to encode trigger/paging messages in MAC layer regardless of whether a new layer on top of MAC layer is supported in RAN2.

Proposal 2: It is proposed to encode trigger/paging messages in MAC layer regardless of whether a new layer on top of the MAC layer is supported in RAN2.

Regarding the content of the paging/trigger message, the legacy LTE and NR RRC paging include the following information:

- pagingRecordList(Optional): one or multiple UE ID

- paging cause(Optional): only support one codepoint, namely “voice”

- systemInfoModification(Optional): only applicable for LTE.

- Multiple carrier load balance indicator (Optional): namely redistributionIndication IE specified in LTE.

In the last meeting, RAN2 has agreed that the AIOT paging message contains the device ID(s) of the devices. In order to page a single or group of devices, the pagingRecordList-like information should be kept in an A-IoT paging/trigger message. Thus, UE ID (to identify the AIoT device) can be an individual UE ID or a group ID. If the AIoT paging message does not contain any device IDs, the message is addressed to all AIoT devices within the coverage of the reader that can receive the AIoT message.

In previous meetings, some companies have proposed to introduce filter criteria to limit the AIoT devices that receive paging messages. However, SA2 concluded that the case “a message containing multiple IDs of A-IoT devices” is not supported in SA2#164 [5]. For this issue, we prefer to not introduce an additional device ID mask to find a group of devices as device ID may comprise multiple bits like reader ID or cell ID. If the CN decides to send messages to a group of devices, it can only send a subset of device ID bits to NG-RAN. In case of the group UEs need to be paging, the A-IoT devices with the same subset of device ID bits can be contained in this inventory message. Therefore, it is need to specify a variable length of the A-IoT device ID in the inventory message.

Proposal 3: We prefer to not introduce an additional device ID mask to find a group of devices as device ID may comprise multiple bits like reader ID or cell ID.

Proposal 4: It is proposed to specify a variable length of the A-IoT device ID in the inventory message.

At the last meeting, RAN2 agreed to include “inventory only” cases and “inventory and command” as baseline use cases, while “Command only” still require further discussion. A typical example of a “Command only” scenario is when an AIoT application has knowledge of the AIoT devices in a given area, but lacks current inventory information. In this case, the application can initiate a command procedure that inventory all devices that meet specific conditions. If the Initial Trigger Message from the reader supports “Command only” cases, readers can send a message in the R2D link to indicate the AIoT devices that meet the criteria to reply with the required data.

Proposal 5: A-IoT paging message can include “command”.

Regarding the paging cause, the paging cause “Inventory”, “command” or “Inventory + command” is not necessary.

However, since the Command refers to an instruction (e.g., Read, Write, Disable) sent by an AF to an A-IoT device, is end-to-end protected between AF and A-IoT Device, the AMF may include one or more Command information in the paging message.

Therefore, the Command Information as String type can be also included in the paging message. In conclusion, UE paging ID (an ID or a group ID) and NAS data(e.g., Command) can be included in the A-IoT paging message.

Proposal 6: UE paging ID (an ID or a group ID) and NAS data (e.g., Command) can be included in the A-IoT paging message.

- Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1: The Reader/RAN node can initiate the periodical paging trigger messages based on the information received from CN/AF.

Proposal 2: It is proposed to encode trigger/paging messages in MAC layer regardless of whether a new layer on top of the MAC layer is supported in RAN2.

Proposal 3: We prefer to not introduce an additional device ID mask to find a group of devices as device ID may comprise multiple bits like reader ID or cell ID.

Proposal 4: It is proposed to specify a variable length of the A-IoT device ID in the inventory message.

Proposal 5: A-IoT paging message can include “command”.

Proposal 6: UE paging ID (an ID or a group ID) and NAS data (e.g., Command) can be included in the A-IoT paging message.

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- R2-24xxxxx Draft Report of 3GPP TSG RAN WG2 meeting #125 bis

- TS 38.300 V18.0.0, NR; NR and NG-RAN Overall Description; Stage 2.

- 3GPP TS 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things" V0.3.0.

- S2-2407594, Reply LS on RAN2 agreements and assumptions for Ambient IoT (S2-2407447/R2-2406150)