3GPP TSG-RAN WG2 Meeting #127	R2-2406739

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.4.3

Source:	China Telecom

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE mode

Document for:	Discussion

- Introduction

In the RAN #102 meeting, a new work item on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. The design target aims to be delay- and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. One of the objectives related to RRM measurement relaxation in idle/inactive mode is listed as follows [1]:

- For IDLE/INACTIVE modes - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

According to the above objective in WID, the following agreements in RAN2#126 meeting were made:

##### Procedure and configuration of LP-WUS in RRC_IDLE INACTIVE

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RRM measurement relaxation and offloading in RRC_IDLE INACTIVE

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR  and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Based on the above progress in RAN2, the measurement of offloading/relaxation requirements needs to be checked by other WGs. RAN2 still needs to continue the discussion on the offloading criterion for the serving cell and the neighbor cell measurement relaxation criteria.

In view of the fact that, this paper mainly discusses the RRM measurement relaxation and offloading of RRC_IDLE/INACTIVE UE MR for serving and neighbor cells.

- Discussion

2.1 Offloading of Serving Cell Measurements to LR

According to the above agreements of the last meeting, the requirements for the relaxation of the RRM measurement are pending for other WG discussions, which means RAN4 progress should be taken into account. In the RAN4#111 meeting, the following cases and corresponding device states were listed and discussed.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |
| #5 Exit case | On | On | Off |

From the RAN4 point of view, case #1 will be treated as a high priority. That means full offloading of serving cell measurement from MR to LR is supported. RAN4 will define the measurement offloading requirements for serving cells and the relaxation measurement requirements for neighboring cell measurement.

In general, the serving cell relaxed measurement can be achieved in the following options:

- Option 1: MR performs serving cell measurement with relaxed periodicity, while LR performs partial serving cell measurement

- Option 2: Not perform measurement in MR, and LR performs all the serving cell measurement

In the RAN1#116 meeting, it was concluded that serving cell measurements can be offloaded to LP-WUR based on LP-SS. Hence, from the perspective of power consumption, partial or all UE serving cell RRM measurements are offloaded from MR to LP-WUR after fulfilling the RRM relaxation criteria.

Observation 1: Partial or all UE serving cell RRM measurements can be offloaded from MR to LP-WUR, when RRM relaxation condition is met.

As outlined in TR 38.869, serving cell measurements based on MR significantly impact UE energy consumption. This conclusion implies that the MR may cause higher power consumption than LP-WUR to perform RRM measurement in ultra-deep sleep. Thus, LP-WUR performs all RRM measurements of serving cells via LP-SS (fully offloading case) can be considered as a lower power consumption solution.

Furthermore, from the perspective of standard impact, it is need to introduce an additional RRM measurement relaxation mechanism if MR-based measurement is applied. In contrast, LP-WUR as an independent radio module has less impact on the existing cell reselection and measurement mechanism. Therefore, it is proposed that LP-WUR could perform the serving cell’s RRM measurement without waking up MR in case the RRM relaxation/offloading criterion is fulfilled.

Proposal 1: LP-WUR could perform the serving cell’s RRM measurement without waking up MR in case the RRM relaxation/offloading criterion is fulfilled.

2.2 The Offloading Criterion for Serving Cell

As one of the objectives of WID, the trigger conditions for serving cell measurement offloading from MR to LR need to be studied.  From the perspective of RAN2, the thresholds for RRM measurement offloading should satisfy similar criteria (e.g. Rel-16 PowSav or Rel-17 RedCap) for relaxing neighbor cell RRM measurement or even with more stringent requirements or thresholds. For instance, the UE supporting LP-WUS needs to receive a strong LP-WUS signal while satisfying the existing criteria for RRM measurement relaxation for neighboring cells. In such cases, the UE could use the LP-WUS and offload RRM serving cell measurement from MR to LP-WUR.

Proposal 2: The current triggering criteria for RRM measurement relaxation in Rel-16 PowSav and Rel-17 RedCap can be taken as a baseline for RRM measurement offloading.

Obviously, if partial UE serving cell RRM measurements are offloaded from MR to LP-WUR, the evaluation of exit conditions for offloading could be performed based on measurement results from both LP-WUR and MR. Thus, the joint evaluation of the threshold based on measurements from both LP-WUR and MR needs to be introduced in Rel-19.

Proposal 3: To introduce joint evaluation of threshold based on measurements from both LP-WUR and MR.

2.3 RRM Measurement Relaxation for Neighbour Cells

In the RAN2#126 meeting, it was agreed that the RRM measurement of the neighboring cell can only be performed by MR. In the cases of the above RAN4 discussion, the LP-WUR is usually active. Therefore, when MR performs relaxed measurement for neighbor cells, the behavior of LP-WUR also needs to be discussed. In our opinion, if UE does not perform measurement relaxation by LP-WUR, MR can apply NR legacy behaviors for RRM measurement.

Observation 2: If UE does not perform measurement relaxation by LP-WUR, MR can apply NR legacy behaviors for RRM measurement.

Suppose the entry conditions for LP-WUS monitoring are fulfilled, before performing the serving cell measurement of MR. In that case, it should be determined whether the criteria for performing neighboring cell measurements are met:

- If the signal quality of the serving cell is good enough and neighboring cell measurement does not need to be performed, there is no need to wake up MR.

- If the neighboring cell measurement is met, then it should be determined whether neighboring cell measurement relaxation can be performed.

Proposal 4: Before waking MR to perform the neighboring cell measurement relaxation, it should be determined whether the criteria for performing neighboring cell measurement are met.

- Conclusions

Based on the discussion in the previous sections we propose the following:

Observation 1: Partial or all UE serving cell RRM measurements can be offloaded from MR to LP-WUR, when RRM relaxation condition is met.

Proposal 1: LP-WUR could perform the serving cell’s RRM measurement without waking up MR in case the RRM relaxation/offloading criterion is fulfilled.

Proposal 2: The current triggering criteria for RRM measurement relaxation in Rel-16 PowSav and Rel-17 RedCap can be taken as a baseline for RRM measurement offloading.

Proposal 3: To introduce joint evaluation of threshold based on measurements from both LP-WUR and MR.

Observation 2: If UE does not perform measurement relaxation by LP-WUR, MR can apply NR legacy behaviors for RRM measurement.

Proposal 4: Before waking MR to perform the neighboring cell measurement relaxation, it should be determined whether the criteria for performing neighboring cell measurement are met.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- R2-2313939 Report of [AT124][505][LPWUS] Update of TR 38.869 for LP-WUS WUR (vivo)