3GPP TSG-RAN WG2 Meeting #127	R2-2406740

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.2

Source:	China Telecom

Title:	Discussion on XR multi-modality

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, the agreement regarding multi-modality was made as follow:

Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

In this contribution, we investigate the possible enhancements for multi-modality awareness of XR and give some considerations.

- Discussion

2.1 Multi-modality awareness

In last meeting, RAN2 asked SA2 to answer that whether or what multi-modal information for UL and DL could be provided for RAN. However, there is no reply until now. Therefore, we will show our considerations based on the previous SA2 agreements.

It has stated in TS23.501 that multi-modal service is a communication service including several interrelated data flows. In order to achieve multi-modal interaction, SA2 has agreed to specify the multi-modal service ID (MMID) to indicate inter-dependence between different data flows in the multi-modal service. Specifically, AF provides MMID to PCF to help it identify the associated flows during admission control. Per TR 23.700-60, the PCF authorizes all QoS requirements of group flows as a whole. Only when all flows’ requirements are authorized, the PCF generates the corresponding configuration. Otherwise, even if only one QoS requirement is rejected, the PCF will reject all the QoS requirements of group flows.

However, the information is only provided to CN, and RAN cannot obtain the information. As a result, inconsistent admission control between CN and RAN may exist. If PCF accepts all the associated flows with same MMID, but RAN rejects one or multiple flows, the admission control at CN will become ineffective.

Observation: Without RAN awareness of MMID, there may exist inconsistencies between CN and RAN.

Therefore, multi-modal service ID should also be visible to RAN to indicate the inter-dependence for multi-modal services. With MMID, a joint admission control between CN and RAN is performed. In this way, during RAN admission control, it will consider the behaviour of PCF to avoid inconsistencies.

Proposal 1: To avoid inconsistencies between CN and RAN, multi-modal service ID should be visible to RAN.

In addition, the synchronization between different media components is critical for satisfying user experience for multi-modal flows. Per TR22.847, typical synchronization thresholds for immersive multi-modality VR applications are shown in Table I [1].

Table I. Synchronization thresholds for immersive multi-modality VR applications

|  | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

It can be observed that the delay between two or more media components should be less than the certain latency KPI.  Theoretically, the presences and realism of users may be decreased with the increase of asynchrony between different modalities. Therefore, from the perspective of RAN, it is essential to be aware of the synchronization threshold between multiple media components to achieve the requirements of immersive multiple-modal XR services.

Proposal 2: Synchronization threshold between multi-modal services should be visible to RAN.

2.2 Assistance Information from UE

In previous SA2 meeting, it was agreed that flow direction, periodicity and N6 jitter information in TSCAI are applicable to XR services. Therefore, RAN needs to be aware of the multi-modal services in both UL and DL direction. The core network may provide the multi-modality information to RAN like the mechanism in Rel-18 XR. Given that the uplink assistance information usually not included in the information from core network, the Rel-19 XR UE can also provide UL assistance information to the network. Specifically, the UL assistance information may include inter-dependence and the synchronization threshold of multi-modal flows to the gNB.

Proposal 3: In Rel-19, UE can provide UL assistance information including the inter-dependence and the synchronization threshold of multi-modal flows to the network.

2.3 Power Saving

To obtain extra power saving gain for XR, C-DRX enhancements may be considered. In legacy, the network can only configure one DRX cycle for UE in one cell. In Rel-18, the XR study focuses on single flow, so only one C-DRX configuration would be sufficient. However, in Rel-19, multiple streams are considered in XR study, the legacy one C-DRX configuration cannot match the traffic characterize of multi-modal services very well. For instance, in order to support multiple traffic patterns, a single C-DRX configuration with long periodicity and long ON duration would be applied, which may be not suitable for data flow with short traffic periodicity and small data size (e.g., audio data flow).

Therefore, it would be beneficial to support multiple active C-DRX configurations corresponding to different XR flows. With the multiple C-DRX configurations, each C-DRX configuration can be tailored to satisfy the QoS requirements of individual flow. In this way, the power consumption can be reduced and the QoS requirements of multi-modal flows may be satisfied better.

Proposal 4: Multiple active DRX configurations are supported to reduce the power consumption of multi-modal flows.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation: Without RAN awareness of MMID, there may exist inconsistencies between CN and RAN.

Proposal 1: To avoid inconsistencies between CN and RAN, multi-modal service ID should be visible to RAN.

Proposal 2: Synchronization threshold between multi-modal services should be visible to RAN.

Proposal 3: In Rel-19, UE can provide UL assistance information including the inter-dependence and the synchronization threshold of multi-modal flows to the network.

Proposal 4: Multiple active DRX configurations are supported to reduce the power consumption of multi-modal flows.

- References

- 3GPP TR 22.847, Study on supporting tactile and multi-modality, V18.2.0, 2022-03.

- R2-240xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan