3GPP TSG-RAN WG2 Meeting #127	R2-2406741

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.4

Source:	China Telecom

Title:	Discussion on XR scheduling enhancement

Document for:	Discussion and Decision

- Introduction

In the RAN2#126 meeting, the scheduling enhancements for XR were widely discussed, and some agreements were made regarding LCP prioritization and LCH restriction. In this contribution, we provide our considerations on delay-aware scheduling enhancements for XR.

2 Discussion

2.1 LCP Prioritization

In the last meeting, the following agreements were made regarding to the LCP prioritization.

- For LCP enhancements, LCP Prioritisation: Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR; The solution should consider impact on UE complexity (as already indicated in SI objective description); For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data. FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH.

To prioritize the transmission of the data with low remaining time, one efficient way is to introduce additional priority configurations for LCH with delay-critical data. However, it is still undetermined whether the priority only applies to the LCHs with delay-critical data or to the whole LCH.

If the priority only applies to delay-critical data, the network configures the higher new priority to delay-critical data and configures the legacy priority to the other data. Upon reception of the UL grant, the UE performs prioritization among the delay-critical data firstly, and then performs prioritization for other data. In this way, the UE needs to perform two rounds of LCH prioritization, introducing extra delay.

Observation: If additional priority only applies to delay-critical data, the UE needs to perform two rounds of LCH prioritization, introducing extra delay.

One the other hand, if the additional priority applies to the whole LCH. The network can adjust the priority of LCH with delay-critical data or other data uniformly. Specifically, the network can allocate additional higher priority for LCH with low remaining time, and allocate the legacy priority to the other data. Upon reception of the resource, UE can allocate resources based on the priority of LCH in descending order. There is no need to perform two rounds of prioritization. To reduce the processing latency and complexity, it is preferred to apply the additional priority to the whole LCH.

Proposal 1: To reduce latency and complexity, applying the additional priority to the whole LCH is preferred.

2.2 LCH Restrictions

In the last meeting, the agreements of LCH restrictions are as follows.

- For LCP enhancements, LCP Restrictions: We try to avoid RAN1 impacts; RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way; For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial; The solutions should not disallow non-delay critical data from using an UL grant.

Per TS38.321, RRC configures mapping restrictions for each LCH to control the LCP procedure. Based on the LCH restrictions, the UE decides which logical channel can use the grant. The existing LCH restrictions shown as follow.

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel: - allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission; - maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission; - configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission; - allowedServingCells which sets the allowed cell(s) for transmission; - allowedCG-List which sets the allowed configured grant(s) for transmission; - allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission; - allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

Similarly, the network can configure new LCH restriction to UE to prioritize the transmission of the data with low remaining time. However, from the network capacity point of view, it is inefficient to restrict the resource only for delay-critical data. For instance, in case the data volume of delay-critical data is small, restricting the resource disallowed for non-delay critical data will result in resource waste. Therefore, the new LCP restriction allows LCH with delay-critical data to use UL grant first. After that, the remaining resource can also be allocated to other LCH with non-delay critical data. In other words, the new LCP restriction only allows LCH with delay-critical data to have priority over LCH with non-delay critical data in resource allocation.

Proposal 2: The new LCP restriction only allows LCH with delay-critical data to have priority over LCH with non-delay critical data in resource allocation.

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation: If additional priority only applies to delay-critical data, the UE needs to perform two rounds of LCH prioritization, introducing extra delay.

Proposal 1: To reduce latency and complexity, applying the additional priority to the whole LCH is preferred.

Proposal 2: The new LCP restriction only allows LCH with delay-critical data to have priority over LCH with non-delay critical data in resource allocation.

- References

- R2-240xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan