3GPP TSG-RAN WG2 Meeting #127	R2-2406742

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.5

Source:	China Telecom

Title:	Discussion on RLC enhancements for XR

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, the RLC enhancements for XR were widely discussed, and some agreements were made regarding to unnecessary retransmission avoidance and timely RLC retransmission.

- For avoiding unnecessary retransmissions: For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - 1. Rx initiated approach - 2. Tx initiated approach - RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. The solution should consider impact on UE complexity (as already indicated in SI objective description); For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs; - Tx side stops retransmit obsolete SDUs; - Rx side updates state variables according to the information from Tx side. For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter; - Tx side just processes the status report as in legacy; - FFS how Rx side determines that an SDU should be abandoned. - For autonomous retransmissions: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered); - Retransmission based on enhanced status report; - Retransmission based on enhanced polling; - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism. Impact on capacity should be considered; RAN2 focuses on the enhancements for UL traffic.

In this contribution, we further discuss the remaining issues of RLC retransmission and give some considerations.

2 Discussion

2.1 Avoid unnecessary retransmission

- Transmitter-based mechanism

Per current specification, the transmitter has the knowledge of the data buffered and the remaining time of the PDUs. Based on the PSDB and buffered time, the transmitter can assess the remaining time for PDU retransmission. Once the remaining time runs out, the transmitter can discard PDUs from PDU retransmission buffer. After discarding PDUs, the transmitter notifies the receiver about the SN gap. A new RLC control PDU, similar to status PDU, can be introduced to inform the receiver AM RLC entity about the discarded RLC data PDUs.

Proposal 1: Based on the PSDB and buffered time, RLC transmitter discards PDUs from retransmission and indicates the discarded PDUs to the receiver by a new RLC control PDU, similar to the status PDU.

In addition, the transmitter can control the maximum retransmission number to avoid unnecessary transmission. In legacy, the transmitter can use maxRetxThreshold to control the number of retransmission corresponding to RLC SDU. Once the RLC retransmission time reaches maxRetxThreshold, the UE triggers RLF. To avoid unnecessary RLC triggering, if the network configures the maxRetxThreshold to avoid unnecessary retransmission for the time sensitive services, it may also need to indicate the UE whether to trigger the RLF when maxRetxThreshold is reached.

Proposal 2: RLC transmitter controls the maximum retransmission number to avoid the unnecessary transmission.

- Receiver-based mechanism

As for receiver-initiated abandoning of SDUs, a new timer, similar to t-Reassembly in RLC UM, is introduced. Upon the new timer expires, the receiver abandons the un-received SDUs. After that, the receiver indicates the transmitter about the abandoned SDUs. Based on it, the transmitter advances its transmitting window accordingly. To reduce specification complexity, the abandoned SDUs indication can be included in the RLC status PDU as a part of status report. However, due to the receiver does not know the remaining time of the PDU, the determination of new time value may need some assistance information from the transmitter.

Proposal 3: A new timer is introduced to assist RLC receiver abandon un-received SDUs, but the determination of timer value needs some assistance information from the transmitter.

2.2 Autonomous Retransmissions

Per TS38.322, the receiving AM RLC entity sends status reporting to transmitting AM RLC entity to provide ACK/NACK status of RLC SDUs. Only when the transmitter receives the NACK status, it triggers retransmission of RLC SDUs. However, the receiving AM RLC does not trigger the status report immediately. The triggering conditions to initiate the STATUS report includes:

- receiving the polling from its peer AM RLC entity,

- detection failuer of an AMD PDU reception.

Obviously, legacy status reporting introduce extra latency for RLC SDU retransmission, which is not suitable for an RLC PDU with a small packet delay budget (PDB). To reduce the latency, one efficient way is to trigger the status report immediately when the receiving RLC entity detects a missing RLC PDU. However, it results in huge signalling overhead.

Proposal 4: Triggering status report immediately upon detecting a RLC PDU missing results in huge signalling overhead.

Per current specification, upon reception of an out of order RLC SDT, the receiver starts t-StatusProhibit timer. Only when the timer expires, the receiver reports its RLC status to the transmitter. The timer values is {ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, …}. Therefore, another way to reduce the latency is to reduce the timer granularity to 1ms, e.g. timer values {ms0, ms1, ms2, ms3, ms4, ms5, …}. In some way, triggering status reporting immediately can be considered as timer values equal to 0ms. Compared to the trigger status reporting immediately, reducing the t-StatusProhibit timer granularity gives more flexibility. For some XR traffic with stringent latency requirement, smaller timer values are used to reduce latency. While for some other traffic without very strict latency requirement, larger timer values are used to reduce the signalling overhead. Therefore, improving the t-StatusProhibit timer accuracy is one efficient way to achieve timely retransmission on RLC layer.

Proposal 5: Reducing the t-StatusProhibit timer granularity is one efficient way to achieve timely retransmission on RLC layer.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: Based on the PSDB and buffered time, RLC transmitter discards PDUs from retransmission and indicates the discarded PDUs to the receiver by a new RLC control PDU, similar to the status PDU.

Proposal 2: RLC transmitter controls the maximum retransmission number to avoid unnecessary retransmission.

Proposal 3: A new timer is introduced to assist RLC receiver abandon un-received SDUs, but the determination of timer value needs some assistance information from the transmitter.

Proposal 4: Triggering status report immediately upon detecting a RLC PDU missing results in huge signalling overhead.

Proposal 5: Reducing the t-StatusProhibit timer granularity is one efficient way to achieve timely retransmission on RLC layer.

- References

- R2-240xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan