3GPP TSG-RAN WG2 Meeting #127	R2-2406746

Maastricht, NL, 19 – 23 August, 2024

Agenda Item	: 7.15.2 (NR_SL_enh2)

Source	: LG Electronics Inc.

Title	: Discussion on carrier selection for SL MAC CE(s)

Document for	: Discussion and Decision

1.	Introduction

This contribution discusses carrier selection for SL MAC CE(s).

2.	Discussion

At the #126 meeting, RAN2 discussed as follows regarding the carrier selection procedure for SL MAC CEs.

#126’s RAN2 discussion [1] for the carrier selection procedure:

R2-2405698 Discussion on carrier selection for SL MAC CE(s) LG Electronics Inc. discussion NR_SL_enh2 Proposal 1-1. UE selects a carrier on which the SL IUC Request was received as a carrier for transmission of a SL IUC Information MAC CE. Proposal 1-2. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data. [Lenovo, Ericsson]: For P1-2, we don’t need any new thing. For P1-1, we can consider similar approach as SL CSI reporting MAC CE. [Vivo]: Support the P1-1 and P1-2. [Apple]: We need to be careful on the determined carrier for IUC since IUC was mainly led by RAN1. [Lenovo]: For P1-2, without this additional change, it can still work with some delay. We had some special handling for SL CSI reporting MAC CE because it is sensitive to delay. Not sure for others. [LG]: In the current MAC, if the grant already configured is not enough to add MAC CE, MAC will generate new grant for MAC CE transmission and it can trigger TX carrier (re)selection. If we follow what Lenovo mentioned, it is not aligned with the current MAC. - P1-1 is agreed. - Revisit P1-2 next meeting.

According to the RAN2 agreement, RAN2 can discuss carrier selection procedure for SL IUC Request MAC CE, condition based SL IUC Information MAC CE and SL DRX Command MAC CE.

As shown below, in legacy single carrier operation, the UE generates a SL grant for transmission of SL MAC CE(s) when SL CSI Reporting/SL DRX command indication/SL IUC Information reporting/SL IUC Request is triggered and there is no SL-SCH resources that can accommodate this SL MAC CE(s).

5.22.1.1 in TS 38.321:

#### 5.22.1.1 SL Grant reception and SCI transmission (skipped) 1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmission(s) of a single MAC PDU, and if SL data is available in a logical channel, or an SL-CSI reporting is triggered, or a Sidelink DRX Command indication is triggered or a Sidelink Inter-UE Coordination Information reporting is triggered, or a Sidelink Inter-UE Coordination Request is triggered; or 1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmission of a single SL-PRS, which has been triggered by the upper layer or by the reception of a SCI from a peer UE: 2> if single carrier frequency is configured: (skipped) 3> else if an SL-CSI reporting or a Sidelink DRX Command or a Sidelink Inter-UE Coordination Request or a Sidelink Inter-UE Coordination Information is triggered: 4> select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or SL-PRS dedicated resource pool, if configured. (skipped) 2> perform the TX resource (re-)selection check on the selected pool of resources as specified in clause 5.22.1.2; 2> if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check (skipped) 3> if the selected resource pool is not SL-PRS dedicated resource pool: 4> select the number of HARQ retransmissions from the allowed numbers, if configured by RRC, in sl-MaxTxTransNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped in sl-MaxTxTransNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) and pending SL-PRS transmission(s), if available allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available or the corresponding sl-DefaultCBR-PartialSensing configured by RRC if partial sensing is selected and CBR measurement results are not available, or the corresponding sl-DefaultCBR-RandomSelection configured by RRC if random selection is selected and CBR measurement results are not available in case the sl-TxPoolExceptional is not used; NOTE 3Ad: For Multi-consecutive slots transmission as specified in clause 8.1.4 of TS 38.214 [7], during resource (re)selection, leave it to UE implementation, regarding whether to calculate the number of HARQ retransmissions from the allowed numbers based on the number of MCSt transmissions, or the number of slot(s) within Multi-consecutive slots transmission. 4> select an amount of frequency resources within the range, if configured by RRC, between sl-MinSubChannelNumPSSCH and sl-MaxSubChannelNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped between sl-MinSubChannelNumPSSCH and sl-MaxSubChannelNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) and pending SL-PRS transmission(s), if available allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available or the corresponding sl-DefaultCBR-PartialSensing configured by RRC if partial sensing is selected and CBR measurement results are not available, or the corresponding sl-DefaultCBR-RandomSelection configured by RRC if random selection is selected and CBR measurement results are not available in case the sl-TxPoolExceptional is not used; (skipped) 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is not configured by RRC: 4> if transmission based on random selection is configured by upper layers: 5> if the selected resource pool is not SL-PRS dedicated resource pool: 6> randomly select the time and frequency resources for one transmission opportunity from the resources pool which occur within the SL DRX Active time, if configured, as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, and the pool(s) in which all RB sets had Sidelink consistent LBT failure detected and not cancelled, if configured, according to the amount of selected frequency resources, the remaining PDB of SL data available in the logical channel(s), and the remaining SL-PRS delay budget of the SL-PRS transmission(s), if available, allowed on the carrier, and the latency requirement of the triggered SL-CSI reporting. (skipped)

The purpose of the procedure which is specified in the 5.22.1.11 is to select a TX carrier for grant generation. However, in the MAC specification, there is no carrier selection procedure for SL IUC Request MAC CE, condition based SL IUC Information MAC CE and SL DRX Command MAC CE. So UE can’t generate SL grant for these SL MAC CEs. Therefore, behaviour by which the UE can perform a carrier selection for the SL grant generation for the SL IUC Request MAC CE, the condition based SL IUC Information MAC CE and the SL DRX Command MAC CE should be specified in 5.22.1.11.

- Carrier selection for SL IUC Request MAC CE and Condition based SL IUC Information MAC CE

Just as the operation of logical channel data and MAC CE were not defined differently in the grant creation procedure of a single carrier, the carrier selection procedure for SL IUC Request MA CE/Condition-based SL IUC Information MAC CE in multi-carrier can use the same procedure as that of logical channel data. Single carrier procedure can be reused with regard to pool selection.

- Carrier selection for SL DRX command MAC CE

Same as SL IUC Request MAC CE’s case

Example of TP (SL IUC Request MAC CE/Condition-based SL IUC Information MAC CE/SL DRX command MAC CE):

#### 5.22.1.1 SL Grant reception and SCI transmission #### 5.22.1.11 TX carrier (re-)selection The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.215 [24] if CBR measurement results are available, or the corresponding sl-defaultTxConfigIndex configured by upper layers if CBR measurement results are not available.If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, 5.22.1.2 or 5.22.1.3.3, the MAC entity shall: 1> if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available, or a Sidelink Inter-UE Coordination Information MAC CE triggered by a condition, or a Sidelink Inter-UE Coordination Request MAC CE, or a Sidelink DRX Command MAC CE as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]): 2> for each carrier configured by upper layers associated with the concerned sidelink logical channel: 3> if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: NOTE 1: In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation, taking into account of sl-HARQ-FeedbackEnabled for the sidelink logical channel. 4> consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel. 1> else: 2> for each sidelink logical channel, if any, where data is available, or a Sidelink Inter-UE Coordination Information MAC CE triggered by a condition, or a Sidelink Inter-UE Coordination Request MAC CE, or a Sidelink DRX Command MAC CE and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1, if the CBR of the carrier is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel, for each sidelink logical channel, if any, where data is available, or a Sidelink Inter-UE Coordination Information MAC CE triggered by a condition, or a Sidelink Inter-UE Coordination Request MAC CE, or a Sidelink DRX Command MAC CE and that are allowed on the carrier for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1: 3> select the carrier and the associated pool of resources. 2> else: 3> for each carrier configured by upper layers on which the sidelink logical channel, or a Sidelink Inter-UE Coordination Information MAC CE triggered by a condition, or a Sidelink Inter-UE Coordination Request MAC CE, or a Sidelink DRX Command MAC CE is allowed, if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel: 4> consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel, or a Sidelink Inter-UE Coordination Information MAC CE triggered by a condition, or a Sidelink Inter-UE Coordination Request MAC CE, or a Sidelink DRX Command MAC CE is allowed.; The MAC entity shall: 1> if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection: 2> if Tx carrier (re-)selection is triggered, for each sidelink logical channel allowed on the carrier where data is available: 3> select one or more carrier(s) among the candidate carriers with increasing order of CBR from the lowest CBR, and select the associated pool(s) of resources: 4> if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel: 5> select one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 4> else: 5> select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 2> else if Tx carrier (re-)selection is triggered, for a Sidelink DRX Command MAC CE or a Sidelink Inter-UE Coordination Request MAC CE: 3> select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or SL-PRS dedicated resource pool, if configured. NOTE 2: It is left to UE implementation how many carriers to select based on UE capability. NOTE 3: It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered. NOTE 4: It is left to UE implementation to determine whether the resource pool for CBR measurement is reused as the resource pool for SL grant creation.

Observation 1. In legacy single carrier operation, the UE generates a SL grant for transmission of SL MAC CE(s) (i.e., SL CSI Reporting MAC CE, SL DRX Command MAC CE, SL IUC Request/Information MAC CE) when SL MAC CE’s transmission is triggered and there is no SL-SCH resources that can accommodate the SL MAC CE.

Observation 2. However, in the MAC specification, there is no carrier selection procedure for SL IUC Request MAC CE, condition based SL IUC Information MAC CE and SL DRX Command MAC CE. So UE can’t generate an SL grant for these SL MAC CEs. Therefore, behaviour by which the UE can perform a carrier selection for grant generation for the SL IUC Request MAC CE, the condition based SL IUC Information MAC CE and the SL DRX Command MAC CE should be specified in 5.22.1.11.

Proposal 1. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data.

3.	Conclusion

This contribution shares our views on carrier selection for SL MAC CE(s), which can be summarized as follows:

Observation 1. In legacy single carrier operation, the UE generates a SL grant for transmission of SL MAC CE(s) (i.e., SL CSI Reporting MAC CE, SL DRX Command MAC CE, SL IUC Request/Information MAC CE) when SL MAC CE’s transmission is triggered and there is no SL-SCH resources that can accommodate the SL MAC CE.

Observation 2. However, in the MAC specification, there is no carrier selection procedure for SL IUC Request MAC CE, condition based SL IUC Information MAC CE and SL DRX Command MAC CE. So UE can’t generate an SL grant for these SL MAC CEs. Therefore, behaviour by which the UE can perform a carrier selection for grant generation for the SL IUC Request MAC CE, the condition based SL IUC Information MAC CE and the SL DRX Command MAC CE should be specified in 5.22.1.11.

Proposal 1. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data.

4.	Reference

[1] Session chair’s note for R18 SL-Evo of #126 meeting (Samsung)