3GPP TSG-RAN WG2 Meeting #127 	R2-2406747

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Agenda Item:	8.2.3

Source:	Spreadtrum Communications

Title:	Discussion on paging procedure of AIoT

Document for:	Discussion and Decision

Introduction

In the RAN2 #125bis and #126 meeting, some progress was made on AIoT paging as follows:

RAN2 #125bis Agreements 1. Legacy paging message for device will not be supported. 2. Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). 3. RAN2 assumes that the device will not support tracking/RAN area update procedure. 4. For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2 RAN2 #126 Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this contribution, the paging procedure and functionality will be analyzed. And we provide our opinions.

Discussion on paging

2.1 AIoT paging functionality

For AIoT paging, it is beneficial to clarify concepts like “round” and “access slot/occasion”. Similar to RFID, different access round can be divided by receiving various AIoT paging messages. And in one round, there are multiple access occasion/slot. The number of access occasion in one round is indicated by reader in AIoT paging message. The boundary of the access occasion can be defined/indicated by the reader sending paging repetition or other messages.

Proposal 1: The boundary of the access occasion can be defined/indicated by the reader sending paging repetition or other message.

Figure 1 A-IoT procedure

2.2 Message content

2.2.1 Initial trigger message for inventory only, inventory and command procedure

For AIoT paging, one issue is what information included in the AIoT paging message from reader to device(s). In last meeting, RAN2 agreed that an identifier is required for AIoT paging message.

Figure 2 Inventory only procedure

For inventory procedure, after reader received service request, reader performs AIoT paging procedure with AIoT devices. In addition to ID information, the paging messages also encompass the following information:

- Service type: To indicate whether paging messages are for inventory or command operations. And the service is for inventory, the paging message will indicate inventory. If this procedure is for inventory and command procedure, the paging message can carry command indication as well as specific operations, such as reader, write etc.

- Resource allocation information: Scheduling information to indicate the frequency/time resource for the AIoT device to respond. For time domain, the number of access occasions within one access round is indicated by the reader.

- RACH indication: For initial inventory overall procedure, multiple devices need to respond, and contention-based random access should be adopted. And when a single device or a small number of devices are required, contention-free random access procedure can be used. In last meeting, it was agreed that AIoT might involve a 4-step CBRA, 2-step CBRA and CFRA procedure. The reader can instruct the device on which random access procedure to use.

Proposal 2: For inventory only, inventory and command procedure, in addition to ID information, the paging message also includes service type, resource allocation information and 2/4-step CBRA, CFRA RACH indication.

Proposal 3: For time domain access resource, the total number of access occasions (e.g., Q value) within one access round is indicated to device by the reader.

2.2.2 Initial trigger message for command only procedure

For command only procedure, it is FFS on whether initial trigger from the reader contains the command. From our understanding, even for the case that a device is already known to the AF and AF tends to send specific command (e.g., to read the device’s data or write some information into the device) to the device, it’s beneficial to inventory this device first. Because the device may have moved and is no longer in the coverage of last reader. So it is necessary to perform an inventory on this device to detect its presence. This can be helpful to avoid unnecessary signalling overhead by sending data to non-existent device(s).

Observation 1: For command only procedure, it is beneficial to firstly inventory this device, even though a device may have been previously inventoried.

Figure 3 Command only procedure

For command only procedure, after reader received service request, reader performs AIoT paging procedure with AIoT devices. In addition to ID information, the paging messages also encompass the following information:

- Service type: To indicate whether paging messages are for command operations. And further the paging message can carry command indication as well as specific operations, such as reader, write etc.

- Resource allocation information: Scheduling information to indicate the frequency/time resource for the AIoT device to respond. For time domain, the number of access occasions within one access round (e.g., Q value) is indicated by the reader.

- RACH indication: For initial inventory overall procedure, multiple devices need to respond, and contention-based random access should be adopted. And when a single device or a small number of devices are required, contention-free random access procedure can be used. In last meeting, it was agreed that AIoT might involve a 4-step CBRA, 2-step CBRA and CFRA procedure. The reader can instruct the device on which random access procedure to use.

Proposal 4: For command only procedure, in addition to ID information, the paging message also includes service type, resource allocation information and 2/4-step CBRA, CFRA RACH indication.

2.3 Monitoring behaviour

According to our understanding, the entire AIoT system is an asynchronous system, because the AIoT device has ultra-low complexity with ultra-low power consumption, and its initial sampling frequency offset (SFO) is 104-105 ppm, which will bring timing and frequency error.

Observation 2: AIoT system is an asynchronous system

RAN2 #126 agreement: From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

For device’s monitoring behaviour of AIoT paging message, and whether it is necessary to define a paging cycle and paging occasion. In last meeting, RAN2 assume the device can receive as long as there is enough energy. For device monitoring details, we will wait for RAN1 further progress.

In RAN plenary #104, energy harvesting impacts are discussed and the following proposal was formulated after the meeting discussion. Although the proposal has not yet reached a conclusion. We need to revisit this issue.

RAN #104 proposal： - RAN WGs are directed to study energy harvesting impacts in the following way: - Device 1 is assumed to have two states: ON, OFF - Device 2a/2b is assumed to have three states: ON, OFF, SLEEP - Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to: - ON state supports at least: transmission, reception for communication - OFF state does not support at least: transmission, reception for communication - OFF state supports at least: energy harvesting - SLEEP state supports at least: - maintaining a memory content from ON state - Maintaining a timer (RAN1 to discuss purpose(s) of timer) - SLEEP state does not support at least: transmission - No additional physical layer signals/channels specific to support of SLEEP are introduced - Identify reader knowledge / control of the above states - Approximately identify durations of the above device states, by company reports. RAN1 is not required to establish a consensus model. - Companies can structure their report(s) of durations in their preferred way - No modeling of device/component (dis)charging characteristics - This part is aimed to be completed in RAN1#118, with RAN#105 to review progress and finalize if necessary - RAN1 & RAN2 co-lead

In RAN plenary #104 meeting, for different device, there are different assumption about the supported states. Device 1 is assumed to have two states: ON and OFF states. Of course, what states of device 1 are supported is depend on RAN1 discussion. And if RAN1 agreed that ON and OFF states are supported. We need to analyse the spec impact to RAN2. For off state, the device does not support at least: transmission, reception for communication. So in the OFF state, only energy harvest can be supported, hence in OFF state, device is shutting down because the battery is exhausted. Therefore, from the RAN2 perspective, there should be no protocol impact for device 1.

Proposal 5: If device 1 support ON and OFF states, there is no spec impact to RAN2.

Device 2a/2b is assumed to have three states, including ON, OFF, SLEEP. On state can support at least: transmission, reception for communication. Sleep state does not support at least: transmission. From the behaviour of the on and sleep states, it can be seen that the ON state is akin to the active state in DRX, while the sleep state is similar to the inactivity state in DRX. To support the ON and sleep states of Device 2a/b, DRX-like DRX duty cycle may be required. And for example, RAN2 can support timer based DRX mechanism. But the SFO impact on timer accuracy needs to further study.

Proposal 6: If Device 2a/b support ON, OFF and SLEEP states, DRX-like duty cycle mechanism may be necessary.

Conclusions

In this contribution, we discuss the Rel-19 paging procedure and functionality. And the proposals are given as follows:

Proposal 1: The boundary of the access occasion can be defined/indicated by the reader sending paging repetition or other message.

Proposal 2: For inventory only, inventory and command procedure, in addition to ID information, the paging message also includes service type, resource allocation information and RACH indication.

Proposal 3: For time domain access resource, the total number of access occasions within one access round is indicated to device by the reader.

Observation 1: For command only procedure, it is beneficial to firstly inventory this device, even though a device may have been previously inventoried.

Proposal 4: For command only procedure, in addition to ID information, the paging message also includes service type, resource allocation information and RACH indication.

Observation 2: AIoT system is an asynchronous system

Proposal 5: If device 1 support ON and OFF states, there is no spec impact to RAN2.

Proposal 6: If Device 2a/b support ON, OFF and SLEEP states, DRX-like duty cycle mechanism may be necessary.

References

[1] RP-234058, Rel-19 Ambient IoT SID, RAN #102 meeting.

[2] TR 23.700-13, SA2 Technical report of Study on architecture support of ambient power-enabled IoT.