3GPP TSG-RAN WG2 Meeting #127 	R2-246749

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Agenda Item:	8.5.2

Source:	Spreadtrum Communications

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion and Decision

Introduction

Based on the outcome of RAN#102 meeting, the following objectives were agreed in the WID (RP-234065):

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we will discuss the issues related to the on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. And in our contribution, we will discuss the issues related to scenarios and the activation/deactivation of on-demand SSB operations and provide our opinions.

On-demand SSB SCell operation

Use cases and scenarios

RAN1 #116 Agreements Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission RAN1 #116bis Agreements For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

The following agreements about on-demand SSB is achieved in RAN1 #116 [1] and 116-bis meeting [2]. There are two cases and two scenarios. Among them, case #1: no always-on SSB means SSB-less for SCell. And case #2 is equivalent to always-on SSB plus on-demand SSB.

For scenario #2, it is one of the most typical scenarios that should be considered for SCell on-demand SSB. The SCell activation and on-demand SSB trigger can happen at the same time. For scenario #3, after SCell activation, on demand SSB SCell operation can be triggered to change the periodicity of SSB for network energy saving gain. In RAN1#116bis, Scenario #2 and Scenario #3 were further elaborated. Scenario #2A, Scenario #3A and Scenario #3B were introduced, and at least Scenario #2 and Scenario #2A are can be supported.

From our view, both scenarios can be supported. So, RAN2 can focus on two scenarios for further study.

Proposal 1: RAN2 wait RAN1 to conclude down-selection of use cases of on-demand SSB..

Triggering Methods of on-demand SSB SCell operation

According to the WID scope, the following three candidate triggering methods will be discussed for on-demand SSB:

- Method 1: UE uplink wake-up-signal using an existing signal/channel

- Method 2: Cell on/off indication via backhaul

- Method 3: SCell activation/deactivation signaling

The method 1 is UE-triggered on-demand SSB operation. And method 2 and 3 are network-triggered on-demand SSB operation.

For method 1, UE transmits UL UWS to Pcell or Scell to trigger gNB to transmit on-demand SSB in Scell.

For method 2, it means that the PCell triggers the SCell to transmit on-demand SSB via backhaul. From UE perspective, UE receives indication of on-demand SSB to be transmitted by PCell. So method 2 needs to be combined with method 3 to trigger the on-demand SSB operation. We can wait for RAN3 progress.

For method 3, UE is informed by gNB via Scell activation/deactivation signaling that on-demand SSB is to be transmitted in Scell. It is the execution signalling for triggering on-demand SSB operation, which means that method 3 is a mandatory method and can be used in combination with method 1 or 2.

According to RAN1 #116 progress, at least the following scenarios (i.e. Scenario #2 and Scenario #3) and two case (case #1 and case #2) should be discussed. And according to RAN1 #116bis agreement, method 3 has two options (i.e. single signaling and separate signaling for on-demand SSB indication and SCell activation command).

Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication RAN1 #117 Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

For scenario 2, Scell is configured for UE but UE does not receive Scell activation. UE should consider scell as serving cell as long as Scell is configured. In this case, PCell can transmit a Scell activation/deactivation signaling to UE to activate Scell and trigger on-demand SSB operation simultaneously, that is Option 2 in RAN1 agreement. Before on-demand SSB triggering, there may be no SSB or has always-on SSB.

For scenario 3, Scell is configured and activated. In this case, Pcell can also transmit a Scell activation/deactivation signaling to UE to modify the periodicity of SSB. And the specific signalling can reuse SCell activation/deactivation MAC CE, that is Option 2 in RAN1 agreement. It may be necessary to add an indication to indicate whether the on-demand SSB or the SCell is activated/deactivated. Or we can introduce a new on-demand SSB activation/deactivation MAC CE to trigger on-demand SSB operation, that is Option 1 in RAN1 agreement.. And before on-demand SSB triggering, on-demand SSB configuration can be configured by RRC signalling to UE.

Proposal 2: For on-demand SSB SCell operation, gNB can reuse SCell activation/deactivation MAC CE or introduce a new on-demand SSB activation/deactivation MAC CE to activate/deactivate on-demand SSB SCell operation.

- 2.3 L3 measurement based on on-demand SSB

In RAN1 #116b [1], it was agreed to support L1 and/or L3 measurement based on on-demand SSB. L3 measurement is RAN2 scope and we should lead the discussion.

Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement

L3 measurement main includes RRM. And we will discuss RAN2 impacts and give our views.

In current spec, network provides L3 mesaurement configuration on a serving cell via dedicated RRC messages. Regarding L3 measurement, the RRC configuration contains Measurement Objects (MO), reporting configuration and quantity configurations etc.

For on-demand SSB, there are serval scenarios and cases. When SCell is configured but before activated, that is scenario 2, L3 measurement can be operated for quality detection of the SCell. And L3 measurement results can be used by the gNB to determine when to activate the SCell. During SCell activation procedure, that is scenario #3A, L3 measurement can be used to obtain accurate time/frequency synchronizaion. After SCell is activated, that is scenario #3B, L3 mesaurement are needed to maintain the radio link stability. The above analysis of different scenarios is applicable at least to case 1. Because for case 1, there is no always-on SSB on the SCell, so UE can perform L3 measurement for on-demand SSB. And for case 2, always-on SSB is periodically transmitted on the cell. We can discuss whether L3 measurement of on-demand SSB is needed.

Proposal 3: L3 measurement based on on-demand SSB is necessary before/during/after SCell activation at least for case #1.

- RRM based on on-demand SSB

In the legacy NR RRM based on SSB, the network will configure SMTC to measure SSB. The SMTC is configured by SCellconfig or MeasObjectNR of Scell for SSB measurement.

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon      OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig            OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                      OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}           OPTIONAL,   -- Cond SCellAddSync

Omit unrelated part

]]

}

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

omit unrelated part

}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

omit unrelated part

}

According to RAN1 #116b updated agreement, the periodicity of on-demand SSB can be changed. And the on-demand SSB can be semi-persist or one-short transmittion. And RAN1 may introduce MAC CE or DCI to indicate UE on-demand SSB transmission. The legacy SMTC configuration of SSB may not work because RRC signalling is slower that MAC CE and DCI. And RRM based on on-demand SSB may failure.

Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Proposal 4: RAN2 study the new L3 RRM configuration (e.g. SMTC) for on demand SSB according to RAN1 conclusion about the transmission pattern and signaling of on-demand SSB.

Conclusions

In this contribution, we discuss the Rel-19 on-demand SSB SCell operation issues. And the proposals are given as follows:

Proposal 1: RAN2 can focus on scenario 2/3 and case 1/2 for further study.

Proposal 2: For on-demand SSB SCell operation, gNB can reuse SCell activation/deactivation MAC CE or introduce a new on-demand SSB activation/deactivation MAC CE to activate/deactivate on-demand SSB SCell operation.

Proposal 3: L3 measurement based on on-demand SSB is necessary before/during/after SCell activation at least for case #1.

Proposal 4: RAN2 study the new L3 RRM configuration (e.g. SMTC) for on demand SSB according to RAN1 conclusion about the transmission pattern and signaling of on-demand SSB.

References

[1] RAN1 #116 meeting chair notes.

[2] RAN1 #116bis meeting chair notes.