3GPP TSG-RAN WG2 Meeting #127	R2-2406751

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.2.2

Source:	Spreadtrum Communications

Title:	Discussion on the functionalities required for Ambient IoT

Document for:	Discussion and Decision

1 Introduction

In RAN2#126 meeting [1], RAN2 discussed the functionalities required for A-IoT and achieved the following agreements:

Agreements on functionality 1 Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. 2 legacy NR BSR/SR is not needed for A-IoT communication. 3 FFS whether further indication of device message size/status is needed. 4 AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed).

In this contribution, we will continue discussion the functionalities required for A-IoT.

2 Discussion

## 2.1 The protocol stack of A-IoT

In RAN2 last meeting, it agreed that SDAP, PDCP and RLC layer are not needed for A-IoT. RAN2 assumes that RRC layer is not necessary between the reader and the device, RAN2 later discusses whether we will have a new AS protocol on top of A-IoT MAC layer or A-IoT MAC.

In our view, to meet the compact protocol stack system design objective for A-IoT, a new AS protocol on the top of A-IoT MAC is not needed. The necessary function A-IoT device required may done in MAC layer, we should define a new MAC layer for A-IoT.

Proposal 1: No a new AS protocol on top of A-IoT MAC layer.

Only MAC and PHY protocol layers are needed in the protocol stack for A-IoT, the compact protocol stack for A-IoT as following Figure 1.

Figure 1 A-IoT protocol stack for topology 1

Proposal 2: For A-IoT topology 1, RAN2 agrees the protocol stack in Figure 1.

## 2.2 Security related questions to SA2 and SA3

In last RAN2 meeting, RAN2 has sent a LS to SA2 and SA3 to inform RAN2 security related agreements/assumptions [2]. RAN2 respectfully asks SA2 and SA3 to take these agreements/assumptions into account for future work and to provide feedback on the aspects which are in the remit of SA2 and SA3 when they have any conclusion.

Besides, RAN2 discussed the command only procedure, and there is a FFS if initial Trigger Message can also include “command” [1].

- From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : - Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. - Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

The initial trigger message is used to identify which devices need to respond. This message may be unprotected. If DL command is included in the initial trigger message, there may be security issue. RAN2 need to consult SA2/SA3 to clarify whether there is a security issue if the command is included in the initial trigger message.

Proposal 3: RAN2 ask SA2/SA3 to clarify whether there is a security issue if the command is included in the initial trigger message.

## 2.3 Functionalities required for A-IoT devices in MAC

Resource allocation

The wireless resource (e.g. time-domain, frequency-domain) used in air interface for A-IoT devices should be allocated. In topology 1, the resource of A-IoT device D2R transmission is allocated by network. Specifically, gNB (reader) can allocate RACH related resource (e.g. time domain resource Q value) by MAC CE to device and data transmission related resource by R2D control information in PRDCH. R2D transmission resource are selected by gNB itself. This is implementation of gNB and has no effect on A-IoT device.

In topology 2, it is gNB allocates R2D and D2R transmission resource used in A-IoT link to intermediate UE. The above R2D and D2R resource can be either dynamically allocated or semi-persistent scheduling.

Proposal 4: For topology 1, gNB allocates the D2R transmission resource to A-IoT device.

- RACH related resource by MAC CE.

- Data transmission related resource by R2D control information in PRDCH.

Proposal 5: For topology 2, gNB allocates R2D and D2R transmission resource used in A-IoT link to intermediate UE.

Segmentation/reassembly

The segmentation and reassembly are functions of RLC layer, the segmentation function requires extra L2 buffer in device. The extra L2 buffer will cause extra memory. Due to the limited memory of the A-IoT device, this function is not expected to be supported.

From RAN2 perspective, to simplify A-IoT design, there is no need to support segmentation for A-IoT data transmission. According the design target for inventory and command use cases in TR 38.848, the maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device. From PHY layer perspective, it is able to provide a TB size of no more than 1000 bits.

Proposal 6: Segmentation is not needed.

Higher layer repetition

RAN1 is studying physical layer repetition to improve the reliability of D2R transmission, and they note that discussions regarding higher-layer repetitions are up to RAN2 [3]. From RAN2 perspective, repetition refers to retransmission.

For the reliability of R2D transmission, it is by reader implementation. If no response is received within the time window from the device after reader R2D transmission, the reader can retransmission R2D message. For the reliability of D2R transmission, there are several ways. Firstly, if reader failed to receive D2R message, it may feedback NACK and allocate retransmission resource to device. Or Device can be (pre)configured by reader to do blind retransmission. Besides, D2R retransmission can be up to device implementation (e.g. the amount of battery left on the device) and the resource for retransmission can be pre-configured.

Proposal 7: R2D repetition can be up to reader implementation.

Proposal 8: D2R repetition can be supported by reader indication or device implementation.

QoS handling

RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL) in last meeting. And there also no SDAP layer, QoS flow to DRB mapping is not required. The LCP will not be needed. Thus, QoS handling in MAC layer is not needed.

Proposal 9: QoS handling in MAC layer is not needed.

Message size indication

As agreed in RAN2 last meeting, legacy NR BSR/SR is not needed for A-IoT communication. For further study whether further indication of device message size/status is needed. In order to make reader better schedule the next D2R transmission, the reader needs to know the message size of next D2R transmission. One possible way is that Device can carry a message size indication during this D2R transmission, indicating gNB that the next D2R message size.

Proposal 10: The message size indication is needed for A-IoT.

## 2.4 Information visible to reader

RAN2 has discussed the level of visibility required by the reader and left a FFS about what information is visible to the reader.

A-IoT services including inventory or command are triggered by CN, it may be that information and signalling is exchanged between devices and CN which is transparent to reader. However, reader also needs to know some necessary information, so that the reader can perform more efficient resource configuration or scheduling.

In our view, the information is required in the reader for A-IoT include the service type of A-IoT (i.e. inventory and/or command). If the service type indicates inventory-only, the reader will trigger another device to inventory after receiving the device ID from one device. If the service type indicates an inventory and command combined procedure, the reader will wait for the following command message after receiving a device ID. Second, the service whether is targeted on single or multiple devices is visible to reader. If single device is triggered, the reader will indicate the device to use contention-free access procedure. If multiple devices are triggered, the reader will indicate the devices to use contention-free access procedure. Moreover, the number of devices is also needed visible to reader, if the service is targeted on multiple devices. So that the reader can allocate corresponding number of access resources.

Proposal 11:  The following information is visible to the reader：

- The service type of A-IoT (i.e. inventory and/or command);

- The service is targeted on single or multiple devices;

- The number of devices, if the service is targeted on multiple devices.

3 Conclusion

In this contribution we discuss the control plane functions and related signalling, with the following proposals:

Proposal 1: No a new AS protocol on top of A-IoT MAC layer.

Proposal 2: For A-IoT topology 1, RAN2 agrees the protocol stack in Figure 1.

Proposal 3: RAN2 ask SA2/SA3 to clarify whether there is a security issue if the command is included in the initial trigger message.

Proposal 4: For topology 1, gNB allocates the D2R transmission resource to A-IoT device.

- RACH related resource by MAC CE.

- Data transmission related resource by R2D control information in PRDCH.

Proposal 5: For topology 2, gNB allocates R2D and D2R transmission resource used in A-IoT link to intermediate UE.

Proposal 6: Segmentation is not needed.

Proposal 7: R2D repetition can be up to reader implementation.

Proposal 8: D2R repetition can be supported by reader indication or device implementation.

Proposal 9: QoS handling in MAC layer is not needed.

Proposal 10: The message size indication is needed for A-IoT.

Proposal 11:  The following information is visible to the reader:

- The service type of A-IoT (i.e. inventory and/or command);

- The service is targeted on single or multiple devices;

- The number of devices, if the service is targeted on multiple devices.

- References

- Draft report of 3GPP TSG RAN WG2 #126, RAN2 chair (interDigtal), Fukuoka, Japan May 20th – 24th, 2024.

- R2-2406150, LS on RAN2 agreements and assumptions for Ambient IoT, RAN2, Fukuoka, Japan, May 20th - 24th 2024.

- Draft report of 3GPP TSG RAN WG1 #116, MCC Support, Athens, Greece February 26th - March 1st, 2024.