3GPP TSG-RAN WG2 Meeting #127	R2-2406752

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.2.4

Source:	Spreadtrum Communications

Title:	Discussion on random access of Ambient IoT

Document for:	Discussion and Decision

1 Introduction

In RAN2#126 meeting [1], RAN2 discussed the random access for A-IoT and achieved the following agreements:

Agreements on “4 step” RA 1 A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information. 2 A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements. 3 A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request). 4 The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. 5 “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access. Agreements on 2 step CB RA 1 A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information. 2 A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed. Agreement on CF RA - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, we will continue discuss the random access of Ambient IoT.

2 Discussion

## 2.1 Contention-based random access for A-IoT

According to the SID [2], further research is needed on DO-A (Device-originated autonomous) traffic type in RAN#104 meeting, the network triggered random access procedure for DO-DTT and DT traffic types should be discuss first. In the following, we will discuss on the A-IoT device random access procedure for DO-DTT and DT traffic type.

For DO-DTT and DT traffic type, the random access of A-IoT device should be firstly initiated by network triggering similar to NR paging triggered random access. The triggering form network may be inventory or command service request. A-IoT device initiates random access procedure to respond to the control information from the inventory or command signals.

Proposal 1: A-IoT device initiates random access procedure to respond to the inventory or command control signalling.

For initial inventory to identify multiple A-IoT devices, when a response is expected from multiple devices, an A-IoT contention-based access (CBRA) procedure initiated by the reader is used. For A-IoT contention-based access procedure, slotted-ALOHA based access is baseline. 4-step CBRA random access for A-IoT is shown in Figure 1.

Msg0: Access trigger. The reader sends signal to trigger A-IoT device to perform access. The trigger signal may be paging/select/query/queryrep/queryadjust message. The resource for device subsequent transmission may also include in the message.

Figure 1 4-step CBRA random access for A-IoT

Msg1: The device sends a random ID to the reader. During the inventory, reader sends a query command which contains a Q value, A-IoT device randomly selects a value from 0 to 2Q-1 as the initial count value, and when the count value is 0 or it is reduced to 0, A-IoT device can send a random ID to reader. If the count value is not equal to 0, device transmits nothing and waits for the next trigger signal indicating the end of the previous slot and the start of the next slot.

Msg2: The reader echos the ID received in Msg1. If reader successful reception the RN16, it sends an ACK which at least carries the same RN16 from the A-IoT device. The resource for device next transmission may also include in the ACK.

Msg3: Device sends Device ID and any other upper layer data (depending on upper layer request). If the A-IoT device receives the ACK with the same RN16 that it sent in Msg1, A-IoT device sends its unique device identity (e.g. like EPC in RFID) to Reader. A-IoT device may also send the device type or capability in the message to reader. If ACK is not received or a different RN16 is received, A-IoT device will not transmit the device ID and waits for the next trigger signal.

Msg4: The reader acknowledgement the reception of Msg3. If reader has subsequent transmission with device, the Msg4 may not needed. The subsequent transmission implicitly indicates that the Msg3 are successful transmission. Otherwise, the reader will send ACK with random ID. Upon receiving the ACK, device considers the access successful. If reader has not received the Msg3 successfully, the reader will send Msg4 with NACK, then the device will retransmit Msg3.

In RAN2 last meeting, it agreed that a random ID is included in Msg1.  RAN2 further study how the random ID is generated (e.g. randomly generated or generated based on Device ID) and the ID size. In RFID, the Random ID (RN16) is randomly generated by device and the size is 16 bits. In a number of 10000 tags, the probability of any two or more tags simultaneously generating the same RN16 sequence is less than 0.1%, regardless of whether the tags are energized or not.

In A-IoT, we prefer the random ID in Msg1 is randomly generated by device like in RFID which is relatively simple. The size of random ID should not be larger than the device ID, but it should not be too short, otherwise there will be many collisions. The size of random ID may need RAN1 further evaluation, can be decided in WID.

Proposal 2: The random ID in Msg1 is randomly generated by device.

If reader has subsequent transmission with device, the Msg4 may not be needed. The subsequent transmission implicitly indicates that the Msg3 are successful transmission. Otherwise, the reader will send Msg4 with ACK (e.g. contains Random ID). Upon receiving the ACK, device considers the access successful. If reader has not received the Msg3 successfully, the reader will send Msg4 with NACK, then the device will retransmit Msg3.

Proposal 3: Msg4 is not needed if reader has subsequent transmission with device.

For some cases (e.g. Low collision probability of access), such as a small number of devices are triggered, Msg1 and Msg2 in 4-step CBRA may not be needed.  For 2-step CBRA, last RAN2 meeting has agreed that A-IoT device can transmit device ID and any other upper layer data directly in Msg1 when receives an initial trigger message. For the further study what device ID is and whether an additional random ID is needed.

In our view, if the device has command service after inventory, it needs a random ID to communication with the reader. In order to reduce the signaling overhead, Msg1 only includes Device ID and any other upper layer data. After reader receiving Msg1, reader generates a random ID for the device based on the device ID. The random ID generation rules are also known to the device side. Then, the random ID is sent to the device in Msg2.

Figure 2 2-step CBRA random access for A-IoT

Proposal 4: Reader echos Msg1 with Random ID in Msg2.

Proposal 5: Msg2 is not needed if reader has subsequent transmission with device.

## 2.2 Contention resolution failure in CBRA

Device only supports time-domain multiple access in RFID, multiple devices may collide in the same access occasion since device is randomly selecting the access occasion. Last RAN2 meeting has agreed that the device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. In other words, device considers the contention resolution failure if the same ID is not received in the response from reader within a certain time, after transmitting the random access ID. There is no handling for failed access in RFID, the devices that experience collisions do not get a chance to access and should wait until the next inventory round to access again upon receiving the new trigger message from reader.

Proposal 6: If contention resolution fails due to collision, A-IoT device will perform the access again in the next access round upon receiving the new trigger message from reader.

## 2.3 Contention-free random access for A-IoT

In last RAN2 meeting, it agreed that for contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion). Chair notes indicate that in the next meeting we will start by discussing single device case for contention-free access.

For dedicated inventory or command for a identified A-IoT device, the specific A-IoT device ID will included in trigger signal in Msg0. Only the device ID of A-IoT device match can initiate access, so it can be a contention-free random access (CFRA). CFRA based access procedure is shown in Figure 3.

Figure 3 CFRA random access for A-IoT

Message 0: Access trigger. The reader sends trigger signal to trigger A-IoT device to perform access. The trigger signal may be paging/query or command (read/write etc.) message which the device ID and resource for RACH are include in.

Message 1: A-IoT device sends device ID and any other upper layer data to reader. If the A-IoT device correctly reception the trigger signal and find the device ID matches, it sends device ID as response. A-IoT device may also send the device type or capability or any other upper layer data within the message to reader.

Message 2: Reader echos Msg1 with Random ID. After reader receiving Msg1, reader generates a random ID for the device based on the device ID. The random ID generation rules are also known to the device side. Then, the random ID is sent to the device in Msg2. If reader has subsequent transmission with device, the Msg2 may not needed. The subsequent transmission implicitly indicates that the Msg1 are successful.

Proposal 7: The CFRA random access procedure for A-IoT as follows can be supported in R19.

- Msg1: A-IoT device sends device ID and any other upper layer data to reader.

- Msg2: Reader echos Msg1 with random ID.

Proposal 8: Msg2 in CFRA random access may not be needed if reader has subsequent transmission with device.

## 2.4 Determine CBRA or CFRA

It has been agreed that random access is triggered by the reader. Therefore, it would be reasonable to leave the reader to decide which type of access procedures to apply. Furthermore, the reader can indicate the appropriate random access type for the device in the trigger message.

Proposal 9: The reader decides the type of random access the device shall use i.e. 2/4 step CBRA or CFRA.

## 2.5 Efficiency enhancement on random access

Slotted-ALOHA is a contention resolution based on TDMA in backscatter wireless system. It is often referred as a probabilistic anti-collision algorithm, only slots that do not collide can be decoded successfully, with an efficiency of 36.8%. The efficiency here is defined as the ratio of the number of devices successfully inventoried to the number of time slots used.

And there indeed is another anti-collision algorithm candidate that is conflict resolution diversity slotted-ALOHA (CRDSA). The CRDSA key idea is to transmit two replicas of the same packet at random two slots within the same frame with a little extra signalling to point to the replica packet location. In case of successful packet reception at network this extra signalling information allows to locate the replica packet within the frame and to accurately cancel it in addition to the one successfully decoded. The effect of the other replica packet is eliminated by iterative self-interference cancellation (SIC) technology. The efficiency of CRDSA is 52%.

Figure 4 Conflict resolution diversity slotted-ALOHA (CRDSA)

| Random access algorithm | Efficiency |
| --- | --- |
| Slotted-ALOHA | 36.8% |
| CRDSA | 52% |

Figure 5 The efficiency of random access algorithm

If there are chances to discuss enhancement for slotted-ALOHA, and the CRDSA access procedure should be considered.

Proposal 10: RAN2 to study CRDSA based access as a candidate for random access.

3 Conclusion

In this contribution we discuss the random access procedure of A-IoT device, with the following proposals:

Proposal 1: A-IoT device initiates random access procedure to respond to the inventory or command control signalling.

Proposal 2: The random ID in Msg1 is randomly generated by device.

Proposal 3: Msg4 is needed only if the reader has not received Msg3 successfully.

Proposal 4: Reader echos Msg1 with Random ID in Msg2.

Proposal 5: Msg2 is not needed if reader has subsequent transmission with device.

Proposal 6: If contention resolution fails due to collision, A-IoT device will perform the access again in the next access round upon receiving the new trigger message from reader.

Proposal 7: The CFRA random access procedure for A-IoT as follows can be supported in R19.

- Msg1: A-IoT device sends device ID and any other upper layer data to reader.

- Msg2: Reader echos Msg1 with random ID.

Proposal 8: Msg2 in CFRA random access may not be needed if reader has subsequent transmission with device.

Proposal 9: The reader decides the type of random access the device shall use i.e. 2/4 step CBRA or CFRA.

Proposal 10: RAN2 to study CRDSA based access as a candidate for random access.

- References

- Draft report of 3GPP TSG RAN WG2 #126, RAN2 chair (interDigtal), Fukuoka, Japan, May 20th – 24th, 2024.

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, RAN#103, March 18 -21, 2024.