3GPP TSG-RAN WG2 Meeting #127	R2-2406753

Maastricht, Holland, Aug 19th – 23rd, 2024

Agenda item:	8.4.2

Source:	Spreadtrum Communications

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion and Decision

1 Introduction

In RAN2#126 meeting, RAN2 discussed the procedure and configuration of LP-WUS in RRC_IDLE INACTIVE and achieved the following agreements [1]:

- RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. - RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. - RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. - From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we will continue discussing the procedure and configuration of LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring.

2 Discussion

## 2.1 LP-WUS configuration

In last RAN2 meeting, it agreed that the LP-WUS related configuration at least includes LP-SS configuration, LP-WUS configuration and entry/exit condition for LP-WUS monitoring. And the LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

Similar to PEI, the LP-WUS configuration is at a cell level for idle/inactive UE, and all the idle/inactive UEs in the cell can use the configuration in the SIB. LP-WUS configuration via dedicated signalling for idle/inactive UE is not needed, it will increase the signalling overhead.

Proposal 1: The dedicated LP-WUS related configuration for IDLE/INACTIVE state is not needed.

According the agreements in RAN2 last meeting, the entry/exit condition for LP-WUS monitoring configuration in SIB may include the threshold(s) of entry/exit LP-WUS monitoring. But for LP-SS configuration and LP-WUS configuration, it highly depends on RAN1 and needs to wait for RAN1 progress. The LP-SS design and LP-WUS design have been discussed in RAN1, but has not concluded yet. RAN2 can postpone the discussion until there is agreement for the LP-SS and LP-WUS design in RAN1.

Proposal 2: The detailed LP-SS and LP-WUS configurations need to wait for RAN1 progress.

## 2.2 Entry/exit conditions for LP-WUS monitoring

LP-WUS has been configured in the cell by SIB, both UE and gNB could apply LP-WUS. Then it need to further determine the entry and exit conditions which UE monitoring LP-WUS. In last RAN2 meeting, RAN2 agreed that if the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

We think whether the measurement of LR is needed or not depends on the configuration of the gNB. If gNB configures same threshold for both LP-WUS monitoring and serving cell RRM measurement offloading. Before UE starting LP-WUS monitoring, UE MR is wake-up and monitors legacy PO, MR also measures the serving cell, so UE use the serving cell measurement result of MR to start LP-WUS monitoring is more suitable due to the measurement accuracy of MR Is higher than that of LR. So the serving cell measurement based on LR as entry condition for LP-WUS monitoring is not needed.

If gNB configures different thresholds, and the threshold for serving cell RRM measurement offloading is stricter than the threshold for LP-WUS monitoring, UE has offloaded the serving cell measurement form MR to LR before LP-WUS monitoring. Then the condition for entry LP-WUS monitoring can be the measurement of LR.

Proposal 3: The serving cell measurement based on LR as entry condition for LP-WUS monitoring is not needed, if MR has been measuring the serving cell before LP-WUS monitoring.

Proposal 4: The serving cell measurement based on LR as entry condition for LP-WUS monitoring is needed, if the serving cell measurement has offloaded form MR to LR before LP-WUS monitoring.

## 2.3 Subgrouping methods for LP-WUS

It agreed that the PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In Rel-17, subgroup indication up to 8 subgroups are carried by PEI for an associated PO. In LP-WUS, RAN1 had discussed and agreed that the maximum number of subgroups per PO is X, where 8 <= X <= 256 [2]. Specifically, this is related to the specific content (e.g. bitmap or codepoint) carried in the LP-WUS. It has been discussed in RAN1, but has not concluded yet.

Agreement For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. • FFS the exact value of X. For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS: Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N. Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N. - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1

From RAN2 perspective, large subgroup in LP-WUS means more payload and will result in more overhead. To achieve the trade-off between the payload size and power saving gain for a UE, the number of subgroups can be carried in LP-WUS is 8, same as PEI.

Proposal 5: The number of subgroups can be carried in LP-WUS is 8.

In last RAN1 meeting [2], they discuss the association relation between PO and LO. The association relationship between PO and LO is equivalent to the relationship between the subgroup of PEI and LP-WUS in RAN2. According to RAN1 agreements, the subgroup of LP-WUS can be same as the PEI, or the subgroup of PEI can be a subset of LP-WUS subgroup and vice versa. Cases with or without intersection between PEI and LP-WUS subgroup are excluded.

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: • Option 1: UEs monitoring the same PO monitor the same LO • Option 2: UEs corresponding to different POs monitor the same LO • Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Combinations of the above options can be considered.

In case LP-WUS and PEI are used together for one UE, it is expected that UE could achieve more power saving gain via two levels of subgroup for both LP-WUS and PEI. UE first monitors LP-WUS, then monitors PEI. The subgroup of PEI should be a subset of LP-WUS subgroup, or the subgroup of PEI and the subgroup of LP-WUS have an intersection, as shown in figure 1a and 1b. In this way, it is expected that some UEs will be filtered out to further monitor PO.

However, the latter way has been excluded by RAN1. The subgroup of PEI is a subset of LP-WUS subgroup, it is easy to implementation. The formula of PEI subgroup can be reused, just make the subgroupsNum (bitmap) shorter.

1a

1b

Figure 1 relationship of LP-WUS subgroup and PEI subgroup

Proposal 6: For monitoring PEI after monitoring LP-WUS, the subgroup of PEI should be a subset of LP-WUS subgroup.

3 Conclusion

In this contribution we discuss the LP-WUS operation in IDLE/INACTIVE modes, with the following proposals:

Proposal 1: The dedicated LP-WUS related configuration for IDLE/INACTIVE state is not needed.

Proposal 2: The detailed LP-SS and LP-WUS configurations need to wait for RAN1 progress.

Proposal 3: The serving cell measurement based on LR as entry condition for LP-WUS monitoring is not needed, if MR has been measuring the serving cell before LP-WUS monitoring.

Proposal 4: The serving cell measurement based on LR as entry condition for LP-WUS monitoring is needed, if the serving cell measurement has offloaded form MR to LR before LP-WUS monitoring.

Proposal 5: The number of subgroups can be carried in LP-WUS is 8.

Proposal 6: For monitoring PEI after monitoring LP-WUS, the subgroup of PEI should be a subset of LP-WUS subgroup.

- References

- Draft report of 3GPP TSG RAN WG2 #126, RAN2 chair (interDigtal), Fukuoka, Japan, May 20th – 24th, 2024.

- Draft report of 3GPP TSG RAN WG1 #117, RAN1 chair’s notes, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.