3GPP TSG-RAN WG2 Meeting #127	R2-2406754

Maastricht, Holland, Aug 19th – 23rd, 2024

Agenda item:	8.4.3

Source:	Spreadtrum Communications

Title:	Discussion on RRM measurement relaxation and offloading in IDLE/INACTIVE mode

Document for:	Discussion and Decision

1 Introduction

In RAN2#126 meeting, RAN2 discussed the RRM measurement relaxation and offloading in RRC_IDLE INACTIVE mode and achieved the following agreements [1]:

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution, we will discuss the RRM measurement relaxation and offloading of RRC_IDLE/INACTIVE UE MR for serving and neighbor cell.

2 Discussion

## 2.1 RRM measurement relaxation for serving cell

In addition to paging monitoring, RRM measurement is another idle/inactive mode procedure that consumes a significant amount of UE power. In IDLE/INACTIVE mode, UE needs to perform serving cell measurement and neighbor cell measurement. If UE still have to wake up and switch on MR frequently to perform serving/neighboring cell RRM measurement, the power saving effect of the LP-WUS mechanism may be highly reduced.

In SI phase, RAN2 has studied the RRM measurement relaxation for serving/neighboring cell and made the follows conclusions [2].

RAN2 SI phase agreements about RRM measurement relaxation: - RAN2 assumes in ultra-deep-sleep, RRM measurement on serving cell via MR is relaxed (may include no measurement) if RRM measurement on LR is feasible/supported. FFS on the details, e.g. how to relax, in which condition. - RAN2 assumes in ultra-deep-sleep, RRM measurement on neighboring cell via MR is relaxed (may include no measurement) if RRM measurement on LR is feasible/supported. FFS on the details, e.g. how to relax, in which condition. - RAN2 has studied and concluded the feasibility for RRM measurement relaxation (including no measurement) for serving cell by MR and neighboring cell by MR at least if RRM measurement on LR for serving cell is feasible/supported. Details are to be decided in WI.

From RAN2’s WI perspective, we need to consider the entering and leaving condition for using LR for serving/neighboring cell RRM measurement.

In R16, the relaxing neighboring cell RRM measurement for intra-frequency or inter-frequency/inter-RAT frequency is allowed for UEs not at cell edge and/or with low mobility. However, it is noted that no relaxation is so far allowed for the serving cell RRM measurement. It is up to RAN4 to decide how RRM measurement on serving cell via MR is relaxed (may include no measurement), RAN2 is expected to consider the new criteria for relaxing serving cell RRM measurement.

For serving cell measurement offloading, in last RAN2 meeting, it agreed that serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required. RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. We think the offloading criterion is when signal quality (e.g. RSRP/RSRQ) of serving cell is greater than a threshold. Or when UE starts LP-WUS monitoring, the serving cell RRM measurement is offloaded from MR to LR.

Proposal 1: UE serving cell RRM measurement can be offloaded from MR to LR with the follows condition:

- Option 1: when signal quality of serving cell measured by MR is greater than a threshold.

- Option 2: when entry condition for monitoring LP-WUS is satisfied.

## 2.2 RRM measurement relaxation for neighboring cell

In R16, the neighboring cell RRM measurement relaxation for intra-frequency or inter-frequency/inter-RAT frequency is allowed for UEs not at cell edge and/or with low mobility, detailed measurement relaxation rules are shown in the figure 1.

Figure 1 the legacy RRM measurement relaxation for neighboring cell

In general, the neighbour cell measurements only need to be performed by the UE if the serving cell is not strong enough, i.e., when the intra-frequency cell re-selection criterion (Srxlev > SIntraSearchP and Squal > SIntraSearchQ) or the inter-frequency cell re-selection criterion (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) is not fulfilled. However, the LP-WUS may be deactivated before the neighbour cell measurements since there will be some neighboring cells areas which are not covered by LP-WUS, as shown in figure 2.

Figure 2 the LP-WUS coverage

In last RAN2 meeting, it has agreed that the RRM measurement of the neighboring cell can only be performed by MR, so the neighbor cell measurement relaxation mechanism in Release 16 can be reused. The current neighbor cell measurement relaxation can achieve UE energy saving, RAN2 considers whether further need to relax neighbor cell RRM measurement for LP-WUS MR. And specify the criteria for relaxing neighbor cell RRM measurement if necessary.

In our view, to simplify the complexity of the design LP-WUS, it at least can reuse the neighboring cell RRM measurement relaxation of R16 for LP-WUS MR, which includes two scenarios: not at cell edge and low mobility, respectively.

When the intra-frequency cell re-selection criterion (Srxlev > SIntraSearchP and Squal > SIntraSearchQ) or the inter-frequency cell re-selection criterion (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) is fulfilled, and if low mobility is also fulfilled, UE does not measure intra-frequency, lower priority frequency and higher priority frequency. Otherwise, UE will measure higher priority frequency.

When SSearchThresholdP < Srxlev < SIntraSearchP/nonIntraSearchP or SSearchThresholdQ < Squal < SIntraSearchQ/nonIntraSearchQ, and if low mobility is fulfilled, UE does not measure intra-frequency, lower priority frequency and higher priority frequency. Otherwise, UE will perform the measurement relaxation, i.e. measure intra-frequency every 2nd DRX, measure lower priority frequency every 2nd DRX and higher priority frequency every minute.

When Srxlev < SSearchThresholdP or Squal < SSearchThresholdQ, and if low mobility is fulfilled, UE will perform the measurement relaxation, i.e. measure intra-frequency every 4th DRX and lower priority frequency every 4th DRX, and does not measure higher priority frequency. Otherwise, UE will perform the measurement criterion of R15, i.e. measure intra-frequency every DRX, measure lower priority frequency every DRX and higher priority frequency every minute.

Proposal 2: RAN2 reuse the neighboring cell RRM measurement relaxation of R16 for LP-WUS MR.

3 Conclusion

In this contribution we discuss the RRM measurement relaxation and offloading for idle/inactive UE, with the following proposals:

Proposal 1: UE serving cell RRM measurement can be offloaded from MR to LR with the follows conditions:

- Option 1: when signal quality of serving cell is greater than a threshold.

- Option 2: when entry condition of using LP-WUS is satisfied.

Proposal 2: RAN2 reuse the neighboring cell RRM measurement relaxation of R16 for LP-WUS MR.

- References

- Draft report of 3GPP TSG RAN WG2 #126, RAN2 chair (interDigtal), Fukuoka, Japan, May 20th – 24th, 2024.

- R2-2400002, “Report of 3GPP TSG RAN WG2 meeting #124, Chicago, USA”, ETSI MCC, RAN2#124, 13 - 17 November, 2023.