3GPP TSG-RAN WG2 Meeting #127	R2-2406755

Maastricht, Holland, Aug 19th – 23rd, 2024

Agenda item:	8.13.3

Source:	Spreadtrum Communications

Title:	Discussion on QoS handling for NR sidelink multi-hop relay

Document for:	Discussion and Decision

1 Introduction

It was agreed in WID [1] to specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay firstly, including the impact on QoS handling for multi-hop.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

In this contribution, we will discuss the QoS handling to support multi-hop L2 U2N relay.

2 Discussion

## 2.1 QoS split

In Rel-17 single-hop L2 U2N relay, it is gNB implementation to handle the QoS split over Uu and PC5 for the end-to-end QoS enforcement of a particular session established between Remote UE and network.

Specifically, in remote UE service establishment procedure, CN provides end-to-end QoS requirements of remote UE’s service to gNB. The end-to-end QoS requirements can be understood as QoS parameters between gNB and remote UE, which should be satisfied by Uu link and PC5 link in U2N relay. From the perspective of gNB, it knows the Uu link condition and the PC5 link condition through the measurement mechanisms, so it is able to know how to satisfy the end-to-end QoS requirement by the two links. Based on this, gNB may divide the end-to-end QoS into per hop-by-hop QoS by implementation and make sure each hop-by-hop QoS to be satisfied by Uu link and PC5 link.

In Rel-19 multi-hop L2 U2N relay, the work starts with one additional hop relay firstly. Compared to a single-hop L2 U2N relay, the difference is that there are two relay UEs between gNB and remote UE. The QoS split mechanism can still apply to multi-hop U2N relay.

Proposal 1: For remote UE end-to-end QoS split in multi-hop L2 U2N relay, the mechanism in single-hop L2 U2N relay is as baseline, i.e. gNB implementation can handle the end-to-end QoS split over multi-hop.

## 2.2 QoS configuration

Once QoS split is done by gNB, it is necessary to convey related QoS configurations to remote UE and/or relay UE. As in single-hop L2 U2N relay, gNB directly configures remote UE and first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling. Moreover, QoS configuration for remote UE and relay UE (first and last) on PC5 hop are configured per PC5 RLC bearer.

Proposal 2: In multi-hop L2 U2N relay, gNB directly configures the remote UE, first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling.

Proposal 3: QoS configuration is configured per PC5 RLC bearer.

## 2.3 Measurements report for QoS

To help gNB to perform QoS split or adjust QoS split, UE(s) need to report measurements or other assistance information to the gNB. The existing SL measurement report and CBR measurement reports can be used by gNB to understand PC5 link conditions and determine QoS configuration.

Proposal 4: Remote UE and first relay UE report SL measurement and CBR measurement to gNB for QoS configuration determination.

## 2.4 QoS flow control

One of the objective in the WID is to specific mechanism supporting QoS handling for multi-hop relaying. However, QoS is likely to be corrupted when network congestion occurs. Packet loss in that case causes TCP to slow down. Thus, some kind of flow control mechanism is required to avoid too packets to be pushed to the overloaded node. Such as, relay UE (first or last) can transmit flow control indication to remote UE for controlling traffic flow when relaying in UL, or relay UE (first or last) can transmit flow control indication to gNB for controlling traffic flow when relaying in DL.

Proposal 5: RAN2 to study flow control mechanism for multi-hop L2 U2N relaying.

## 2.5 Latency reduction

Another issue regarding QoS handling is the latency caused by multi-hop architecture. There is some concern about the overall latency of the QoS related procedures in control plane.

In IAB, the pre-emptive BSR is used by an IAB-MT to provide expected volume of data to its scheduler. In multi-hop L2 U2N relaying, similar mechanism can be introduced. The pre-emptive BSR might help to reduce UL scheduling latency by data buffering at the relay UE. First relay UE receives the data from remote UE and then sends to last relay UE. Last relay UE will triggers pre-emptive BSR.

Observation 1: Pre-emptive BSR is beneficial for multi-hop relay to reduce latency.

Proposal 6: Support pre-emptive BSR in multi-hop L2 U2N relaying.

3 Conclusion

In this contribution we discuss the QoS handling for multi-hop relay, with the following proposals:

Proposal 1: For remote UE end-to-end QoS split in multi-hop L2 U2N relay, the mechanism in single-hop L2 U2N relay is as baseline, i.e. gNB implementation can handle the end-to-end QoS split over multi-hop.

Proposal 2: In multi-hop L2 U2N relay, gNB directly configures the remote UE, first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling.

Proposal 3: QoS configuration is configured per PC5 RLC bearer.

Proposal 4: Remote UE and first relay UE report SL measurement and CBR measurement to gNB for QoS configuration determination.

Proposal 5: RAN2 to study flow control mechanism for multi-hop L2 U2N relaying.

Observation 1: Pre-emptive BSR is beneficial for multi-hop relay to reduce latency.

Proposal 6: Support pre-emptive BSR in multi-hop L2 U2N relaying.

- References

- RP-241609, “New WID: NR sidelink multi-hop relay”, LG Electronics, InterDigital, FirstNet, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June, 2024.