3GPP TSG RAN WG2 #127	   													                        R2-2406764

Maastricht, Netherlands, August 19th –23th, 2024

Agenda item:	8.2.4

Source:	ETRI

Title:	Further discussions on A-IoT random access

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN2 discussed the A-IoT random access and reached the following agreements.[1].

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

This contribution discusses additional considerations based on the 4-step random access procedure agreed upon in the previous meeting.

# Discussions

## RA configurations

In A-IoT, the inventory and command services are initiated based on network requirements, necessitating appropriate radio resource configuration. Radio resources in A-IoT can be divided into those for the R2D (Reader-to-Device) link, which is used for transmitting control information and data from the reader to the device, and those for the D2R (Device-to-Reader) link, which is used for transmitting data from the device to the reader. These services can be started upon requests from the A-IoT controller or AF (Application Function), with each series of service execution units being defined as a transmission set. For example, a service that detects the IDs of nearby devices and reads the status information of a specific device can be organized into inventory and command services, forming a single transmission set.

Proposal 1: The transmission set is a radio resource block used for data transmission between the reader and the device to provide a series of services.

Observation 1: The reader can configure a transmission set and related radio resources based on the requirements from the higher layers.

A single transmission set can consist of radio resources for R2D and(or) D2R transmission. R2D resources may include those for transmitting configuration information within the transmission set as well as those for data transmission from the reader to the device. In A-IoT, devices often cannot receive periodic system information such as NR’s SIB due to battery constraints and low capability. Therefore, using configuration information in transmission sets allows for the resource configuration for R2D or D2R transmission and the transmission of necessary information to the devices.

In contrast, D2R resources can include those for random access and resources for data transmission from the device to the reader. The resources for random access allow multiple devices to transmit their IDs to the reader. To achieve this, devices utilize the configuration information from R2D (i.e., Config. in Figure 1) to acquire the setup details for the radio resources, including the configuration and operational parameters for random access.

Within a single transmission set, multiple devices can perform random access, requiring the allocation of multiple RA resources, defined as access occasions. Access occasions can be classified into shared access occasions and dedicated access occasions. In the case of CBRA (Contention-Based Random Access), devices select one of the shared access occasions to transmit data. Conversely, in CFRA (Contention-Free Random Access), devices use dedicated access occasions

Proposal 2: RA resources can be composed of multiple access occasions.

Proposal 3: Access occasions are classified as shared access occasions for CBRA and dedicated access occasions for CFRA.

The Figure 1 illustrates an example of the A-IoT resource configuration described earlier.

Figure 1. Example of the A-IoT resource configuration

Based on the previously described resource configuration and operational procedures, the device requires the following configuration information for random access.

Proposal 4: RAN2 is asked to discuss the below RA configuration information.

- RA resource configuration

  - radio resource(s) configuration for CBRA and CFRA

  - radio resource(s) configuration of access occasions

  - radio resource(s) configuration of Msg2 (i.e. response message of message1 in RA)

- Device behavior configuration

  - waiting time (or waiting access occasions) for Msg2 reception after Msg1 transmission;

  - window size for selecting access occasions;

  - maximum number of retransmission attempts;

  - transmission power ramping configuration, if needed

The resource configuration for random access may be adaptable, taking into account factors such as device density and network load. Therefore, the reader needs to transmit this configuration information to the devices through message-based procedures, which needs to be discussed by RAN2.

Proposal 5: The RA configuration is delivered via explicit signaling before the RA procedure.

## Device ID of Msg1 for 4-step RA procedure

The device ID included in Msg1 of the RA procedure requires further discussion on the following two issues.

- Issue#1: Device ID generation mechanism

The purpose of Msg1 in the random access procedure is to inform the network of the device’s presence, and for this, it is crucial that each device generates and transmits a unique ID to the reader. There are two approaches to the ID used in Msg1: using a random number or using the device ID.

- Random number based approach: In this method, the device selects a random value within a range specified by the reader and uses it as a temporary ID. The device requires the maximum value of the window for random number selection, which can either be received from the reader or pre-configured.

Observation 2: When using a random number as the device ID, it’s necessary to define a maximum window size for the selection.

- Device ID based approach: This method uses a pre-established device ID set by higher layers. It can involve using the entire device ID or a part of it. SA2 has discussed device identifiers and categorized them into permanent IDs and temporary IDs. It was concluded that a long temporary ID is suitable between the controller and the reader, while a short temporary ID is suitable between the device and the reader. Therefore, even when generating an ID for Msg1 using the higher layer’s device ID, it is more efficient in terms of resource consumption and security to generate and transmit a shorter temporary ID rather than using the full-length device ID. Additionally, if generating a temporary ID, there should be a configuration on how to select this from the entire device ID.

Observation 3: When using a device ID, it is necessary to generate a temporary ID based on a portion of the device ID.

Using a random number and using part of the device ID are similar methods. However, as discussed by SA2[3], since temporary IDs are assigned by the CN (Core Network) through a registration process, the device ID used in Msg1 of the RA procedure should be generated from the permanent ID. The permanent ID, which may consist of the MNO ID, Owner ID, and Instance ID, increases the probability of generating identical IDs compared to using a random number. Therefore, generating the device ID for Msg1 using a random number has significant advantages over using part of the device ID.

Proposal 6: For a 4-step RA, the device ID in Msg1 should be generated using a random number.

- Issue#2: Device ID length

Based on the requirements from SA1 use cases, the inventory service should consider a device density of up to 1.5 million devices as shown Figure 2 However, this figure represents the maximum number of devices per square kilometer in an indoor environment, not the number of devices a single reader must handle. For instance, if a single reader performs an inventory service in an area of 100 meters by 100 meters or 200 meters by 200 meters, it should consider the simultaneous access of up to 1,000 or 60,000 devices, respectively. However, since the maximum connection density is described as 150 devices per 100m² for indoor scenarios in TR 38.848, we believe that 16 bits are sufficient to accommodate this requirement. Therefore, considering the reader’s service area and the device’s connection density, the device ID in Msg1 is sufficient at 16 bits.

Proposal 7: In a 4-step RA, a 16-bit device ID in Msg1 is sufficient.

Table 5.16.6-1: Potential key performance requirements for the use of Ambient IoT devices for automated supply chain distribution

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT devices for automated supply distribution | >10 s | 99% | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km2 | 30m indoor (note 1), 400m outdoor | 600 000 m2 | NA | NA | NA | NA | [3] m (Indoor, 90% confidence level and in horizontal) , cell-level outdoor |
| NOTE 1: The storage area of a private warehouse is typically around 10000 square feet, which is smaller than that of a typical public warehouse. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 2. Example of performance requirements for the use of Ambient IoT devices for automated supply chain distribution (3GPP TR22.840[2])

## Additional information of Msg2

The contents in Msg2 of the RA procedure can encompass not only the device ID received in Msg1 but also additional information such as the scheduling and security configuration for Msg3 transmission. For A-IoT, the scheduling may be a new, simplified approach tailored to the device’s capabilities, rather than the traditional DCI-based scheduling used in NR. For example, resources allocated with fixed sizes in the frequency and time domains could be structured as x and y coordinates in a table format or as a linear list, using a bitmap to map these to device IDs. However, since scheduling mechanism is closely related to the radio resource allocation and transmission schemes considered by RAN1, RAN2 should consider RAN1 discussion. Considering the security aspect, since Msg3 includes the device ID, it can also contain encryption configurations needed for secure transmission over air interface between the device and the reader. Therefore, RAN2 should also consider the discussions from SA2 and SA3.

Observation 4: Scheduling and security configurations can be included as additional content in Msg2.

Proposal 8: When considering scheduling and security configuration as additional information in Msg2, the work from RAN1/SA2/SA3 should be taken into account.

## Contention resolution

In the last RAN2#126 meeting, there was a discussion on whether contention resolution is completed through the reception of Msg2, which includes the device ID transmitted in Msg1 during the RA procedure. However, if two or more devices simultaneously transmit Msg1 with the same device ID using the same radio resources, and the reader decodes this message without errors and echoes it using Msg2, the reception of Msg2 on devices cannot be considered as contention resolution. While using a sufficiently long random number as the device ID can reduce the contention probability, it also has the drawback of increased radio resource. Therefore, further discussion on this issue is necessary.

Proposal 9: RAN2 is requested to have further discussions on contention resolution.

## Usage of Msg4 in 4-step CBRA

In the last RAN2 meeting, it was agreed that if the reader does not receive Msg3 due to various during the 4-step RA procedure, Msg4 would be used. For example, during the RA procedure, if the device runs out of energy available for communication or has low transmission power, Msg3 may not be transmitted.

In this case, the reader can transmit Msg2 and, if Msg3 is not received after a certain period, it can infer a transmission error has occurred. To address this, Msg4 could be used as a message instructing the retransmission of Msg3. However, since the reader cannot know whether it received Msg2 on the device or if Msg3 was transmitted, it is difficult to use Msg4 to handle the transmission failure of Msg3. Therefore, further discussion on this issue is necessary.

Observation 5: Further discussion is needed on using Msg4 to handle the transmission failure of Msg3.

As agreed in the previous RAN2 meeting, Msg4 is not part of the 4-step RA procedure and is not always transmitted. However, Msg4 can be used as an optional procedure in A-IoT service scenarios when necessary. For example, if command services such as read, write, or disable are needed after inventory, the reader can optionally use Msg4 to facilitate these procedures.

Proposal 10: Msg4 can be optionally used for additional services depending on higher layer requirements.

## Access failure handling

After transmitting the initial Msg1, if the device does not receive Msg2 within a certain period or if the received Msg2 does not contain its device ID, the device recognizes that contention has occurred and initiates the re-access procedure. For re-access, the device selects a new access occasion and retransmits Msg1 with backoff mechanism after determining that the initial Msg1 transmission failed. The backoff mechanism for Msg1 retransmission can involve increasing the window size for access occasion selection to reduce the collision probability. Therefore, the access occasion for Msg1 retransmission could be another access occasion within the current transmission set or one within a new transmission set. To support this, the device needs configurations such as the waiting time for Msg2 reception after Msg1 transmission, the window for selecting access occasions, and the maximum number of retransmission attempts. Additionally, depending on the device’s capability, it may selectively increase its transmission power during Msg1 retransmission, which also requires appropriate configuration.

Proposal 11: The device needs the following configuration information for re-access

- waiting time (or waiting access occasions) for Msg2 reception after Msg1 transmission;

- window size for re-selecting access occasions;

- maximum number of retransmission attempts;

- transmission power ramping configuration, if needed

If the reader does not receive Msg3 after sending Msg2, the reader can retransmit Msg2 to request a retransmission of Msg3. Therefore, if the device receives Msg2 from the reader after sending Msg3, it should interpret this as a retransmission request and resend Msg3.

Proposal 12: If the device receives Msg2 from the reader after sending Msg3, it retransmits Msg3.

# Conclusions

Based on the discussion in the contribution, we have the following observation and proposals:

RA Configurations

Proposal 1: The transmission set is a radio resource block used for data transmission between the reader and the device to provide a series of services.

Observation 1: The reader can configure a transmission set and related radio resources based on the requirements from the higher layers.

Proposal 2: RA resources can be composed of multiple access occasions.

Proposal 3: Access occasions are classified as shared access occasions for CBRA and dedicated access occasions for CFRA.

Proposal 4: RAN2 is asked to discuss the below RA configuration information.

- RA resource configuration

  - radio resource(s) configuration for CBRA and CFRA

  - radio resource(s) configuration of access occasions

  - radio resource(s) configuration of Msg2 (i.e. response message of message1 in RA)

- Device behavior configuration

  - waiting time (or waiting access occasions) for Msg2 reception after Msg1 transmission;

  - window size for selecting access occasions;

  - maximum number of retransmission attempts;

  - transmission power ramping configuration, if needed

Proposal 5: The RA configuration is delivered via explicit signaling before the RA procedure.

Device ID of Msg1 for 4-step RA procedure

Observation 2: When using a random number as the device ID, it’s necessary to define a maximum window size for the selection.

Observation 3: When using a device ID, it is necessary to generate a temporary ID based on a portion of the device ID.

Proposal 6: For a 4-step RA, the device ID in Msg1 should be generated using a random number.

Proposal 7: In a 4-step RA, a 16-bit device ID in Msg1 is sufficient.

Additional information of Msg2

Observation 4: Scheduling and security configurations can be included as additional content in Msg2.

Proposal 8: When considering scheduling and security configuration as additional information in Msg2, the work from RAN1/SA2/SA3 should be taken into account.

Contention resolution

Proposal 9: RAN2 is requested to have further discussions on contention resolution.

Usage of Msg4 in 4-step CBRA

Observation 5: Further discussion is needed on using Msg4 to handle the transmission failure of Msg3.

Proposal 10: Msg4 can be optionally used for additional services depending on higher layer requirements.

Access failure handling

Proposal 11: The device needs the following configuration information for re-access

- waiting time (or waiting access occasions) for Msg2 reception after Msg1 transmission;

- window size for re-selecting access occasions;

- maximum number of retransmission attempts;

- transmission power ramping configuration, if needed

Proposal 12: If the device receives Msg2 from the reader after sending Msg3, it retransmits Msg3.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #126”, May 2024.

- 3GPP TR 22.840, Study on Ambient power-enabled Internet of Things (Release 19),  Dec. 2023.

- 3GPP TR 23.700, Study on Architecture support of Ambient power-enabled Internet of Things, May, 2024