3GPP TSG-RAN WG2 Meeting #127	R2- 2406775

Maastricht , NL, August 19 - August 23, 2024

Agenda Item:	8.6.2

Source:	Lekha Wireless Solutions

Title:	Inter CU LTM Discussion

Document for:	Discussion, Decision

Introduction

At last meeting, we discussed inter-CU LTM and made the following agreements [1]:

- Agreements on inter-CU LTM: - Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. - Xn-based and N2-based inter-CU LTM - Xn-based inter-CU LTM is prioritized in Rel-19. - Stage-2 signaling flows and procedures - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). - Preparation: - RRC Configuration/structure - The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. - LTM Candidate ID - For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. - Max number of LTM candidate IDs - The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations. - Early sync: - RAR based option - RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM. - Execution: - LTM Cell Switch Command - R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. - LTM Cell switch completion - Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. - The LTM completion defined for intra-CU LTM is followed for R19 LTM. - FFS:Support of mixture of inter-CU and intra-CU LTMs

This contribution discussed different aspect to support inter-CU mobility design.

Discussion on Inter CU LTM

RAR based TA for Conditional LTM

- RAR (Random Access Response) based TA (Timing Advance) is used during the initial access or re-connection phase to establish synchronization with the network. It is delivered as part of the Random Access Response message in the Random Access Procedure providing initial timing adjustments needed for the UE to properly connect to the network. The UE initiates the Random Access procedure, and after a short delay, the network sends the RAR.

- Observation: PDCCH order based TA is preferred over the RAR based TA for conditional and non-conditional inter-CU LTM

- The RAR-based TA involves additional steps compared to the PDCCH order, including the time required for the UE to perform the random access and receive the RAR. This process usually results in a latency of around 2 to 3 subframes, which is about 20-30 milliseconds. This can be slightly longer due to the additional random access procedure steps. PDCCH order-based Timing Advance is generally faster and more direct compared to RAR-based Timing Advance, which involves a more complex process including initial access and response steps. PDCCH ordered TA is primarily applicable in scenarios requiring quick adjustments to uplink timings in response to real-time conditions, promoting flexibility and efficiency in managing ongoing connections. The PDCCH order is scheduled on the PDCCH (Physical Downlink Control Channel), which is sent on a fixed schedule every 1 ms (subframe).  The latency for this process is typically around 1 to 2 subframes, considering the scheduling and processing delays, which translates to about 10-20 milliseconds (ms).  Hence, for both conditional LTM (movement adaptive network) and non-conditional LTM (predefined schedules), PDCCH order TA shall be sufficient.

- Proposal 1: PDCCH order based TA is preferred over the RAR based TA for conditional and non-conditional inter-CU LTM scenarios.

Support for a mixture of Intra CU and Inter CU LTM

- Due to mobility, UE establishes the connection with new gNB through Intra or Inter CU mechanism. The source CU prepares the LTM candidate configuration through various interaction with other CU’s and shared it to UE through corresponding serving DU. In the last RAN meeting, the maximum candidate cell configuration is limited to 8.

- In order to support the mixture of intra and inter CU LTM candidate configuration, the following issues needs to be discussed.

Observations:

- Introduce a new RRC-configured Release 19 ID: If the Release 19 ID differs between the source cell and the target cell, the UE will carry out PDCP re-establishment, which includes updating the security key.

- In Rel-18 LTM [2], an IE ltm-NoResetID is used to determine whether to perform RLC re-establishment for inter-DU LTM, and UE uses this ID to compare with the ID in current serving cell. For inter-CU LTM, a similar indication can be introduced for inter-CU cases which indicates the need for PDCP reestablishment and security update.

- From our understanding, this indication could be enough for UE to perform inter-CU LTM cell switch.

- Proposal 2 : Introduce a new RRC-configured Release 19 ID: If the Release 19 ID differs between the source cell and the target cell, the UE will carry out PDCP re-establishment, which includes updating the security key.

- Single Reference Configurations or Multiple reference configurations:

- In Release-18, a single reference configuration can be set up for intra-CU LTM. This reference configuration is created by the CU, using details provided by the DUs associated with LTM candidate cells. Since the full configuration of a candidate cell is produced by layering the candidate cell's configuration on top of the reference configuration, the candidate cell configuration itself doesn't need to include the information already present in the reference configuration. This suggests that generating the reference configuration might require a complex process, potentially involving multiple exchanges between the CU and DUs to optimize both the reference and candidate cell configurations.

- For a unified LTM-CSI-ResourceConfig-List structure to function, the source CU needs to prepare a list of CSI/L1 resource configurations, similar to how it's done in intra-CU LTM in Release-18. To achieve this, the source CU should provide a "template" or reference configuration that candidate CUs can use to generate a delta configuration. This approach is not entirely new; in traditional L3 handover, the source CU would supply the current UE configuration, and the target CU would provide a delta configuration based on that. The difference here is that there are multiple candidates, and the configuration would likely be applied at the time of the LTM switch rather than immediately.

- Figure 1: Consolidated Reference Configuration to support Intra CU and Inter-CU LTM

- For the intra-CU cell switch for the inter-CU LTM (i.e., cell switch between Cell A, B and C), the reference configuration is still valid and delta config can be applied for cell B and C for signalling overhead reduction.

- For the intra-CU cell switch for the inter-CU LTM (i.e., cell switch between cell D and others), the full config will be provided and the subsequent LTM can also be supported.

- Proposal 3 : Single Reference Configuration with additional delta config for Intra CU LTM and Full configuration for Inter CU LTM if required, in order to support the mixture of Intra and Inter CU LTM candidate configuration.

2.3 Support Procedure for mixture of Intra and Inter CU LTM without DC

- Observation: Considering Rel 18 Intra CU LTM as a baseline, the signalling procedure to support Intra CU and Inter CU LTM is shown below. The Serving CU sends the reference configuration to candidate CU’s. With the reference configuration, each candidate CU follows Intra CU LTM procedure to prepare candidate configuration. Each candidate CU sends the prepared candidate configuration to serving CU. The serving CU consolidate and shared it to UE as LTM candidate configuration as well as to candidate CU’s for subsequent LTM.

Figure 2: Signalling procedure to support Intra CU and Inter-CU LTM

Proposal 4: Signalling procedure to support the mixture of Intra CU and Inter CU without Dual connectivity is depicted. The serving CU is responsible to send Reference configuration to candidate CU’s and it consolidates, then send it to UE and candidate CU’s for RRC Reconfiguration and subsequent LTM procedures respectively.

Impacts of PDCP in NR-DC for inter-CU LTM in SN

Observation:

Case 1: For MCG bearer, MN contains NR PDCP. SN enables dual connectivity to load balance the MN/MCG [3]. Due to inter CU mobility in SN, there is no impact in PDCP. MN maintains the PDCP control plane.

Case 2: For SCG bearer, SN contains NR PDCP. MN enables dual connectivity to load balance the SN/SCG [3]. Due to inter CU mobility in SN, the following impacts can happen.

- PDCP reestablishment is needed.

- Security key needs to update.

- L2 reset is required at UE.

- Case 3: For Split bearer, MN and SN contains NR PDCP. The data will be shared among MN and SN to increase the UE’s throughput. Due to inter CU mobility in SN, the following impacts can happen.

- NR PDCP user plane should change in the SN.

- Security key will be different for MN and SN.

- UE should use different security key to decode the received information from MN and SN. SA3 should clarify this. From RAN2 perspective, It is difficult to support Split bearer with two security keys and PDCP reestablishment should happen while decoding SN transmitted information. It increases the signaling overhead. If there is a unique security key across CU’s, then the impact of split bearer might be reduced.

Proposal 5: Supporting Split bearer based NR-DC in Inter CU LTM is more complicated due to two different security key used in MN and SN. It is preferable to consider MCG and SCG bearer for NR-DC Inter CU LTM.

Proposal 6: The impact of PDCP in SN/SCG bearer for Inter CU LTM needs PDCP reestablishment, MAC Reset and security key update. Based on SA3 concerns on security key, RAN2 needs to investigate further.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: PDCCH order based TA is preferred over the RAR based TA for conditional and non-conditional inter-CU LTM scenarios.

Proposal 2 : Introduce a new RRC-configured Release 19 ID: If the Release 19 ID differs between the source cell and the target cell, the UE will carry out PDCP re-establishment, which includes updating the security key.

Proposal 3 : Single Reference Configuration with additional delta config for Intra CU LTM and Full configuration for Inter CU LTM if required, in order to support the mixture of Intra and Inter CU LTM candidate configuration.

Proposal 4: Signalling procedure to support the mixture of Intra CU and Inter CU without Dual connectivity is depicted. The serving CU is responsible to send Reference configuration to candidate CU’s and it consolidates, then send it to UE and candidate CU’s for RRC Reconfiguration and subsequent LTM procedures respectively.

Proposal 5: Supporting Split bearer based NR-DC in Inter CU LTM is more complicated due to two different security keys used in MN and SN. It is preferable to consider MCG and SCG bearer for NR-DC Inter CU LTM.

Proposal 6: The impact of PDCP in SN/SCG bearer for Inter CU LTM needs PDCP reestablishment, MAC Reset and security key update. Based on SA3 concerns on security key, RAN2 needs to investigate further.

References

- R2-2405701 “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung).

- 38.300, NR; NR and NG-RAN Overall Description; Stage 2, 18.1.0

- 38.323, NR; Packet Data Convergence Protocol (PDCP) specification, 18.2.0