3GPP TSG-RAN WG2 Meeting #127		R2-2406777

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on PDCP window update based on the SN gap report

Agenda Item:	7.5.3

Document for:	Discussion and Decision

1	Introduction

In RAN2#126 meeting [1], RAN2 has discussed the RX_NEXT update for PDCP SN report [2], and have no agreement on this issue:

R2-2404287	PDCP SN gap report and updating RX_NEX	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

=>	Noted

This contribution will continue to discuss this issue and provide our views.

2 Discussion

The issue proposed in [2] is shown in the following figure:

Figure.1 PDCP Receive Window Update based on SN Gap report

As shown in the above figure, in case that pdu-SetDiscard is configured and PSI-based SDU discard is activated, transmitting PDCP entity will trigger SN gap report when the discardTimerForLowImportance expires of packet #1. The SN gap report can include FDC=0 and bitmap=010 to inform the receiver of discarded packet #1 and packet #3. Then, receiving PDCP entity will update RX_NEXT=4, RX_REORD=4 and start t-Reordering timer based on the last TS 38.323 below. As pointed in contribution [2], when t-Reordering expires, meaning that unreceived packet #0 and packet #2 will never be delivered to upper layer even if still received after this point.

Observation 1: The contribution [2] pointed that If RX_NEXT is updated past all SDUs indicated as discarded, the receiving PDCP entity may prematurely update RX_DELIV past unreceived high-importance SDUs when pdu-SetDiscard is configured and PSI-based SDU discard is activated.

TS 38.323 At reception of a PDCP SN gap report from lower layers, the receiving PDCP entity shall: - consider each PDCP SDU, if any, with the bit in the discard bitmap set to '1', or with the associated COUNT value equal to the value of FDC field as discarded; - if RX_DELIV is less than or equal to the largest COUNT value associated with the discarded PDCP SDUs: - if RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report: - update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1; - if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s): - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before: - all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded, with COUNT value > RX_DELIV; - if t-Reordering is running, and if RX_DELIV >= RX_REORD: - stop and reset t-Reordering; - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering.

From our perspective, the issue pointed by the contribution will not happen. The reasons are as following:

Firstly, it is a rare case that transmitting PDCP entity will associated interlaced SN/COUNT value to PDU set with different importance.	 Due to the high data rate of XR service, huge amounts of packets will arrival AS layer in short time. Then, UE implementation can guarantee that PDCP SDUs belongs to the same PDU set will be assigned with continuous SN/COUNT values.

Observation 2: UE implementation can guarantee in a certain degree that PDCP SDUs belongs to the same PDU set will be assigned with continuous SN/COUTN values, which makes the problem pointed by the contribution [2] not exist.

Due to PDCP pre-processing, PDCP SDU will be pre-processed and delivered to RLC layer. The transmission of PDCP SN gap report will be done by RLC layer later than RLC packet which has been buffered in RLC before PDCP SN gap report trigger. For example, in case that transmitting PDCP entity will associate the interlaced SN/COUNT values to PDU set with different importance, PDCP will deliver the packet #0,1,2,3 to RLC layer based on PDCP pre-processing. When the discardTimerForLowImportance expires of packet #1, PDCP will discarded packet #1,3 and indicates RLC layer to further discard. At same time, PDCP SN gap report (i.e., FDC=1, bitmap=010) will be delivered to RLC for transmission. From RLC layer, only buffered packet#0,2 and new arrived PDCP SN gap report wait for transmission which will be assigned with large RLC SN than the buffered packet#0. Then, the smart UE implementation will prioritize transmission of RLC PDU carrying buffered packet#0 over RLC PDU carrying PDCP SN gap report to make the transmitting window move forward. That means that packet #0,2 has been transmitted for a while via Uu when the PDCP SN gap report is received by receiving PDCP entity. By setting suitable reordering timer value, it can be guaranteed that packet #0,2 will be received by PDCP entity during the running of t-Reordering timer, which means that problem in [2] does not exist.

Observation 3: Due to PDCP pre-processing, the transmission of PDCP SN gap report is done by RLC layer later than the transmission of buffered RLC packet which arrives the RLC before PDCP SN gap report. By further setting suitable t_reordering timer value, it can guarantee that packets with COUNT value which is less than the largest COUNT value associated with discarded packet will be received by PDCP.

Based on the above analysis, the problem pointed by the contribution [2] is not a big issue. Then, there is no need to further modify the PDCP window update operation.

Proposal: No further modification on the PDCP window update based on the SN gap report is needed and no spec impact.

3 Conclusion

This document has discussed the PDCP window update based on the SN gap report raised in [2], based on the discussion, we have the following observations and proposal:

Observation 1: The contribution [2] pointed that If RX_NEXT is updated past all SDUs indicated as discarded, the receiving PDCP entity may prematurely update RX_DELIV past unreceived high-importance SDUs when pdu-SetDiscard is configured and PSI-based SDU discard is activated.

Observation 2: UE implementation can guarantee in a certain degree that PDCP SDUs belongs to the same PDU set will be assigned with continuous SN/COUTN values, which makes the problem pointed by the contribution [2] not exist.

Observation 3: Due to PDCP pre-processing, the transmission of PDCP SN gap report is done by RLC layer later than the transmission of buffered RLC packet which arrives the RLC before PDCP SN gap report. By further setting suitable t_reordering timer value, it can guarantee that packets with COUNT value which is less than the largest COUNT value associated with discarded packet will be received by PDCP.

Proposal: No further modification on the PDCP window update based on the SN gap report is needed and no spec impact.

3 Reference

[1] RAN2#126 chairman note.

[2] R2-2404287 PDCP SN gap report and updating RX_NEXT.