3GPP TSG-RAN WG2 Meeting #127	R2-2406779

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	7.5.3

Source:	OPPO

Title:	Further discussion on the DRX_SFN_COUNTER initialization

Document for:	Discussion, Decision

Introduction

This paper will further discuss the DRX_SFN_COUNTER initialization for handover.

Discussion

Regarding how to initialize DRX_SFN_COUNTER, RAN2 reached the following agreements after several meetings’ discussions.

#125 Agreements:

1 	Address the issue of DRX-config crossing the H-SFN boundary:

Initialize DRX_SFN_COUNTER to 1 when the DRX-config RRC signalling containing drx-TimeReferenceSFN is received during the first half of a hyper frame (SFN is between 0 and 511).

Otherwise, initialize DRX_SFN_COUNTER to 0.

# 126 Agreements:

=>	Remove the sentence “in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]” from the procedure text. Adopt the TP for TS 38.321 for DRX_SFN_COUNTER initialization.   FFS HO case

Some companies mention initialization misalignment on the DRX_SFN_COUNTER during handover, as the source/target cells may have unaligned SFN.

DRX_SFN_COUNTER R2-2404547 Open issues for DRX_SFN_COUNTER for XR ZTE Corporation, Sanechips Proposal 1: The DRX_SFN_COUNTER is initialized in the first symbol of the slot immediately after the first PUSCH transmission which contains the response message of the RRC (re-) configuration message (i.e. point of initialization is same as the one captured in the current MAC spec) Proposal 2: If, the first symbol of the slot immediately after the first PUSCH transmission which contains the response message of the RRC (re-) configuration message falls in the first part of the HFN, the DRX_SFN_COUNTER shall be initialized to 0. Otherwise, it shall be initialized to 1. => Noted R2-2404173 Discussion on DRX_SFN_COUNTER Qualcomm Incorporated Proposal 1. UE determines the initial value of DRX_SFN_COUNTER based on the time when a HARQ ACK for the RRC message containing a DRX re-/configuration is sent. UE applies the new DRX configuration when it decodes the RRC message (as in legacy). => Noted R2-2404513 DRX SFN counter initialization Ericsson discussion Rel-18 NR_XR_enh-Core Proposal 1 Adopt the TP above for initializing the DRX_SFN_COUNTER. => Noted R2-2405047 Discussion on the DRX_SFN_COUNTER initialization OPPO discussion Rel-18 NR_XR_enh-Core Proposal 1 Update the MAC spec to initialize DRX_SFN_COUNTER upon the corresponding RRC (re-)configuration message for DRX being received. => Noted R2-2404772 Remaining issues for DRX operations for XR Huawei, HiSilicon discussion Rel-18 NR_XR_enh-Core Proposal 1: Remove the sentence “in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]” from the procedure text. Adopt the TP for TS 38.321 for DRX_SFN_COUNTER initialization. => Noted Discussion - Nokia thinks that we don’t need a specific timing and thinks we should go with Huawei’s proposal. Oppo, Xiaomi, LG agree. - Samsung thinks that ZTE and Ericsson’s make sense as it should be done after synchronization with target cell. - ZTE thinks that the problem is that the processing time is unknown. We are leaving it to UE implementation that may cause problem so it would be good to make the point very clear. - Qualcomm thinks that there is still ambiguity when the RRC message was received and whether the UE and the NW are aware. - LG thinks that the UE applies the configuration when the RRC configuration message is received and HW proposal addresses this. Huawei thinks that when the network would send the message and then increment it by one and so would the UE. - Samsung is concerned with the HO case. ZTE is not concerned with the HO case. Xiaomi explains that we agree that the UE would initialize when it acquires a new SFN in target cell. - Lenovo thinks that we never specify small timing issues and ambiguities. - ZTE explains that the network won’t know that the UE applied the configuration until it receives the RRC response. - Huawei explains that one problem is that the UE would apply part of the configuration in one point and another one at another point. ZTE thinks that the configuration would apply at the same time, it is just about the counter managing and we initialize counter at different points. - Intel suggests to add a clarification for the HO scenario.

We tend to think no special handling is needed for handover. It is because, regardless of whether the source/target cells have synchronous or asynchronous SFN timing, only the target cell and the UE need to initialize the DRX_SFN_COUNTER. As the RRCReconfiguration message and the RRC reconfiguration complete message are exchanged between the target cell and the UE, the UE and the target cell would detect the SFN wrap-around issue based on factors such as timing of the target cell and the value of drx-TimeReferenceSFN. This assumes that the time interval between these two messages would not exceed 10240ms.

No spec change on the DRX_SFN_COUNTER initialization for handover.

Conclusion

We have the following proposals:

Proposal 1	No spec change on the DRX_SFN_COUNTER initialization for handover.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)