3GPP TSG-RAN WG2 Meeting #127	R2-2406780

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

The Rel-19 NES WID has one objective related to on-demand SIB1 for UEs in RRC idle/inactive mode.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

This paper will outline our opinions on the following critical issues related to on-demand SIB1:

- Support for Scenario3/Case 1

- Necessity of WUS configuration acquisition from NES Cell

- Necessity of SIB1 validity

- Support for Case 3

- De-prioritization of the legacy UE attempting to switch to the NES Cell from its camping Cell A

Discussion

Whether to support Scenario 3/Case 1

After discussing potential scenarios introduced in RAN1#116, RAN2 agrees to study Scenario 1a and wait for RAN1’s progress on Scenario 3 which has a tight dependence on RAN1.

Agreements on on-demand SIB1: - At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded. Scenario 3 (case 1 in RAN1) - RAN2 to wait for RAN1’s progress whether to support scenario 3.

Till now, Case 1/ Scenario 3 has been deprioritized/excluded by the conclusions achieved in RAN1#117.

Agreement For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Combining the conclusions above, we understand that RAN2 can formally deprioritize/exclude Scenario 3/Case 1 from Rel-19 NES.

- RAN1#117 has already deprioritized/excluded Scenario 3/Case 1.

RAN2 deprioritizes/excludes Scenario 3/Case 1 for on-demand SIB1 from Rel-19 NES.

Necessity of WUS configuration acquisition from NES Cell

Currently, RAN2 has agreed that Cell A is obliged to provide the UL WUS configuration for NES Cell. This question is whether the WUS configuration can be provided by the NES cell after the UE camps on that cell. Having the NES cell provide the UL WUS configuration would prevent the NES UE from reselecting to Cell A again in order to re-acquire the UL WUS configuration of the NES cell.

Transmission of WUS configuration 1. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

We are hesitant to support the NES cell provision because the UL WUS configuration is not frequently changed and the proposed solution does not fit all scenarios.  For example, as the NES cell's SIB1 is only transmitted within a specific time window, if the UL WUS configuration updates outside this time window, the NES Cell cannot inform the UL WUS configuration via SIB1. Consequently, the UE has to reselect Cell A without any option.

Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window - FFS: How the search space zero configuration is provided (e.g. from searchSpaceZero in MIB or from a new search space that is indicated by UL-WUS configuration) - FFS: Details of the time window, including at least the starting time and duration - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

- The update of UL WUS configuration may occur outside the OD-SIB1 time window of NES Cell.

RAN2 confirms that acquiring the UL WUS configuration from NES Cell is unnecessary.

If the stored UL WUS configuration is outdated, the UE may fail to acquire the SIB1 of NES Cell and thus eventually go back to Cell A. This attempt to acquire SIB1 is useless but introduces unnecessary latency and UE power consumption. To resolve this issue, RAN2 may consider using a Short Message by NES Cell to inform the UE the change of the UL WUS configuration.

The outdated UL WUS configuration stored by the NES UE causes the failure of SIB1 acquisition and eventually requires the UE to go back to Cell A for the latest UL WUS configuration.

Necessity of SIB1 validity

In legacy, SIBx(other than MIB/SIB1) validity is supported for power saving, allowing the UE to bypass the SI acquisition and use the stored valid SIBx.

- 5.2.2.2.1 SIB validity The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB. When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49].

A proposal has been submitted in RAN2#126 to expand the aforementioned approach to SIB1, however, RAN2 has not yet reached a conclusion due to conflicting views among companies. We understand the intention of supporting the validity of SIB1 but feel it is unfeasible to implement the valueTag/areaScope-based approach for SIB1, as the MIB of NES Cell does not have enough reserved bits. It is worth noting that the NES Cell can still use a Short Message to inform the SIB1 update, and the UE can skip the SIB1 acquisition until the Short Message is received.

Paging and SIB1 update in NES cell 17. After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

- NES Cell’s MIB does not have enough reserved bits to inform validity indication for its SIB1.

RAN2 does not extend the legacy SIB validity (i.e., the valueTag/areaScope-based approach) mechanism to SIB1 of NES Cell.

Whether/how to support Case 3

In Case 3, which is Option 2+B+Y, the proponent mentioned in the latest meeting that the UE can ask for Cell A's SIBX, which carries NES Cell's SIB1, using the legacy SI request mechanism and can directly obtain NES Cell's SIB1 from Cell A.

- On target cell of UL WUS transmission: - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option Y: UE receives on-demand SIB1 from Cell A

It appears that the UE can reuse the legacy on-demand SI request procedure for the SIB1 acquisition, but this approach brings a higher demand on the NR system:

- Close coordination between Cell A and NES Cell

As the SIB1 content is determined by and comes from NES Cell, close coordination and interchange are essential between Cell A and NES Cell, otherwise, the NES Cell’s SIB1 provided by Cell A would be outdated (if the SIB1 update is not interchanged in time), or, delayed (if the latest SIB1 is delivered by NES Cell only after it receives the on-demand SIB1 request from Cell A).

- More support from Cell A and the corresponding scalability issue

Compared to Case 2, Case 3 asks for more support from Cell A, that is, Cell A needs to additionally handle UL WUS reception and SI message delivery to inform other cells’ SIB1. This puts an additional load on Cell A and increases its energy consumption. The scalability issue becomes critical when the number of NES Cells grows, as Cell A has to handle on-demand SIB1 requests for multiple NES Cells.

- Compared to Case 2, Case 3 requires 1) close coordination between Cell A and NES Cell, and 2) more support from Cell A which puts additional load on Cell A and increases its energy consumption.

RAN2 deprioritizes Case 3 for on-demand SIB1 from Rel-19 NES.

Deprioritize the legacy UE attempting to switch to the NES Cell

RAN2#126 has discussed the impact on the legacy UE and achieved the following agreements. As the first bullet has a tight dependence on RAN1, we would like to share more detailed opinions on the second bullet.

Legacy UE impact 14. NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) 15. How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

The intention is to implement individual cell reselection operations for both legacy UE and NES UE. This will involve deviating from the current design of broadcasting unified reselection parameters for all UEs camping on this cell. Otherwise,

- The NES UE will be hard to reselect the NES Cell if the frequency priority of NES Cell is lower than that of Cell A. Or,

- The legacy UE will waste power and delay the reselection to a suitable cell if the frequency priority of NES Cell is higher than that of Cell A.

To implement the differentiated handling for legacy UE and NES UE, the network can configure separate cell reselection parameters used by the NES and legacy UE. E.g.,

- different cell reselection priority, so that the legacy UE can deprioritize the NES frequency and the NES UE can still handle the NES frequency normally.

- different allowed/excluded cell list, so that the legacy UE can exclude the NES cell in the excluded cell list from the reselection candidate cells and the NES UE can still reselect to such NES cell if the cell reselection rule fulfils.

To implement differentiated cell reselection handling for the legacy UE and NES UE, the network can configure separate cell reselection parameters for legacy and R19 NES UEs (e.g., priority and/or allowed/excluded cell list).

Conclusion

We have the following observations:

Observation 1	RAN1#117 has already deprioritized/excluded Scenario 3/Case 1.

Observation 2	The update of UL WUS configuration may occur outside the OD-SIB1 time window of NES Cell.

Observation 3	NES Cell’s MIB does not have enough reserved bits to inform validity indication for its SIB1.

Observation 4	Compared to Case 2, Case 3 requires 1) close coordination between Cell A and NES Cell, and 2) more support from Cell A which puts additional load on Cell A and increases its energy consumption.

We have the following proposals:

Proposal 1	RAN2 deprioritizes/excludes Scenario 3/Case 1 for on-demand SIB1 from Rel-19 NES.

Proposal 2	RAN2 confirms that acquiring the UL WUS configuration from NES Cell is unnecessary.

Proposal 3	The outdated UL WUS configuration stored by the NES UE causes the failure of SIB1 acquisition and eventually requires the UE to go back to Cell A for the latest UL WUS configuration.

Proposal 4	RAN2 does not extend the legacy SIB validity (i.e., the valueTag/areaScope-based approach) mechanism to SIB1 of NES Cell.

Proposal 5	RAN2 deprioritizes Case 3 for on-demand SIB1 from Rel-19 NES.

Proposal 6	To implement differentiated cell reselection handling for the legacy UE and NES UE, the network can configure separate cell reselection parameters for legacy and R19 NES UEs (e.g., priority and/or allowed/excluded cell list).

Reference

[1] RP-240170 Revised WID for Enhancements of network energy savings for NR