3GPP TSG-RAN WG2 Meeting #127	R2-2406781

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.7.1.1

Source:	OPPO

Title:	Discussion on the LS from SA2 and RAN3

Document for:	Discussion, Decision

Introduction

A SA2 LS [2] includes two additional questions on Rel-19 XRM. Also, RAN3 asks about RAN2’s understanding of UL PSI-based PDU discarding [3]. In this contribution, we share our views on these questions.

Discussion

2.1 Consideration of Question 1 in [2]

In [1], SA2 mentions a method in which periodicity is provided dynamically via the GTP-U header and asks for RAN2’s feedback on such a method.

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

We feel reluctant to support such a dynamic method for the reasons below:

- Rel-18 XR already supports the CP-based periodicity informing. If periodicity is updated, the updated periodicity can be informed by the TSCAI message.

- We do not see the benefit of the UP-based method beyond the current CP-based one, as it is unclear why the periodicity would be changed very frequently.

- To provide the periodicity, Rel-18 XR already supports the CP-based method. This CP-based method can also be used to inform the update of periodicity.

RAN2 does not see the need to support dynamic periodicity provision via in-band signalling.

2.2 Consideration of Question 2 in [2]

[1] mentions one concept called “time to next burst”. This defines the time interval between the current burst and the next burst. To help SA2’s progress, SA2 is interested in knowing RAN2’s opinion on how useful time to next burst is for RAN resource scheduling.

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

In our memory, during the Rel-18 XR discussion, there was a similar conversation about Inter-Burst Time, which has a comparable concept and usage as “time to next burst”. At that time, RAN2 expressed doubts about the usefulness of this dynamic information and mentioned RAN2’s doubts in R2-2306572.

To us, nothing has changed since that time, thus, we prefer RAN2 to keep our previous understanding.

- In Rel-18, RAN2 already expressed doubts about the benefit of Inter-Burst Time which has a comparable concept and usage as time to next burst.

RAN2 does not see the benefit of time to next burst.

2.3 Consideration for [3]

In [3], RAN2 is asked about clarification on the UE behaviour on PSI-based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

With regards to this matter, RAN2 has already addressed a similar discussion in Rel-18 and has reached the following conclusion.

Agreements: For PSI-Based SDU Discard Activation/Deactivation MAC CE, Di field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with PSI-based SDU discard and with RLC entity(ies) associated with this MAC entity.

It interprets that when both MN and SN independently provide a MAC CE to the UE for a split bearer, UE follows the MAC CEs received from MN and SN. In such a scenario, the UE does not need to manage conflicts between the commands received from MN and SN. Namely, the UE always follow the command for the corresponding split DRB regardless of which node’s command is received.

For the split DRB, RAN2 confirms that the UE always follows the PSI-based activation/deactivation command received from the MN and/or SN, while the UE does not need to manage the conflicts between the commands from MN and SN.

Conclusion

We have the following observations:

Observation 1	To provide the periodicity, Rel-18 XR already supports the CP-based method. This CP-based method can also be used to inform the update of periodicity.

Observation 2	In Rel-18, RAN2 already expressed doubts about the benefit of Inter-Burst Time which has a comparable concept and usage as time to next burst.

We have the following proposals:

Proposal 1	RAN2 does not see the need to support dynamic periodicity provision via in-band signalling.

Proposal 2	RAN2 does not see the benefit of time to next burst.

Proposal 3	For the split DRB, RAN2 confirms that the UE always follows the PSI-based activation/deactivation command received from the MN and/or SN, while the UE does not need to manage the conflicts between the commands from MN and SN.

Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] S2-2407351 LS on FS_XRM Ph2 (Contact: vivo)

[3] R3-243957 LS on UL PSI based PDU discarding in NR-DC(Contact: Qualcomm)