z3GPP TSG-RAN WG2 Meeting #127	R2-2406782

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.7.2

Source:	OPPO

Title:	Discussion on the multi-modality support

Document for:	Discussion, Decision

Introduction

The revised work item of Rel-19 XR for NR phase 3 was approved in RAN#103 with the following objective [1]:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed).

In this contribution, we provide our further considerations on this objective.

Discussion

RAN2#125bis has a preliminary discussion on multi-modality and achieves the following conclusions.

- For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information. FFS what information is needed/useful, e.g. just mulit-modal ID, association between the flow, synchronization requirement etc.

- RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE

- RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge

- Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas

In RAN2#126, RAN2 confirms its support for RAN-aware multi-modality in Rel-19 for UL/DL. Accordingly, RAN2 sends an LS asking SA2 whether/what information could be provided for multi-modality. As a reference, RAN2 lists information examples (i.e. MMSID and/or synchronization thresholds) that would be useful for coordinated handling of multi-modal flows.

- Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

- Ask SA2 whether/what information could be provided to RAN

In reference to [2], SA2#163 did not send the reply LS but plans to reach a conclusion in the August meeting. In light of the checkpoint deadline, we recommend that RAN2 waits for further SA2 progress and continues the parallel discussions unrelated to the CN work, such as the impact of multi-modality on the UE aspect.

- On whether/what information can be provided from SA2 for coordinated handling of multi-modal flows, SA2 failed to respond in May but intends to reply in August.

RAN2 waits for further SA2 progress on whether/what information to be provided for multi-modality and continues the parallel discussions independent of the CN work (e.g. the UE impact due to multi-modality).

For the uplink, regardless of the multi-modal information provided by CN, the UE assistant report is always supplementary for the gNB to be aware of multi-modal information. In our opinion, the multi-modal information from UE should at least include the association between the QoS flows, otherwise, the gNB cannot guarantee the sync requirement of the multi-modal service. Assuming this association does not change frequently, the UE would report such an association via UAI. As usual, we can rely on the internal implementation for UE awareness of the multi-modal information. For instance, the UE AS knows the association between QoS flows based on the interaction from the UE NAS, which identifies such association by the MM Service ID and the QoS rule.

For the uplink, the gNB knows the multi-modal information by the UE report, which includes the association between the QoS flows.

For the uplink, RAN2 assumes the UE identifies the multi-modal information by its implementation.

Even if the UE/gNB has the multi-modal information, the current QoS flow-DRB mapping rule allows for 1) different DRBs to be associated with packets having the same MM requirement, and 2) a single DRB to be associated with packets with and without MM requirement. This is because the MM service ID is not a factor considered in the data flow-QoS flow mapping.

- RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows

- For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed

- Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs

Accordingly, the NR system encounters situations where the MM packet may appear and disappear intermittently. In order to ensure the sync transmission for the MM packets for the LCHs associated with the multi-modal flow and maintain fairness among LCHs in the absence of MM packets, the gNB can configure additional LCH parameters, such as LCH priority, in addition to the existing ones.  These additional parameters are intended to be used when the MM packet(s) arrive on these LCHs. As a result, the UE will utilize the legacy LCH parameters in the normal case and switch to using the additional LCH parameters when the MM packet(s) arrive.

RAN2 studies the enhanced LCP mechanism to allow for the transmission of the multi-modal packets at the same/similar time.

In the enhanced LCP procedure, the gNB can configure additional LCH parameters (e.g. LCH priority) besides the legacy ones, which aims to be used when the MM packet(s) arrives on these LCHs.

In the enhanced LCP procedure, the UE switches to use additional LCH parameters configured for one LCH when the MM packet belonging to this LCH arrives.

In addition, some companies would like to discuss admission control for multi-modal flows in RAN, which aligns with the following agreement.

- Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas

The typical use case is to utilize the association information during the NG-RAN handover. As mentioned by proponents, the source gNB can choose a proper destination considering the acceptance/rejection of the associated flows, or, the target gNB can perform joint admission control for the UE mobility by using the association information transferred from the source gNB. As the aspect mainly involves the interaction between the NG-RANs and the related actions on the NG-RAN side, we prefer to leave the discussion to RAN3.

Leaves to RAN3 to discuss the association information-based joint access control.

Conclusion

We have the following observations:

Observation 1	On whether/what information can be provided from SA2 for coordinated handling of multi-modal flows, SA2 failed to respond in May but intends to reply in August.

We have the following proposals:

Proposal 1	RAN2 waits for further SA2 progress on whether/what information to be provided for multi-modality and continues the parallel discussions independent of the CN work (e.g. the UE impact due to multi-modality).

Proposal 2	For the uplink, the gNB knows the multi-modal information by the UE report, which includes the association between the QoS flows.

Proposal 3	For the uplink, RAN2 assumes the UE identifies the multi-modal information by its implementation.

Proposal 4	RAN2 studies the enhanced LCP mechanism to allow for the transmission of the multi-modal packets at the same/similar time.

Proposal 5	In the enhanced LCP procedure, the gNB can configure additional LCH parameters (e.g. LCH priority) besides the legacy ones, which aims to be used when the MM packet(s) arrives on these LCHs.

Proposal 6	In the enhanced LCP procedure, the UE switches to use additional LCH parameters configured for one LCH when the MM packet belonging to this LCH arrives.

Proposal 7	Leaves to RAN3 to discuss the association information-based joint access control.

Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] R2-2405782 LS on multi-modality awareness at RAN, contact: Nokia