3GPP TSG-RAN WG2 Meeting #127	R2-2406783

Maastricht, Netherlands, Aug 18th – 23rd, 2024

Agenda item:	8.7.1.1

Source:	Samsung

Title:	Discussion on incoming LSs

Document for:	Discussion & Decision

Introduction

This document discusses about questions from incoming LSes [1][2].

Discussion

LS from SA2

In the LS [1] from SA2, there are two questions for RAN2:

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?.

Question 1 is whether RAN2 sees any values in introducing a new in-band signalling (i.e., in GTP-U) for providing the periodicity (of DL traffic) to gNB. Although DL traffic periodicity is useful for gNB, in Rel-18, it is already feasible for 5GC to provide the DL traffic periodicity to gNB through NG AP TSC Assistance Information (TSCAI) as specified in TS 23.501. It is redundant to additionally introduce another solution for providing the same information.

Proposal 1. RAN2 to respond that, since 5GC already can provide periodicity information to gNB through TSCAI, it is redundant to develop another in-band solution for providing the periodicity information.

Question 2 is whether the time to next burst information provided by application server, is useful for RAN resource scheduling. The time to next burst information discussed here is supposed to be provided by application server, which means it indicates the interval between two consecutive data burst generated at application server. Since there will be jitter encountered by each data burst between the application server and gNB, it is hard for gNB to know the exact burst arrival time relying solely on the time to next burst information from the application server. Also, gNB is already provided with many useful information, e.g., periodicity, N6 jitter, end of data burst, PDU set information, etc., which can be exploited to enable efficient resource scheduling and UE power saving management. Also, utilization of such information will be handled by gNB implementation, and there is no RAN2 spec impact.

Proposal 2. RAN2 to respond that, the time to next burst provided by application server is not an essential information for RAN resource scheduling, and no RAN2 spec impact is foreseen on utilization of this information.

LS from RAN3

In the LS from RAN3 [2], there is one question for RAN2:

... Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

The question is to let RAN2 clarify the UE behavior, regarding how UL split-bearer reacts to PSI-Based SDU Discard Activation/Deactivation MAC CE received from MN and/or SN.

Recall that in RAN2#125bis, we already discussed the same issue triggered by the proposal from Qualcomm, which leads to the conclusion, i.e., “no further optimizations will be supported in Rel-18 for UL split-bearer.” as shown below.

... PSI based SDU discard with UL split bearers R2-2402387 Issues related to PSI-based discard Qualcomm Incorporated discussion Rel-18 NR_XR_enh-Core Proposal 2. A UL split-bearer deactivates PSI-based SDU discard once it has received a MAC CE deactivating the discard from either MN or SN. - Ericsson doesn’t think this makes sense. Lenovo thinks it makes sense. ZTE indicates that we agreed to not optimize this. Nokia thinks that we would have a problem. Qualcomm thinks that if we don’t specify anything there will be ambiguity. => No further optimizations will be supported in Rel-18 for UL split-bearer

It means no further optimization regarding PSI-based SDU discard activation/deactivation in the case of UL split-bearer is considered for Rel-18 XR.

By referring to the current MAC spec (captured below), our understanding is that, if a MAC entity (of MN or SN) receives a PSI-based SDU Discard Activation/Deactivation MAC CE indicating activation/deactivation of the PSI-based SDU discard, it shall indicate the activation/deactivation of the PSI-based SDU discard, to the corresponding PDCP entity, for each DRB (including UL split-bearer). That is, a UL split-bearer should activate/deactivate PSI-based SDU discard, if it receives the corresponding MAC CE at least from one of the cell groups.

Proposal 3. RAN2 to respond that, a UL split-bearer should activate or deactivate PSI-based SDU discard, if it receives PSI-based SDU Discard Activation/Deactivation MAC CE, which activates or deactivates the PSI-based SDU discard for the UL split-bearer, from at least one of the MN and SN.

TS 38.321, v18.2.0 ... 5.18.34 Activation/deactivation of PSI-based SDU discard The MAC entity shall for each DRB configured with discardTimerForLowImportance: 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB: 2> indicate the activation of the PSI-based SDU discard for the DRB to upper-layers. 1> if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB: 2> indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers. ...

Conclusion

RAN2 is requested to discuss and agree the following proposals:

For SA2 LS:

Proposal 1. RAN2 to respond that, since 5GC already can provide periodicity information to gNB through TSCAI, it is redundant to develop another in-band solution for providing the periodicity information.

Proposal 2. RAN2 to respond that, the time to next burst provided by application server is not an essential information for RAN resource scheduling, and no RAN2 spec impact is foreseen on utilization of this information.

For RAN3 LS:

Proposal 3. RAN2 to respond that, a UL split-bearer should activate or deactivate PSI-based SDU discard, if it receives PSI-based SDU Discard Activation/Deactivation MAC CE, which activates or deactivates the PSI-based SDU discard for the UL split-bearer, from at least one of the MN and SN.

References

[1] S2-2407351, LS on FS_XRM Ph2, SA2.

[2] R3-243957, LS on UL PSI based PDU discarding in NR-DC, RAN3.