3GPP TSG-RAN WG2 Meeting #127	R2-2406784

Maastricht, Netherlands, Aug 18th – 23rd, 2024

Agenda item:	8.7.4

Source:	Samsung

Title:	Scheduling enhancements for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In RAN2#126 meeting, RAN2 made the following agreements for scheduling enhancements:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR.

The solution should consider impact on UE complexity (as already indicated in SI objective description)

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

- - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

We try to avoid RAN1 impacts.

RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

The solutions should not disallow non-delay critical data from using an UL grant.

LCP prioritization within a logical channel will not be considered in RAN2 discussions

FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR)

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

FFS whether DSR triggering is impacted

FFS whether PDU set importance needs to be included

There are three directions for candidate enhancements, i.e. LCP restriction/selection, LCH prioritization, and DSR enhancements.

Discussion

Delay-Critical Data in LCP Enhancements

In Rel-18, the concept of delay-critical data was well defined by comparing remaining PDCP discardTimer value with remainingTimeThreshold configured by NW, for DSR triggering and reporting. Some companies argue that another threshold different from the one configured for DSR should be introduced for defining another concept of delay-critical data prioritized in LCP, with the rationale that DSR triggering/reporting time is different from that of the LCP, such that the remainingTimeThreshold should be tailored for respective urgencies. But we see the benefit from introducing different thresholds for LCP and for DSR is marginal, since NW would anyway allocates UL resource for the delay-critical data as soon as possible after receiving DSR, such that the time gap between DSR and LCP would be minimized by NW as much as possible. So, we think the remainingTimeThreshold applied in DSR can be reused in LCP enhancements as a baseline, to avoid unnecessary signalling overhead and UE complexity.

Proposal 1. RAN2 to consider the criterion for delay-critical data applied in Rel-18 DSR as the baseline for that applied in LCP enhancements in Rel-19 XR.

LCP Restriction

LCP restriction has been introduced in Rel-15 NR for resource separation of traffic flows with different QoS characteristics. The basic principle of LCP restriction is that a group of LCHs with similar QoS characteristics have dedicated resources, e.g. dedicated cell, dedicated CG, dedicated PUSCH duration, etc.

We have agreed not to consider solutions requiring RAN1 impact, which highly limits the possible directions/benefits we can expect with LCP restriction enhancements, e.g., the UL grant dedicated for delay-critical data, which requires signalling via DCI is no longer of our interest. That is, without RAN1 impact, we see little value in enhancing LCP restriction.

Observation 1. There is little benefit to enhance LCP restriction without RAN1 impact.

Also, if we enhance LCP restriction for delay-critical data, it means dedicated UL resource needs to be reserved only for the delay-critical data, even when non-delay critical data can be served with residual UL resource, resulting in the resource waste.

On the other hand, we see more room in LCH prioritization than LCP restriction, from the perspectives of higher flexibility and finer granularity, in prioritizing the delay-critical data, while not strictly preventing the non-delay-critical data from being served, during LCP.

Proposal 2. RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

LCH Prioritization

For the prioritization of the delay-critical data, as we agreed in the last meeting, one simple solution is introducing one additional LCH priority, supposed to be higher than the legacy one, for LCH to apply, only when delay-critical data is buffered in the LCH, to temporarily prioritize the LCH with delay-critical data. We think this direction of approach is efficient in achieving the prioritization goal, while “light-weight”, since most of the legacy LCP procedures can be reused after an LCH priority determination phase.

Proposal 3. RAN2 to confirm that LCP prioritization enhancement is based on introducing an additional LCH priority per LCH for LCP, which is applied only when delay-critical data is buffered in the LCH.

One issue would be whether we should apply the additional LCH priority for the entire data of the LCH (LCH-level prioritization) including both delay-critical and non-delay-critical data, or only for the delay-critical data of the LCH (SDU-level prioritization), when allocating UL resource for the LCH.

This issue closely relates to the trade-off between complexity and capacity. That is, to enable SDU-level prioritization, UE should check whether each buffered SDU of an LCH is delay-critical or not, during LCP when allocating UL resource, which brings in non-trivial complexity compared to the legacy LCP, while, with LCH-level prioritization, UE only needs to determine the appropriate LCH priority based on the existence of the delay-critical data, and reuse the legacy LCP procedure, when allocating UL resource. On the other hand, SDU-level prioritization can enable higher XR capacity, since the UL resource is fully exploited by the delay-critical data across different LCHs, while LCH-level prioritization cannot completely resolve the issue of delay-critical data being delayed by non-delay-critical data.

Since LCP is one of the most complicated mechanisms in UE implementation, which also faces tight delay budget (i.e., the time gap between UL grant reception and PUSCH transmission), we think LCH-level prioritization is a reasonable compromise to be considered in Rel-19 XR.

Proposal 4. The additional LCH priority is applied to the entire data of the LCH with both delay-critical and non-delay-critical data.

Data Prioritization over MAC CE

For delay-critical data, in-time delivery is important. For this, dedicated configured grant or resource separated by LCP is introduced over multiple releases of NR. In Rel-18, multiple CG occasions, DSR reporting and discard mechanism were introduced to support in-time delivery with higher resource efficiency. However, most of the MAC CEs, e.g., BSR/DSR/PHR, etc., still have higher absolute priorities over any of XR data, and those MAC CEs can use any type of uplink grants. This could make unexpected interruption during LCP and MAC PDU generation, resulting in delay-critical data being delayed due to such MAC CEs. Therefore, we see value in data prioritization over some less important/urgent MAC CEs.

Proposal 5. RAN2 to consider prioritizing delay-critical data over less important/urgent MAC CEs.

DSR Enhancements

In RAN#126 meeting, DSR enhancement was discussed in terms of multi-pair reporting. The current DSR only reports one pair of remaining time and buffer size, based on a single remainingTimeThreshold, per LCG. Considering it is likely that multiple delay-critical data with different timeline information would coexist, we have agreed to report multiple pairs of remaining time and buffer size, to let NW have holistic delay status information.

Firstly, to report multiple pairs with extended DSR, NW firstly should configure multiple remainingTimeThresholds. As in the case of legacy DSR, the configuration can be introduced per LCG, which is a reasonable compromise for granularity and signalling overhead.

Proposal 6. NW configures multiple remainingTimeThresholds per LCG for DSR enhancements.

As with the remaining time reported in legacy DSR, for the multiple remaining time extension, it is reasonable to consider reporting the shortest remaining value of the running PDCP discardTimer that is within each remaining time range, where each remaining time range is configured with each two consecutive remainingTimeThresholds as upper and lower bounds, as well as the one with the smallest remainingTimeThreshold as upper bound.

Similarly, UE can report multiple buffer size values per LCG, indicating the volumes of the delay-critical data, whose remaining times belong to the multiple remaining time ranges, respectively. FFS how to handle the data reported by the legacy DSR blindly without considering the remaining discardTimer value (e.g., PDCP/RLC control or data PDU for retransmission).

For example, as illustrated in Fig. 1, if NW configures two remainingTimeThresholds (i.e., Threshold 1 < Threshold 2) for an LCG, among the buffered PDCP SDUs, UE can report the shortest remaining time below Threshold 1 (t1) as one remaining time value, and report the shortest remaining time within Threshold 1 and Threshold 2 (t2) as another remaining time value. Correspondingly, the two buffer size values would be the volume of the delay-critical data with remaining time below Threshold 1 (b1), and that with remaining time within Threshold 1 and Threshold 2 (b2+b3), respectively.

Fig. 1. DSR enhancements for multi-pair remaining time and buffer size reporting.

Proposal 7. With DSR enhancements, for the LCG configured with multiple remainingTimeThresholds, UE reports multiple pairs of the shortest remaining time and the volume of the delay-critical data, corresponding to the multiple remaining time ranges, defined by the two consecutive remainingTimeThresholds as upper and lower bounds, and by the smallest remainingTimeThreshold as upper bound, respectively.

When there are multiple remainingTimeThresholds configured for an LCG, we need to specify, based on which remainingTimeThreshold, the DSR should be triggered. Since the intention of introducing the multiple remainingTimeThresholds is to report the holistic delay status across different remaining time ranges, it is desirable to consider the smallest remainingTimeThreshold as the triggering threshold, which would otherwise, will trigger DSR at the point that only the delay status belonging to the longer remaining time range is always reported.

Proposal 8. For the LCG configured with multiple remainingTimeThresholds, the smallest remainingTimeThreshold is considered for DSR triggering.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

-- LCP enhancements --

Proposal 1. RAN2 to consider the criterion for delay-critical data applied in Rel-18 DSR as the baseline for that applied in LCP enhancements in Rel-19 XR.

Observation 1. There is little benefit to enhance LCP restriction without RAN1 impact.

Proposal 2. RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR.

Proposal 3. RAN2 to confirm that LCP prioritization enhancement is based on introducing an additional LCH priority per LCH for LCP, which is applied only when delay-critical data is buffered in the LCH.

Proposal 4. The additional LCH priority is applied to the entire data of the LCH with both delay-critical and non-delay-critical data.

Proposal 5. RAN2 to consider prioritizing delay-critical data over less important/urgent MAC CEs.

-- DSR enhancements --

Proposal 6. NW configures multiple remainingTimeThresholds per LCG for DSR enhancements.

Proposal 7. With DSR enhancements, for the LCG configured with multiple remainingTimeThresholds, UE reports multiple pairs of the shortest remaining time and the volume of the delay-critical data, corresponding to the multiple remaining time ranges, defined by the two consecutive remainingTimeThresholds as upper and lower bounds, and by the smallest remainingTimeThreshold as upper bound, respectively.

Proposal 8. For the LCG configured with multiple remainingTimeThresholds, the smallest remainingTimeThreshold is considered for DSR triggering.