3GPP TSG-RAN WG2 #127	R2-2406785

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.2.5

Source:	Ericsson

Title:	Topology 2 and DO-A for Ambient IoT

Document for:	Discussion/Decision

# Introduction

RAN2 has made the below agreement regarding general aspects in RAN2#125bis:

Agreements - Unless explicitly stated all agreements apply to all device types and for both topologies. - From RAN2 perspective, the aim is that the design on the interface between gNB/intermediate UE and A-IoT device is common for Topology 1 and Topology 2. - RAN2 will support two use cases, “inventory” and “command”. The definition, detailed wording is FFS - Baseline procedure: Step A: Based on the service request, the gNB/intermediate UE sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the gNB/intermediate UE as needed,: Details FFS - We will study the support of both “inventory” and “command” in the same procedure. - FFS if Initial Trigger Message can also include “command”. - RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

RAN2#126 agreements on general aspects:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that A-IoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

This contribution discusses remaining aspects of A-IoT with a focus on Topology 2 considerations, DO-A assessment. In addition, registration and device identification are also discussed.

# 2	Discussion

## 2.1	Connection topologies

According to the SID [1], both the deployment scenario 1 with Topology 1 and deployment scenario 2 with Topology 2 and UE as intermediate node need to be studied. The study needs to ensure the common air interface between devices and gNB/intermediate UE in both deployment scenarios and connection topologies.

It is being discussed in RAN3 to have a common network architecture for both Topology 1 and Topology 2 [2]. From protocol stack’s perspective, we think main differences from Topology 1 are around the questions which sublayer of UE reader handles A-IoT procedures, i.e., whether RRC or NAS or PDU layer.

RAN2 impact from Topology 2 depends on which sublayer of UE reader (RRC, NAS, or PDU) handles A-IoT procedures in communication with serving gNB.

Given that all the A-IoT communications require legacy Uu radio resources (being consumed by UE readers and network), we think RAN2 can assume gNB is involved in the A-IoT procedures for radio resource management and not to impact on other legacy NR UEs, i.e., RRC should handle A-IoT related information. Fig. 1 below shows an example of protocol stacks for both topologies. In this case, the same air interface between A-IoT devices and reader is used in both topologies, and the same interface between and RAN node serving the devices and CN (A-IoTF) is used in both topologies.

In Topology 2, serving gNB controls radio resources consumed by UE reader for A-IoT procedures.

RAN2 assume RRC sublayer at UE reader handles A-IoT related information.

RAN2 to study how gNB controls UE reader(s) regarding UE reader selection, radio resource allocation, coordination among UE readers, and handling RRC state and mobility of UE reader.

As for UE reader (re)selection, we assume the UE is authorized to be a reader by CN however, RAN should be involved in selecting what UE readers to reach out for A-IoT communication based on some RAN metrics such as Uu channel quality, which RRC state the UE is currently in, or mobility status.

A UE reader can be in any RRC state, the gNB needs ensure that the UE reader is in RRC_CONNECTED when performing A-IoT procedures. This is because A-IoT procedures such as inventory, command, disabling are expected to require relatively long-duration communication between UE reader and serving gNB to accomplish, which is not suitable for UE reader in RRC_IDLE/RRC_INACTIVE. In addition, in order for the gNB to assign resources to UE reader for A-IoT traffic using the legacy scheduling procedure (e.g., dynamic scheduling or configured scheduling), UE reader needs to be in RRC_CONNECTED. Hence, in case the UE reader is in RRC_IDLE or RRC_INACTIVE, gNB needs to transition it to RRC_CONNECTED, e.g., by means of legacy paging.

In addition to RRC state handling, for radio resource management and coordination among UE readers, the legacy RRC signaling framework (interface between UE reader and serving gNB) may need to incorporate new signaling procedures/messages e.g., to indicate and schedule a particular UE reader to perform A-IoT procedures, or to avoid a particular UE reader occupy air interface too long due to serving A-IoT devices.

A-IoT procedures are expected to require relatively long-duration communication between UE reader and serving gNB, which is not suitable for UE reader in RRC_IDLE/RRC_INACTIVE.

UE reader is in RRC_CONNECTED while in communication with A-IoT devices.

Figure 1: Example of protocol stack for both Topology 1 and Topology 2

## 2.2	DO-A Assessment

The focus for traffic types in the SID scope is DO-DTT and DT so far. It is, however, mentioned in the SID that:

- From RAN#104, the study will assess whether the harmonized air interface design can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design is/are not sufficient for the DO-A use case.

For DO-A use cases, a device typically triggers an UL transmission autonomously. Whereas for DO-DTT and DT use cases, for any UL transmission of devices, the gNB/intermediate UE reader needs to send a triggering message beforehand which may comprise resource assignment information based on which the devices can perform UL transmissions either in a backscatter manner or actively generated by devices.

One of the aspects to be considered is how devices supporting DO-A use cases (e.g., device type 2b) initiate random access to deliver DO-A traffic considering the RAN2#125b agreement below:

- Random Access is triggered by the reader

In our understanding, if the active devices upon having UL data can initiate random access themselves without being triggered by network, there needs a separate DL signaling channel to specify resource information for the active devices, similar to the legacy random access with SIB signaling. This is significantly different from current design solution being developed in RAN2 and RAN1 and thus not considered as part of harmonized design. We think random access for delivering DO-A traffic can still be triggered by reader as long as requirements from DO-A use cases are met.

Random access for DO-A use cases can be triggered by the reader.

Regarding DO-A use cases, one example is smart laundry, as an indoor inventory use case (see also clause 5.15 in [3]). Another class of DO-A use case is indoor sensors (e.g., see also clause 5.6, 5.13 in [3]). We assume that DO-A traffic (sensory data) typically has stricter end-to-end latency requirement compared to non DO-A use cases. As an example, end-to-end latency requirement in the use case of 5.13 base station machine room environmental supervision is 1s. This means devices that deliver DO-A traffic cannot wait too long for a reader to trigger them to perform random access. In this case, network/reader should provide a frequent signaling channel to allow devices supporting DO-A traffic to access and send their buffered DO-A traffic by mimicking the legacy procedure where the gNB can signal UEs of RACH resources in SIB1. This can be done e.g., by sending the A-IoT paging/the initial trigger message more often even though reader is not requested to do so by CN. We think this can be left to the gNB/Intermediate UE implementation to trigger/send periodic A-IoT paging/initial trigger message.

To not alert passive devices unintentionally (i.e., no service request from CN), there needs to be an indication from the reader to target only one device type, e.g., device type 2b so that devices with other types can skip monitoring, which can be beneficial from device energy saving perspective.

Latency requirement for DO-A traffic types restricts the maximum waiting time of devices if they are triggered by reader.

Network/reader provides frequent DL signaling to allow DO-A devices to access and deliver their buffered traffic.

Periodic A-IoT paging can be used to trigger DO-A devices. Periodicity is left to the gNB/Intermediate UE implementation.

A device type specific indication is used to not alert non DO-A devices unintentionally.

Devices supporting DO-A traffic type may or may not have UL data when receiving an A-IoT paging message. In case devices have no UL data to deliver, it would be beneficial to define device behavior, i.e., whether to reply to the A-IoT paging or not and what information to contain in the UL reply message. For example, a straightforward option is devices supporting DO-A simply ignore the A-IoT paging in this case. Another option is devices always send UL report without UL data, e.g., containing only a device ID of some form. This helps harmonize better current solution for DT and DO-DTT traffic types. Yet another option is network/reader dynamically indicates to the DO-A supporting devices in the A-IoT paging message itself whether UL report is required or not. The indication can be device type/group specific or device specific. It is proposed that network sends an indication to indicate whether to respond A-IoT paging message in case of no UL data.

Devices supporting DO-A traffic type may or may not have UL data to deliver when receiving A-IoT paging.

gNB/intermediate UE sends an indication indicating DO-A devices whether to respond to A-IoT paging in case of no UL data.

For better harmonization with DT and DO-DTT, DO-A devices can report device ID in case of no UL data when receiving A-IoT paging.

## 2.3	Registration procedure

We think that it is beneficial to have registration for A-IoT devices, if they are capable. Keeping device context helps avoid the need of setting up security, authentication, device ID allocation every time device accesses to the network. Thus, we assume that A-IoT devices need to register to network to get authorized to receive services associated with considered use cases and there is device context established and stored at network and device side.

It is necessary to register device context information for subsequent communication as there is no need to perform identification, authentication, security setup every time when a service procedure (e.g., inventory or command) is initiated.

RAN2 assumes existence of “registration procedure (e.g., authorization and authentication)” catering for A-IoT requirements on authorization and authentication.

Registration procedure can be designed to be standalone or in connection to inventory, command procedures to reduce signaling overhead. Figure 2 below shows an example of registration procedure which is performed in connection to the initial inventory procedure for reduced signaling.

Registration is conditional to device capability. CN determines registration or not based on device capability information.

RAN2 assumes registration can be performed standalone or in connection with inventory procedure for reduced signaling overhead.

Figure 2: Message flow for registration procedure (in connection with inventory procedure)

Steps 1-6: Similar to inventory only procedure.

As an example, device can report in step 5 its registration capability, which helps CN to determine if the registration in next steps 7-10 can be performed for the device. However, CN can also check device capability from e.g., UDM based on device ID provided in step 5. Hence, the registration procedure is not mandatory for all A-IoT devices, i.e., steps 7-10 are in dashed lines and textboxes.

Step 7: In case the device is registration capable, the CN perform authentication, temporary device ID allocation and security determination.

Step 8: the gNB/intermediate UE and device perform relevant procedures over air interface to exchange upper layers (e.g., A-IoT NAS) information related to authentication, security setup, and temporary device ID provisioning.

Step 9: the CN registers device information (e.g., into UDM).

Step 10: the device, the gNB/intermediate UE, CN store device context information for subsequent communications. As described in command only procedure, the temporary device ID can be used to start a subsequent command procedure and security can be used to protect command data without establishing/setup again.

Send a LS to SA2, SA3 and CT1 asking for feedback while indicating the above statements captured in P7-8.

## 2.4 Device identification

Recent RAN2 agreements leave some FFSs about device identification in A-IoT paging message as well as in the UL transmission:

- As baseline, the “inventory only” case is supported by the procedure:

  - Step A: A-IoT paging;

  - Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS.

- As baseline, the “inventory and command” case is supported by the procedure:

  - Step A: A-IoT paging;

  - Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS.

- RAN2 will study the following cases for A-IoT paging message:

  - a message containing an ID of a single A-IoT device.

  - a message containing a group ID that maps to multiple A-IoT devices.

  - a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message.

  - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

The aspect can be summarized in a RAN2 question that what type of device ID to be used in the transmission over air interface between A-IoT device and reader in different scenarios below:

- (1) A-IoT paging

- (2) UL message for Inventory report

- (3) UL/DL message for command

- (4) UL/DL message for other proc (e.g., reg, disabling)

This depends on multiple aspects, e.g., how SA working groups define device identifiers, whether the inventory is initial or subsequent inventory, i.e., if network has communicated with the device earlier and kept any device context or not, i.e., also related to device capability. This also depends on whether the inventory of a device population is realized via a group ID or a mask. We note that both mask-based inventory and group ID based inventory are being discussed in SA2, e.g., TR23-700-13.

For the ease of discussion, RAN2 can assume for each A-IoT device in 5GS, there are AS scope device ID and upper layers (NAS) scope device IDs. Considering the constrained device capability in terms of storage, the former is assumed to be a temporary ID being used between device and reader during a communication session (or access/inventory round). Whereas the latter may consist of more than one ID, e.g., both temporary ID and permanent/static ID. In one example, permanent ID is SUPI like ID, temporary ID allocated by CN is GUTI like ID, and AS scope temporary ID is RN16/RNTI like ID. Fig. 6 shows an example of A-IoT device ID used in the inventory procedure covering both RAN and CN scope.

RAN2 assume for each A-IoT device, there are AS device ID and upper layers (e.g., NAS) scope device ID(s). AS device ID is temporary, whereas upper layer scope ID can include both temporary and permanent/static IDs.

First, it is important to understand the need and possible usage of AS device ID in A-IoT. We assume AS device ID can be promoted from the contention resolution ID (or random number) picked by a device itself or assigned by network/reader, e.g., in case of CFRA. More detailed discussion about AS device ID can be found in [4]. This AS device ID can be used to identify the transmission to/from the device (equivalent to RNTI in NR), especially in case reader is in communication with multiple devices.

While FDM for multiple access is under discussion in RAN1, in case both TDM and FDM are employed for multiple access, reader can communicate with multiple devices at the same time, which is very different from TMD only access where network communicates with only one device at a time, as in RFID. In this case, given different A-IoT device capabilities and inventory and command can be performed in the same procedure or in separation, it can happen that reader communicates with multiple devices in an intertwined manner, as illustrated in Figure 3. In this case, reader needs a way to associate transmission with the transmitting/receiving devices. For example, one option is to use AS device ID. Another option is via resource allocation, e.g., device 1 always uses resource 1 (specified in both time and frequency domains) in whole communication session (e.g., inventory round). RAN2 should discuss the way reader can identify transmission over air interface.

AS device ID can be promoted from contention resolution ID picked by device or assigned by reader.

If TDMA + FDMA is adopted for multiple access, reader can communicate with multiple devices at the same time and thus there needs a mechanism to identify transmission is from/to which device.

Figure 3: Example of reader in communication with multiple devices in parallel

Group ID is mentioned as a case for A-IoT paging. We assume this group ID is managed at upper layer, e.g., A-IoT NAS and if CN provides RAN/reader with a group ID (instead of list of explicit device IDs), RAN/reader is expected to transparently deliver the group ID to targeted devices, as in the case of mask above.

If gNB/reader receives explicit device IDs from CN for a service request, gNB/reader can send corresponding AS device ID(s) in A-IoT paging, if available.

RAN2 assume the group ID is allocated by CN and gNB/reader does not need to make any change.

Similar to step 4, device ID to be reported in the step 6 in figure 4 can be different. Note that this is the third message rather than the first message in 4-step RACH. RAN2 can assume permanent/static device ID cannot be sent in plaintext in the UL transmission. Thus, device can send a concealment of permanent/static device ID, e.g., SUCI-like ID in the first UL message intended to report to CN. In addition, device can send the temporary device ID that is mapped to permanent/static device ID, if available. This is applicable to all types of procedures, e.g., inventory only, inventory + command, command only and other possible procedures such as device disabling, registration, etc.

RAN2 assume permanent/static individual device ID cannot be sent in plaintext.

In the first UL message intended to reply to CN, A-IoT device sends a temporary device ID, if available.

RAN2 assume A-IoT device sends a concealment of the permanent/static device ID in the first UL message, if no temporary ID available.

Figure 4: Device ID used over air interface (which device ID to be sent in steps 4 and 6)

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 impact from Topology 2 depends on which sublayer of UE reader (RRC, NAS, or PDU) handles A-IoT procedures in communication with serving gNB.

Observation 2	A-IoT procedures are expected to require relatively long-duration communication between UE reader and serving gNB, which is not suitable for UE reader in RRC_IDLE/RRC_INACTIVE.

Observation 3	Latency requirement for DO-A traffic types restricts the maximum waiting time of devices if they are triggered by reader.

Observation 4	Devices supporting DO-A traffic type may or may not have UL data to deliver when receiving A-IoT paging.

Observation 5	For better harmonization with DT and DO-DTT, DO-A devices can report device ID in case of no UL data when receiving A-IoT paging.

Observation 6	It is necessary to register device context information for subsequent communication as there is no need to perform identification, authentication, security setup every time when a service procedure (e.g., inventory or command) is initiated.

Observation 7	Registration is conditional to device capability. CN determines registration or not based on device capability information.

Observation 8	AS device ID can be promoted from contention resolution ID picked by device or assigned by reader.

Observation 9	If TDMA + FDMA is adopted for multiple access, reader can communicate with multiple devices at the same time and thus there needs a mechanism to identify transmission is from/to which device.

Observation 10	RAN2 assume permanent/static individual device ID cannot be sent in plaintext.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In Topology 2, serving gNB controls radio resources consumed by UE reader for A-IoT procedures.

Proposal 2	RAN2 assume RRC sublayer at UE reader handles A-IoT related information.

Proposal 3	RAN2 to study how gNB controls UE reader(s) regarding UE reader selection, radio resource allocation, coordination among UE readers, and handling RRC state and mobility of UE reader.

Proposal 4	UE reader is in RRC_CONNECTED while in communication with A-IoT devices.

Proposal 5	Random access for DO-A use cases can be triggered by the reader.

Proposal 6	Network/reader provides frequent DL signaling to allow DO-A devices to access and deliver their buffered traffic.

Proposal 7	Periodic A-IoT paging can be used to trigger DO-A devices. Periodicity is left to the gNB/Intermediate UE implementation.

Proposal 8	A device type specific indication is used to not alert non DO-A devices unintentionally.

Proposal 9	gNB/intermediate UE sends an indication indicating DO-A devices whether to respond to A-IoT paging in case of no UL data.

Proposal 10	RAN2 assumes existence of “registration procedure (e.g., authorization and authentication)” catering for A-IoT requirements on authorization and authentication.

Proposal 11	RAN2 assumes registration can be performed standalone or in connection with inventory procedure for reduced signaling overhead.

Proposal 12	Send a LS to SA2, SA3 and CT1 asking for feedback while indicating the above statements captured in P7-8.

Proposal 13	RAN2 assume for each A-IoT device, there are AS device ID and upper layers (e.g., NAS) scope device ID(s). AS device ID is temporary, whereas upper layer scope ID can include both temporary and permanent/static IDs.

Proposal 14	If gNB/reader receives explicit device IDs from CN for a service request, gNB/reader can send corresponding AS device ID(s) in A-IoT paging, if available.

Proposal 15	RAN2 assume the group ID is allocated by CN and gNB/reader does not need to make any change.

Proposal 16	In the first UL message intended to reply to CN, A-IoT device sends a temporary device ID, if available.

Proposal 17	RAN2 assume A-IoT device sends a concealment of the permanent/static device ID in the first UL message, if no temporary ID available.

# 4	References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024

R3-244387, “On A-IoT RAN functional and logical architecture”, Ericsson, RAN3#125.

TR22.840, “Study on Ambient power-enabled Internet of Things”

R2-2407244, “Discussion on DL reachability for Ambient IoT”, Ericsson, 3GPP TSG-RAN WG2 #127, Maastricht, Netherlands, 19th – 23th August 2024

R2-2406818, “Functionality aspects for A-IoT.docx”, Ericsson, 3GPP TSG-RAN WG2 #127, Maastricht, Netherlands, 19th – 23th August 2024.

R2-2406786, “Discussion on UL multiple access for Ambient IoT”, Ericsson, 3GPP TSG-RAN WG2 #127, Maastricht, Netherlands, 19th – 23th August 2024.