3GPP TSG-RAN WG2 #127	R2-2406786

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.2.4

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

Random access (RA) is one model of UL multiple access that needs to be studied for Ambient IoT (A-IoT). RAN2 has agreed to study 4-step like RA and 2-step like RA. In addition, contention-based access procedure and contention-free access procedure are also to be discussed. We further discuss details for different procedures in this paper.

# 2	Discussion

## 2.1	RAN1 and RAN2 agreements

In RAN2#126, the following agreements were made related to access procedure in random access AI 8.2.5.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements.

- A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

- RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information.

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed.

- From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion

In addition, we use following terms.

CR ID: Contention resolution ID

AS ID: Access Stratum ID

## 2.2	Random Access Trigger

In the last meeting, it is agreed to have random-access for an inventory or command procedure based on two steps or four steps initiated by a network trigger message for traffic DO-DTT. These message flows are tentatively described in below Figure 1.

Figure 1: An example on (a) 4-step contention-based RA (left side, agreed in last meeting), (b) 2-step contention-based RA (top right side, agreed in last meeting) and (b) 2-step contention-free RA (bottom right side).

One aspect of RA configuration is selection or the choice between 2-step and 4-step RA. In our view, the selection of RA type for the device to perform should be left to Reader. Otherwise, this can lead to extreme blind decoding at the Reader not knowing if the incoming initial RA message is Msg1 or MsgA. Another rationale is that 2-step and 4-step RA procedures’ collisions success depend on the inventory load size which Reader may be aware of (if CN divulges the load/group size information) but not the devices itself as their transmission transparent to each other. However, for the purpose of study we are fine with study both access mechanisms, i.e., device autonomously select the RA method, or the reader/gNB indicates the access method.

RAN2 to assume reader-based selection between random-access types (2-step or 4-step) as the baseline for inventory or command use case. FFS on device based autonomous selection.

Given there are two choices for RA procedures, therefore, one of the choices can be set as default as the initial tigger need not to indicate the selected choice every time. The 4-step RA is suitable default choice due to some benefits:

(i) The 2-step RA performs well in low load scenarios. The 4-step RA is useful in high load/collision scenario possibly due to smaller initial message size, however its performance does not deteriorate in low load scenario given collision probability is already low due to smaller contending load (hence large or smaller initial RA message size may have little influence).

(ii) If 2-step RA fails, then it can fallback to 4-step RA for next reattempt.

Based on above mentioned points, it makes sense to employ one RA type as default procedure, device may fallback to another option/RA type if deemed necessary, which can be studied.

RAN2 to study one RA type as a default and fallback access method.

## 2.3	Random Access Message Identifier

RAN2 further studies what ID will be included in the very first message in a RA procedure. Depending on RA type (2-step or 4-step), the choice of identifier can be different due to difference in message type and the purpose it serves.

- For Msg1, the primary goal for ID is to utilize for collision resolution (CR), and other IDs inclusion in this Msg1 can be optional.

- For MsgA, the primary goal for ID is to communicate device ID; the collision resolution can be done solely based of (function/part of) device ID or by including additional ID in MsgA subject to the requirement and policies.

Below is the table providing pros and cons of initial RA message (Msg1/MsgA) options with payload containing ID types. In the indicated options, subject to policies and RA type, the payload (ID) may or may not be a device ID and the whole payload may or may not necessarily act as CR ID, see Fig. 2 below for generalized representation of relation between payload and CR ID.

Table 1: Message ID types

| Options | Message Type | ID Type - ID used in Msg1 payload - ID assigned as CR (from ID uased in Msg1 payload) | Comments/Pros/Cons |
| --- | --- | --- | --- |
| 1 | Msg1 without data | Payload: Random ID CR ID: Random ID (same as payload) | A rand pool can be specified and the contending devices select one of the rand number and include in the Msg1. This rand ID can be short compared to anticipated device ID, and thus simplify Msg1 construction cost for energy starved devices. Given 4- step RA tends to be useful in high load or collision scenario compared to 2-step RA, it is preferred to have smaller initial message, i.e., Msg1 size for contention. |
| 2 |  | Payload: Part/function of device ID CR ID: Part/function of device ID (same as payload) | In 4-step RA, the first UL msg can contain ID based on device ID. Given Msg3 will have the opportunity to supply full device ID or remaining part of device ID if some of it (or function of it) already included in Msg1, then the likelihood of collision success may suffer if this ID (in Msg1), for instance, happens to be a common value among the devices indicative of group or class identifier. Currently, we do not how device ID will look like or what are its consituents but it is safe to assume that some bits of device ID may represent group or class information of the device which more than one device can share. One can cirucmvent the issue, if gNB knows (specified or indicated by CN) what are device ID constituents, and thus, for collision resolution purpose, gNB utilizes the device ID part (selected for Msg1 and CR) which is less likely to be common. |
| 3 |  | Payload: Random ID + part/function of devie ID CR ID: Random ID (part of payload) | The Msg1 payload contains ID composed of two parts: random ID and part of device ID. The rand ID can be used for collision resolution where this ID can be echoed back in Msg2 at the least. Although full ID (payload of Msg1) can be included in Msg2 but it will increase the Msg2 cost. This is useful in scenarios where device IDs are extremely large and Msg3 size can be curtailed by offloading part of the device ID related payload to Msg1. It is important to remark that Ambient IoT devices will have poor synchronization and thus these device cannot afford to have large message sizes. In addition, the energy storage is limited and large message construction may be difficult for such devices. |
| 4 | Msg1 with data | Payload: Full device ID CR ID: Full device ID (same as payload) | The MsgA delivers full device ID. The device ID used for collision resolution, which means gNB echoes back (part of) device ID in Msg2. |
| 5 |  | Payload: Full device ID CR ID: Part of device ID (part of payload) | The MsgA delivers full device ID. In some scenarios, say with extremely low power passive devices, it is costly and unnecessary to echo back full device ID in MsgB as the ID can be extremely large. Then in order to reduce Msg2 cost, it is deemed sufficient to echo back some bits (part of device ID), say N number of LSBs or MSBs as an example. This options can suffer form similar issues described in option 2 above. |
| 6 |  | Payload: Full device + random ID CR ID: Random ID | The Msg1 delivers full device ID plus the rand ID for the purpose of improved collision resolution. This is useful in scenarios where it is costly to echo back full device ID in MsgB as the ID can be extremely large. Consider an example if device ID is 256 bit long, then in order to reduce Msg2 cost, it is deemed sufficient to echo back some bits, say 16 bits. Now these 16 bits can be carved from device ID, say MSBs or LSBs, etc. But there can some issue with selection of 16 bits in case gNB or CN does not have knowledge of ID structure or constiuents. For instance, these 16 bits can end up being common among devices, e.g., if the carved part indicative of the class or device group, then all or large number of devices (if contending for same resource) may receive the same feedback message (Msg2), resulting in collision error. |

Figure 2: The payload in initial RA message contains ID where the part of the payload is used as CR ID. This is generalized representation.

In 4-step contention based RA, the ID included in Msg1 is primarily used for collision resolution.

Regarding A-IoT Msg1 content for 4-step like RA, both options including randomly generated (referred to as Option 1) or generated based on Device ID (referred to as Option 2), need to be carefully studied. Option 1 limits generation of CR ID in AS domain, without interacting other work groups, however, multiple devices may generate the same random ID for contention resolution if there are high number of devices contending to the same inventory procedure, which would result into collision. Option 2 would require RAN2 to interact with other work groups, e.g., SA2, which may incur more design complexity/efforts for RAN2. Meanwhile, usage of part of the device ID may be helpful to reduce the collision probability of CR ID between devices. In our views, both options have pros and cons, therefore RAN2 needs to carefully study before making decision.

In addition, for 4-step contention based RA, it may be feasible to include other information/content pending on RAN1 discussion. As captured in the above table, one of the possibilities is to include part of the device ID (e.g., content of Msg3) in Msg1

For 4-step contention-based RA, RAN2 to study pros and cons of both options regarding CR ID in Msg1 payload:

- (i) Option 1: CR ID is randomly generated by device

- (ii) Option 2: CR ID is generated by device based on the device ID.

For 4-step contention-based RA, FFS on additional information based on Msg3 payload to be included alongside CR ID in Msg1 payload.

In 2-step contention-based RA, the ID(s) included in Msg1 is for the purpose of collision resolution and communication device ID.

For 2-step contention-based RA, RAN2 to study pros and cons of both options regarding Msg1 payload:

- (i) Option 1: Msg1 contains only the device ID,

- (ii) Option 2: Msg1 contains both device ID and additional random ID.

For 2-step contention-based RA, RAN2 to study pros and cons of three options regarding contention resolution (CR) ID echoed in Msg2:

- (i) Option 1: CR ID is the indicated device ID in Msg1

- (ii) Option 2: CR ID is the fraction of indicated device ID in Msg1

- (iii) Option 3: CR ID is the indicated random ID in Msg1.

RA procedures highly sensitive to contending load size, contending message size, etc. Utilizing a fixed size of contending message, e.g., for Msg1 can be suboptimal. For instance, if small load is contending for resources, then Msg1 size can be set small as the base station require smaller CR ID and vice versa. However, there can other factors which can impact the choice of message size for contention, and some of them are listed below.

Table 2: Preference of initial RA message size

| Large initial UL RA message size | Small initial UL RA message size |
| --- | --- |
| - Relative contending load is small; if collisions are not epcted much, then large initial message can be consider which can even full device ID or part of Msg3 payload - Msg3 size restrictions applied (where some of its payload offloaded to Msg1) - Low synchronization error - Less energy use if Msg3 and successive messages not needed | - Relative contending load is large; if collision sare expected better too use smaller initial message size - No Msg3 size restrictions - Large synchronization error |

Given the size of initial RA message size can influence contention performance, therefore itis beneficial to study dynamic message design (size, content) for UL message subject to group size, signalling/use case type (inventory, command). The indication of message size or parameters can be provided in the trigger message.

A fixed identifier size based payload in an initial random-access message may not be optimal as it is sensitive to contending load size and the associated the collision probability and the overall energy usage.

Based on above discussion, we believe, there is no single solution on ID behaviour, rather a dynamic initial message and contention resolution ID configuration can be considered in terms of ID size and constituents (part/full) device ID and random ID, which can be indicated in trigger message.

RAN2 to discuss the feasibility of multiple sizes of Msg1.

If there are large number of devices requiring access which are known to the gNB/intermediate UE, it may be feasible for the gNB/intermediate UE to provide dedicated resources to multiple known devices in a scheduling round. We see this is useful for the gNB/intermediate UE to reduce DL signalling overhead. For contention-free RA, RAN2 can assume single device based mechanism as a baseline, and further study if multi device based case is also supported. Two solutions can be considered for 2-step contention-free random access with multiple devices:

Solution 1: The gNB/intermediate UE has configured AS IDs for the devices a-priory. For any access event triggered by the network the GNB/intermediate UE use AS IDs to perform 2-step contention-free random access with multiple devices. The allocation of AS ID to the device can be similar to legacy framework.

Solution 2: the CN sends device IDs to gNB/intermediate UE to trigger their access. For instance, the network can indicate device IDs to the gNB/intermediate UE, e.g., in NGAP commands. The gNB/intermediate UE can include those IDs in DL access trigger signalling.

We believe, single device can achieve contention-free access even under RA procedure where inventory or command triggers RA for group of devices containing only one device. The contention-free, the message includes data (device ID/inventory report) which can be based on 2-step RA. Depending on how resource allocation is defined for RA procedure, there may require tweaking or addition parameters but that can be left for later study.

RAN2 assumes single device based contention free RA as a baseline.

Given the inventory use case can serve large number of device and thus RA catering such large group can cause lot of collisions. Hence, RAN2 should consider contention-free access for multiple devices. The details on resources can be allocated based on progress in two aspects. The details on resource allocation for contention-free access can be left to study later subject to following progress

RAN1 progress on access mechanism for multiple devices,

RAN2 progress on RA design.

Our aim is to utilize similar resource allocation design like in RA with necessary changes to enable contention-free access. For instance, indication of devices and their resources in trigger message, instead of RA pool where devices autonomously select resources.

The discussion on resource allocation details for contention-based RA is subject to RAN1 progress on scheduling and resource design.

The discussion on resource allocation details for contention-free RA is subject to RAN1 progress on scheduling and resource design and RAN2 progress on contention-based RA design.

RAN2 to consider multiple devices based contention free RA pending on RAN1 progress on resource allocation for multiple devices.

## 2.4	Handling contention-failure and access failure

The transmissions in contention based random-access procedure may be prone to collisions when there is high number of devices contending the channel. In addition, in general, the channel error probability for A-IoT transmissions would be higher than of typical NR transmission due to device’s limited capabilities and most likely its inability to do channel measurements. As a consequence, A-IoT transmissions can be unreliable and thus, a suitable feedback mechanism can be useful to indicate device’s UL message success or failure. However, the feedback has a cost on device energy and processing resources which are already scarce and limited.

Regarding contention-failure and access failure for Msg1, below cases are expected:

Case 1: gNB/intermediate UE doesn’t decode any device’s transmission successfully; this may occur due to high contention or link failure.

Case 2: gNB/intermediate UE decodes/reads at least one device’s transmission successfully.

For case 1, gNB/intermediate UE cannot provide any response message to devices. In this case, a device would be able to figure out that its previous transmission was unsuccessful if the device does not receive response message (Msg2) from gNB/intermediate UE.

After transmitting an initial RA message (Msg1 with/without data), a device considers the transmission unsuccessful if the device doesn’t receive a response message subsequently carrying its contention-resolution identifier (Msg2).

In addition, after a device has won contention resolution and provided data to the gNB/intermediate UE in Msg3, it can be further studied if any acknowledgement message is needed from the gNB/intermediate UE perspective. Companies have different views on whether Msg4 is needed. Some companies think Msg4 is needed to resolve contention that there may be multiple devices choose the same temporary identifier in Msg1. Some other companies think the probability that multiple devices choose the same temporary identifier in Msg1 is rather low, therefore Msg4 is not needed. In addition, it may be beneficial to introduce Msg4 to acknowledge whether Msg3 transmission has been successful or unsuccessful. Anyway, we are open to discuss this. In addition, if Msg4 is needed for contention resolution, the content for Msg3 would need to be further discussed. In the below table, we summarise pros and cons of utilizing Msg4.

Table 3: Msg4 pros and cons

| Msg4 pros | Msg4 cons |
| --- | --- |
| - Msg4 provides acknowledgement for the data/Msg3 transmission, allowing eventual contention resoultion, given two devices may select same random ID in Msg1, and thus may receive same positive acknowledgement in the form of Msg2.This increases reliability of transmissions. - This also provides an opportunity to allocate AS ID for future allocations, addressing, or CF procedures. | - The transmission reliability requirement may not be high, so mandating Msg4 can be unnecessary. In other words, it may be sufficient for device to recover from the failure relying on upper layer retransmission, - Msg4 increases monitoring cost of the devices. - Other devices also need to spend more time in contention due to increased time consumed by Msg4 transmission for every device; many devices may not have the propsects to stay awake for long time due to extreme low energy storage. |

For 4-step contention-based random access, study if Msg4 is needed to acknowledge Msg3 transmission considering the reliability requirements of the use case and energy usage cost at the devices.

In the last RAN2 meeting, different options on how to handle contention-failure and access failure were discussed briefly. The feasible options comprise:

Option 1: Device which experiences contention-failure or access failure, re-accesses in the same round.

Option 2: Device which experiences contention-failure or access failure, re-accesses in a different round.

Option 3: the round length (i.e., the number of time occasions) is adaptive. One round can be terminated earlier upon detection of too high collision. A new round with more time occasions is initiated.

A round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool via a contention-based manner.

With option 1, devices which have failed to transmit don’t need to wait for the next round. Devices which have failed and other devices which have not transmitted would access the rest time occasions at the same time. The queue would pile up during the rest time occasions in this round.

Option 2 may be simpler from gNB/intermediate UE design perspective as devices are assumed to retry in a next round upon unsuccessful access attempt. gNB/intermediate UE is not required to reserve occasions and resources for re-accesses ahead. However, in a different round, devices performing re-accesses may suffer from longer access delay.

With option 3, the length for each round (i.e., number of time occasions) is set by gNB/intermediate UE depending on collision rate.

A round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool in a contention based manner.

For handling contention resolution failure and access failure, RAN2 to study the three options:

  - Option 1: a device which experiences contention-failure or access failure, re-accesses in the same round.

  - Option 2: a device which experiences contention-failure or access failure, re-accesses in the next round.

  - Option 3: the round length is adaptive. The round can be adjusted by increasing its length or terminating earlier upon detection of too high collision. More time occasions are added in current round or in the new round (if current one terminated).

On a question whether the rest competing devices also need to be signalled with an explicit acknowledgement message, we think this is not needed, since the acknowledgement message to the winners have already indicated that other competing devices (with different temporary IDs) fail to their accesses. However, in the same message, additional information can be indicated for the failed devices.

As there can be more than one competing device, where one or more devices have won the contention (depending on resource usage policies) which can be indicated in Msg2, and additionally address the devices which have failed the contention without any explicit addressing. The reasons gNB cannot identify some devices can be numerous. For example, collisions or it can be as well due to channel failure, and gNB able correctly identify the device resulting in failing to positively acknowledge the device. Hence, in such scenarios, the gNB can respond with information in Msg2 that can help device re-access the inventory round. For instance, the Msg2 can target unidentified failed devices with backoff indicator (BI) e.g., information related to allowed access in same or different or adjusted round, see following proposals discussion, or exercise halting of immediate re-access for probable failed devices in order to decrease collision probability, etc.

For devices with unsuccessful random access, RAN2 to study the response message (Msg2) indicating additional information related to back-off and re-access.

## 2.5	Inventory Sessions

Once the device has been inventoried, in RFID, the device moves to another state, where it restricts the device for certain time period, before reverting back to state where inventory is allowed, which is known as persistence. First, we need to understand, if such policies are desirable in A-IoT services. In A-IoT, such policies can be implemented to restrict redundant responses to inventory triggers sent by same or multiple readers. In SA2, a similar discussion is being carried out using Transaction ID particularly in respect of multi-reader scenario. Henceforth, if there is such need, subject to A-IoT application policies, the devices can be prohibited or barred from inventory for certain time period after inventory is performed, and once this time period is over, the device is ready for inventory. See Fig. 4 which demonstrates an example, where two states are defined, one with inventory is allowed and other is with barred state. The time period for barred state can be configured based on network or application policies which can be static or configurable and can be studied. However, before working out on details, it is also important to study and confirm the validity of such use case or behaviour.

Figure 4: Once inventory is performed, the device can be barred from participating in inventory for certain period.

RFID defines multiple sessions contingent on persistence time length which restricts inventorying of a device for certain period.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In 4-step contention based RA, the ID included in Msg1 is primarily used for collision resolution.

Observation 2	In 2-step contention-based RA, the ID(s) included in Msg1 is for the purpose of collision resolution and communication device ID.

Observation 3	A fixed identifier size based payload in an initial random-access message may not be optimal as it is sensitive to contending load size and the associated the collision probability and the overall energy usage.

Observation 4	The discussion on resource allocation details for contention-based RA is subject to RAN1 progress on scheduling and resource design.

Observation 5	The discussion on resource allocation details for contention-free RA is subject to RAN1 progress on scheduling and resource design and RAN2 progress on contention-based RA design.

Observation 6	In 4-step RA, Msg4 helps with contention resolution, and provides possibility to allocate AS ID or parameters for subsequent procedures. The cost is more energy usage and increase in contention time for already energy starved devices.

Observation 7	A round can be defined as a time period over which devices can randomly select resources for its UL transmissions from a resource pool in a contention based manner.

Observation 8	RFID defines multiple sessions contingent on persistence time length which restricts inventorying of a device for certain period.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to assume reader-based selection between random-access types (2-step or 4-step) as the baseline for inventory or command use case. FFS on device based autonomous selection.

Proposal 2	RAN2 to study one RA type as a default and fallback access method.

Proposal 3	For 4-step contention-based RA, RAN2 to study pros and cons of both options regarding CR ID in Msg1 payload:

(i) Option 1: CR ID is randomly generated by device

(ii) Option 2: CR ID is generated by device based on the device ID.

Proposal 4	For 4-step contention-based RA, FFS on additional information based on Msg3 payload to be included alongside CR ID in Msg1 payload.

Proposal 5	For 2-step contention-based RA, RAN2 to study pros and cons of both options regarding Msg1 payload:

(i) Option 1: Msg1 contains only the device ID,

(ii) Option 2: Msg1 contains both device ID and additional random ID.

Proposal 6	For 2-step contention-based RA, RAN2 to study pros and cons of three options regarding contention resolution (CR) ID echoed in Msg2:

(i) Option 1: CR ID is the indicated device ID in Msg1

(ii) Option 2: CR ID is the fraction of indicated device ID in Msg1

(iii) Option 3: CR ID is the indicated random ID in Msg1.

Proposal 7	RAN2 to discuss the feasibility of multiple sizes of Msg1.

Proposal 8	RAN2 assumes single device based contention free RA as a baseline.

Proposal 9	RAN2 to consider multiple devices based contention free RA pending on RAN1 progress on resource allocation for multiple devices.

Proposal 10	After transmitting an initial RA message (Msg1 with/without data), a device considers the transmission unsuccessful if the device doesn’t receive a response message subsequently carrying its contention-resolution identifier (Msg2).

Proposal 11	For 4-step contention-based random access, study if Msg4 is needed to acknowledge Msg3 transmission considering the reliability requirements of the use case and energy usage cost at the devices.

Proposal 12	For handling contention resolution failure and access failure, RAN2 to study the three options:

a.	Option 1: a device which experiences contention-failure or access failure, re-accesses in the same round.

b.	Option 2: a device which experiences contention-failure or access failure, re-accesses in the next round.

c.	Option 3: the round length is adaptive. The round can be adjusted by increasing its length or terminating earlier upon detection of too high collision. More time occasions are added in current round or in the new round (if current one terminated).

Proposal 13	For devices with unsuccessful random access, RAN2 to study the response message (Msg2) indicating additional information related to back-off and re-access.

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, August 8-19, 2024