3GPP TSG-RAN WG2 Meeting #127	R2-2406787

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

# Introduction

The WID [1] has the following objective related to the procedure and configuration of LP-WUS indicating paging monitoring triggerd by LP-WUS for IDLE/INACTIVE modes:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In this contribution, we discuss the related issues considering the progress in RAN1 and RAN2 so far.

# Discussion

## Configuration Aspects of LP-WUS

### Network awareness of LP-WUS monitoring by UE

During the SI phase, the following was agreed in RAN2#124 meeting:

RAN2#124 Agreement The network Need not to be aware of whether the UE is monitoring LP-WUS or not in IDLE/INACTIVE.

If the network needs to be aware of whether the UE is monitoring LP-WUS or not in IDLE/INACTIVE, it requires the UE to enter RRC_CONNECTED and then send a dedicated RRC message, which leads to increased power consumption and signaling overhead. Thus, we suggest RAN2 confirms that the network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE modes.

Proposal 1: RAN2 confirms that the network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE.

### Signalling for configuration/enabling of LP-WUS

The following was agreed in RAN2#125bis:

RAN2#125bis Agreement The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

The LP-WUS related configuration should be cell specific since it is for IDLE/INACTIVE state. Dedicated RRC signaling increases the signalling overhead and limits the scenarios in which LP-WUS can be used. For example, for LP-WUS UEs with mobility, there is no information about the incoming UEs in a new cell and the AS dedicated signaling does not work.

Proposal 2: Dedicated RRC signaling is not supported for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

However, although the LP-WUS related configuration is cell-specific, there can be UE/group-specific control for enabling LP-WUS. If LP-WUS functionality is enabled for all the UEs supporting LP-WUS in a camped cell, for example, when a UE receives LP-WUS configuration via SIB from the camped cell, it can use LP-WUS functionality. It is simple but may lead to a lot of UEs using LP-WUS, which means the false paging probability increases and causes more transitions from LP-WUR to MR. This will reduce the power saving gain or even result in negative power saving gain considering the maximum number of subgroups for LP-WUS discussed in RAN1 won't exceed 256.

To reduce false paging probability, the network (CN or RAN) decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.). For example, the CN uses NAS signaling to indicate whether a specific UE can use LP-WUS functionality, which is similar as Rel-17 CN-assigned subgrouping method. As an alternative, without impact on CN, the RAN can indicate UEs that can use LP-WUS functionality via SIB. Once the UE is enabled to use LP-WUS functionality, it uses the LP-WUS related configuration via SIB.

Proposal 3: The network indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality, the detailed signalling is FFS.

### Content of LP-WUS related configuration

The following on LP-WUS related configuration was agreed in RAN2#126:

RAN2#126 Agreement The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

As LP-SS configuration is needed only if OOK-based receiver is supported, LP-SS configuration should be an optional parameter in the LP-WUS related configuration. For the entry/exit condition, whether LP-SS configuration should be present or not when LP-WUS related configuration is present can be discussed. However, the relevant threshold(s) for entry/exit condition must be specified. If the entry/exit condition is not configured, default threshold values can be considered.

Proposal 4: If entry/exit condition is not configured when LP-WUS configuration is provided by the network, default value of threshold(s) for entry and exit condition should be considered.

### LP-WUS entry/exit condition

The following was agreed as baseline for entry/exit condition in RAN2-126:

RAN2#126 Agreement Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

UE usually performs RRM measurements by MR as per legacy behavior. Thus, the serving cell measurements by MR are always available and good enough. As the hardware capability of MR is better than that of LP-WUR, the measurements by LP-WUR do not provide any additional benefit for LP-WUS entry decision. On the other hand, using both MR and LP-WUS measurements can make the LP-WUS entry procedure complicated. Hence, the measurement by LP-WUR is not needed for LP-WUS entry condition.

Proposal 5: The measurement by LP-WUR is not needed for LP-WUS entry condition.

After entering LP-WUS mode, the regular measurement on SSB and/or LP-SS can ensure that UE will exit LP-WUS mode immediately after the exit condition is fulfilled. For the exit condition, the baseline of LP-WUS exit condition should be that the serving cell measurement by LP-WUR (e.g., based on SSB and/or LP-SS depending on type of the receiver) is below configured threshold(s).

Proposal 6: RAN2 to consider that the serving cell measurement by LP-WUR below configured threshold(s) as the baseline for LP-WUS exit condition. No need to introduce any additional conditions.

In addition to exit condition, the UE should turn on MR when it receives LP-WUS indication to monitor paging, or has data to transmit in UL.

One important thing to consider while discussing entry/exit condition is to avoid frequent switching between LP-WUR and MR (i.e., ping-pong behaviour). Ping-pong can happen when a UE with mobility moves back and forth at the edge of LP-WUS coverage. It can also happen when the UE is stationary due to inconsistent measurement results over a short period of time (e.g., measurement deviation or instantaneous blockage of the channel). As ping-pong between LP-WUR and MR can severely impact the UE power saving gain, RAN2 needs to consider avoiding ping-pong e.g., by prolonging the decision of entry/exit condition prior to entry/exit.

Proposal 7: RAN2 to consider how to avoid ping-pong behavior for LP-WUS entry/exit procedure.

### Configuration for different types of LP-WUR

RAN1 has been discussing to support both OOK-based and OFDM-based receiver for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS for obtaining more power saving gain. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver.

Proposal 8: NW should configure whether only OFDM-based LP-WUR is supported.

## Issues for paging monitoring triggered by LP-WUS

The primary purpose of LP-WUS in RRC_IDLE/INACTIVE is to indicate paging monitoring. RAN1 made the following related agreements in RAN1#116 [2]:

RAN1#116 Agreements - For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. - It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO - Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

LP-WUS subgroup design is one of the main issues to be considered for LP-WUS paging design and RAN2 has agreed [3][4] that Rel-17 PEI subgrouping method can be reused.

RAN2#125bis Agreement The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. RAN2#126 Agreement RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

### LP-WUS and PEI

When LP-WUS and PEI are both configured and used, there are two cases of potential subgrouping types possible for LP-WUS and PEI.

Case 1 - Same type of subgrouping method used for LP-WUS and PEI: In this case, both LP-WUS and PEI use either CN assigned subgrouping or UE_ID based subgrouping. For CN assigned subgrouping, CN can ensure proper subgrouping of the LP-WUS and PEI by implementation to reduce the power consumption. For UE_ID based subgrouping, it is straightforward to support it without CN control.

Case 2 - Different type of subgrouping method used for LP-WUS and PEI: In this case, CN assigned subgrouping is applied for LP-WUS and UE_ID based subgrouping is applied for PEI and vice-versa. We do not see any functional issues other than a slight impact on the power saving gain. Hence what type of subgrouping to apply for LP-WUS and PEI can be left to NW implementation.

Proposal 9: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping are applied.

### LP-WUS subgrouping

In practice, not all UEs in a subgroup are paged. When the UEs that are not paged in the subgroup wake-up (i.e., false alarm wake-up), power consumption for these UEs increases. If LP-WUS subgrouping is used together with the existing subgrouping for PEI, the false alarm wake-up for the UEs can be reduced. In this method, referred to as two-level subgrouping, UEs in the first level subgrouping monitor LP-WUS. And the UEs in the first level subgrouping are further distributed to different subgroups in the second level subgrouping.

The two-level UE_ID based subgrouping is shown in Figure 1. UE1 and UE2 are placed in the same group assigned with group ID 1 in LP-WUS as the first-level subgrouping. Then in the second-level subgrouping in PEI, UE1 and UE2 are placed in different groups. When UE1 is paged, UE2 does not need to monitor paging based on its PEI so that its paging false alarm can be further reduced.

Figure 1: Illustration of 2-level subgrouping

We calculate paging false alarm numerically and present the results in Table 1 when different number of UEs in one PO use LP-WUS with/without monitoring PEI. A brief description of calculating paging false alarm rate is summarized in Appendix. We assume the number of subgroups for both PEI and LP-WUS is 8, the paging probability of UE is 1% and the subgrouping of LP-WUS is independent or orthogonal to PEI subgrouping. The paging false alarm rate for arbitrary UE comes from the probability that UE receives the invalid paging when the subgroup of this UE is indicated by LP-WUS to wake up to monitor legacy PO/PEI.

Table 1. Paging false alarm rate for LP-WUS with/without monitoring PEI

| Number of UEs in one PO | Paging false alarm rate |  |
| --- | --- | --- |
|  | LP-WUS only | LP-WUS + PEI |
| 32 | 74.6% | 2.9% |
| 64 | 87.1% | 6.7% |
| 96 | 91.2% | 10.4% |
| 128 | 93.3% | 13.9% |
| 256 | 96.5% | 26.5% |

As seen from the numerical results, the UE can achieve a great reduction of paging false alarm rate after receiving LP-WUS with using PEI as the second-level independent grouping.

Observation 2: If UEs in the same subgroup indicated by LP-WUS are further placed in different subgroups by PEI, false paging probability can be further reduced.

Proposal 10: When using LP-WUS and PEI together, to obtain additional power saving gain, UEs from the same LP-WUS subgroup can be divided to different PEI subgroups.

To achieve the two-level subgrouping described above, especially when UE_ID based subgrouping is used for both LP-WUS and PEI, the UE_ID based subgrouping for LP-WUS should be independent of UE_ID based subgrouping for PEI subgrouping. For UE_ID based subgrouping for LP-WUS, the formula for UE_ID based subgrouping for PEI in TS 38.304 can be taken as base line and different bits can be used for LP-WUS and PEI subgrouping by adding new parameter(s). To be more specific, the design should make sure the bits used for LP-WUS subgrouping do not overlap with the bits used for PEI subgrouping. This can be addressed by introducing a value K multiplied with N*Ns in the formula of UE_ID based subgrouping as below:

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS)

Similar to the function of N*Ns used in UE_ID based subgrouping formula of PEI, the effect of K is to separate the UEs in one LP-WUS subgroup into different PEI subgroups. Otherwise, UEs in one LP-WUS subgroup may be in the same PEI subgroup, resulting no further paging false alarm reduction by using PEI.

Proposal 11: The UE_ID based subgrouping formula in TS 38.304 can be reused for LP-WUS subgrouping with introducing new parameter(s), to ensure that the LP-WUS subgrouping can use different bits than the PEI subgrouping in the truncated UE ID, i.e.,

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

where:

- N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state;

- Ns: number of paging occasions for a PF;

- UE_ID: 5G-S-TMSI mod X_LP-WUS, X_LP-WUS = 8092 * maximum number of LP-WUS subgroups;

- subgroupsNumForUEID_LP-WUS: number of subgroups for LP-WUS UE_ID based subgrouping in a PO, which is broadcasted in system information

- K: configurable value which is equal to or larger than the maximum value of subgroupsNumForUEID for PEI;

Another issue is the number of LP-WUS subgroups. RAN1 has given the range of subgroup number as 8~256 in RAN1#117 meeting [3].

RAN1#117 Agreement For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16]. • FFS the exact value of Z For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. • FFS the exact value of X.

According to the group paging probability formula PG = 1 - 1 - pN , where p is the probability of an individual UE being paged and N is the number of UEs in one group, it can be derived that with the N increasing in one subgroup, the group paging probability PG will rise. It’s also clear from Table 1 that paging false alarm rate increases with increasing N which further impacts the power saving gain negatively. So, the design goal should be to allocate smaller number of UEs in each subgroup and to cover all the UEs in a cell, the number of subgroups should be larger. The number of subgroups can be configurable to make it more flexible for NW implementation. But the maximum number of subgroup number should be left to RAN1 to decide.

Observation 3: On the maximum number of subgroups, more bits is beneficial from RAN2 perspective to reduce the paging false rate, to achieve higher power saving.

Proposal 12: The maximum number of subgroups can be left to RAN1 to decide.

## SI reception procedure considering LP-WUS

RAN2 made the following agreement in RAN2#126 for SI reception procedure considering LP-WUS:

RAN2#126 Agreement From RAN2 perspective, no new procedure is introduced for SI reception/updates.

The agreement indicates that there will be no extra optimization introduced for UE to receive SI change when using LP-WUS. UE should reuse the legacy system information acquisition procedure to monitor short message in paging DCI for change of system information as well as ETWS and CMAS notification. SI changes from UE’s camped cell may involve critical configuration updates, e.g., LP-WUS configuration. In order to ensure that UEs monitoring LP-WUS do not experience any issues because they miss any SI updates, the consensus is that LP-WUS should be able to wake up all the UEs to receive SI update as well as ETWS/CMAS notification.

Proposal 13: LP-WUS should be able to wake up all UEs to monitor paging for SI update/ETWS/CMAS notification.

When SI changes, the network can reuse LP-WUS indicating paging monitoring and wake up UEs in all the subgroups. Then the UE will use MR to receive paging DCI that indicates SI change and acquires the updated SI in the following modification period. As in NB-IoT, a straight forward way is to introduce a common LP-WUS and all the UEs using LP-WUR can monitor a common LP-WUS occasion. If the common LP-WUS is received, all the UEs irrespective of their subgroup will transit to MR.

Proposal 14: Introduce a common LP-WUS to wake up all the UEs in RRC_IDLE/INACTIVE to monitor paging for SI update/ETWS/CMAS notification.

## LP-WUS prioritization in cell reselection

In practical deployment, we cannot guarantee that all the frequencies support LP-WUS. It would be beneficial for UEs supporting LP-WUS to camp on the frequency supporting LP-WUS as often as possible in order to obtain more power saving gain. For example, NW broadcasts the frequencies of neighbor cells that supporting LP-WUS, so that UE can prioritize the neighbor cells that supports LP-WUS during cell reselection procedure.

In this case, some enhancements of cell (re)selection can be also considered, e.g., LP-WUS UEs may regard the frequency supporting LP-WUS to be the higher priority. Especially, camping on a lower frequency band/carrier is beneficial due to smaller path loss to obtain better coverage performance, thus a LP-WUS capable UE can take a lower band/carrier with LP-WUS deployment as higher priority.

Proposal 15: UE supporting LP-WUS prioritizes the frequency supporting LP-WUS in the cell (re)selection procedure.

# Conclusion

In this contribution, we discuss procedure and configuration of LP-WUS for IDLE/INACTIVE modes, and have the following proposals.

Configuration Aspects of LP-WUS

Proposal 1: RAN2 confirms that the network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE.

Proposal 2: Dedicated RRC signaling is not supported for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Proposal 3: The network indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality, the detailed signalling is FFS.

Proposal 4: If entry/exit condition is not configured when LP-WUS configuration is provided by the network, default value of threshold(s) for entry and exit condition should be considered.

Proposal 5: The measurement by LP-WUR is not needed for LP-WUS entry condition.

Proposal 6: RAN2 to consider that the serving cell measurement by LP-WUR below configured threshold(s) as the baseline for LP-WUS exit condition. No need to introduce any additional conditions.

Proposal 7: RAN2 to consider how to avoid ping-pong behavior for LP-WUS entry/exit procedure.

Proposal 8: NW should configure whether only OFDM-based LP-WUR is supported.

Issues for paging monitoring triggered by LP-WUS

Proposal 9: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping are applied.

Proposal 10: When using LP-WUS and PEI together, to obtain additional power saving gain, UEs from the same LP-WUS subgroup can be divided to different PEI subgroups.

Proposal 11: The UE_ID based subgrouping formula in TS 38.304 can be reused for LP-WUS subgrouping with introducing new parameter(s), to ensure that the LP-WUS subgrouping can use different bits than the PEI subgrouping in the truncated UE ID, i.e.,

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

where:

- N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state;

- Ns: number of paging occasions for a PF;

- UE_ID: 5G-S-TMSI mod X_LP-WUS, X_LP-WUS = 8092 * maximum number of LP-WUS subgroups;

- subgroupsNumForUEID_LP-WUS: number of subgroups for LP-WUS UE_ID based subgrouping in a PO, which is broadcasted in system information

- K: configurable value which is equal to or larger than the maximum value of subgroupsNumForUEID for PEI;

Proposal 12: The maximum number of subgroups can be left to RAN1 to decide.

SI reception procedure considering LP-WUS

Proposal 13: LP-WUS should be able to wake up all UEs to monitor paging for SI update/ETWS/CMAS notification.

Proposal 14: Introduce a common LP-WUS to wake up all the UEs in RRC_IDLE/INACTIVE to monitor paging for SI update/ETWS/CMAS notification.

LP-WUS prioritization in cell reselection

Proposal 15: UE supporting LP-WUS prioritizes the frequency supporting LP-WUS in the cell (re)selection procedure.

# References

RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

RAN1#116 chairman notes;

RAN2#125-bis chairman notes;

RAN2#126 chairman notes;

RAN1#117 chairman notes;

# Appendix

## Calculation of paging false alarm rate

- In this appendix, we provide a brief description on how we obtain the results of paging false alarm rate.

- Case 1: LP-WUS only

- For UEs that are paged in a LP-WUS subgroup, the conditional probability that n UEs are paged can be expressed as:

P1 = Pn UEs are pagedSubgroup is paged = CNLn pn(1-p)NL-n1 -(1-p)NL ,

- where p is the per-UE paging probability and NL is number of UEs in the subgroup of LP-WUS. Among NL UEs, there are n UEs that are paged, the conditional probability that a target UE is included can be expressed as:

P2 = PTarget UE is pagedn of NL UEs are paged = CNL-1n-1 CNLn = nNL

- Combining the above 2 equations, we can obtain the probability that a target UE is paged in a certain subgroup as following:

Ptarget =  n=1NLP1×P2 =n=1NLCNLn pn(1-p)NL-n1 -(1-p)NL×nNL

- The paging false alarm rate of a certain UE after being indicated by LP-WUS to monitor PO can be expressed as:

- PFAR, Case 1=1-  Ptarget =1-n=1NLCNLn pn(1-p)NL-n1 -(1-p)NL×nNL

- Case 2: LP-WUS with PEI

- The difference between Case 1 and 2 is that for Case 2, once UEs are indicated by LP-WUS to wake up, they will be filtered by PEI with another independent subgrouping. The probability of one certain PEI subgroup being indicated for paging can be regarded as:

PG= P(One certian PEI Subgroup is paged) = 1 - 1 - pNP,

- where NP is the number of UEs in the subgroup number of PEI. Similar to Case1, the paging false alarm rate of a certain UE after being indicated by LP-WUS to monitor PEI and PO can be expressed as:

- PFAR, Case 2= PG×(1-  Ptarget) =( 1 - 1 - pNP)×1-n=1NLCNLn pn(1-p)NL-n1 -(1-p)NL×nNL

- It can be shown that compared to the formula of Case 1, UE monitoring LP-WUS with PEI can obtain a reduction of 1 - pNP off in FAR.