3GPP TSG-RAN WG2 Meeting #127                                       R2-2406790

Maastricht, Netherlands, Aug 19th – 23th, 2024

Source:        ZTE Corporation, Ericsson

Title:           Discussion on DL MAC CE in Rel-17 and Rel-18 SP SRS in RRC_INACTIVE

Agenda item:   6.4

Document for:  Discussion and Decision

- Introduction

The Rel-17 CR R2-2404625[1] and Rel-18 CR R2-2404626[2] submitted in RAN2#126 meeting[3] are postponed due to some companies views:

R2-2404625 Correction on SP SRS activation/deactivation MAC CE ZTE Corporation CR Rel-17 38.321 17.8.0 1840 - F NR_pos_enh-Core - Postponed R2-2404626 Correction on SP SRS activation deactivation MAC CE(R18) ZTE Corporation CR Rel-18 38.321 18.1.0 1841 - F NR_pos_enh-Core - Postponed Discussion: Huawei understand the idea but think the correction does not completely resolve the problem, because there is also a cell ID. ZTE understand that the network would use the last serving cell ID. Huawei think SP-SRS in RRC_INACTIVE in Rel-17 is not a big problem, because the RRCRelease can just configure periodic SRS. Qualcomm note that there is subsequent DL data transfer; the UE can send MO-LR via SDT and SP-SRS can be activated with subsequent DL data. They also wonder if the cell ID by itself is enough. ZTE intended that the cell ID would use the last serving cell, but they could revise the CR to make a similar change to the cell ID. Intel think there is nothing wrong with the cell ID, just the BWP ID, and they think changing the behaviour for the cell ID might create a problem in Rel-18. Huawei wonder if we will reuse this MAC CE in Rel-18; Intel understand that we will not. ZTE suggest that we change BWP ID now and see how the cell ID should be handled in light of the MAC CE decisions for aggregated SRS.

Where in the discussion, the following cases related with MAC CE/inactive SRS is mentioned:

- Reuse old DL MAC CE in Rel-17 INACTIVE SRS, within last serving cell;

- Reuse old DL MAC CE in Rel-18 INACTIVE SRS, within multiple cells;

This paper discusses the above cases and provides analysis on Rel-17 CR and Rel-18 CR.

- Discussion

Reuse DL MAC CE in Rel-17 INACTIVE SP-SRS

The following are agreed during Rel-17 RAN2 discussion, SRS in RRC INACTIVE agenda:

RAN2#116 agreement SP Positioning SRS Activation/Deactivation MAC CE is reused for triggering SRSp transmission in RRC_INACTIVE. (12/12) RAN2#117 agreement Proposal 4 (modified): Support the following options for activation of SP-SRSp transmission in RRC INACTIVE: - Option a: If there is ongoing SDT, the network can send SRS activation command to the UE in INACTIVE. - Option b: Send the Activation MAC CE along with the SRSp configuration when gNB releases the UE to RRC_INACTIVE Proposal 5 (modified): Support the following for deactivation of SP-SRSp transmission in RRC INACTIVE: - If gNB chooses to send the SP-SRSp deactivation command to the UE in INACTIVE, gNB can send SP-SRSp deactivation command to the UE if there is ongoing SDT. - If gNB chooses not to send the SP-SRSp deactivation command to the UE in RRC_INACTIVE, no additional mechanism is specified (i.e. the gNB can only wait for the TA timer to expire)

Issue of Positioning SRS Resource Set's BWP ID

The SRS configuration is within UE’s last serving cell, and the configured SRS frequency range can be within initial BWP or outside the initial BWP. The initial BWP has an ID 0. When outside the initial BWP, the BWP of the SRS configuration in RRC Release-Suspend Config is explicitly indicated via frequency parameters, not via BWP ID. So BWP ID in DL MAC CE is not valid.

SRS-PosRRC-InactiveConfig-r17 ::= SEQUENCE { srs-PosConfigNUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r17 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r17 BWP OPTIONAL, -- Need S bwp-SUL-r17 BWP OPTIONAL, -- Need S inactivePosSRS-TimeAlignmentTimer-r17 TimeAlignmentTimer OPTIONAL, -- Need M inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17 OPTIONAL -- Need M }

UE should ignore the BWP ID field when the DL MAC CE is used in RRC_INACTIVE. The corresponding Rel-17 CR is provided in R2-2406788.

Proposal 1: For Rel-17 SRS in RRC_INACTIVE, UE should ignore the BWP ID field when the SRS is configured outside initial BWP. RAN2 to agree Rel-17 CR in R2-2406788.

Issue of Positioning SRS Resource Set's Cell ID

The problem is whether current interpretation of Positioning SRS Resource Set's Cell ID in the DL MAC CE is also valid in RRC INACTIVE?

When UE enters RRC_INACTIVE (per UE level control), the UE stores the cell configuration and BWP configuration. Only when UE initiates RRC Resume, UE release the MCG Scell configuration when UE does not support corresponding capability. That is to say, UE always maintain MCG Pcell and Scell configuration before RRC Resume. See below 38.331 explanation:

- TS38.331-i20 - 5.3.13 RRC connection resume - <omitted> - 5.3.13.2 Initiation Upon initiation of the procedure, the UE shall: <omitted> 1> if the UE is in NE-DC or NR-DC: 2> if the UE does not support maintaining SCG configuration upon connection resumption: 3> release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored; 1> if the UE does not support maintaining the MCG SCell configurations upon connection resumption: 2> release the MCG SCell(s) from the UE Inactive AS context, if stored;

The maintained MCG Pcell is the last serving cell of the UE, with the serving cell index =0, see below 38.331 explanation:

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

Observation 1: When UE is in RRC_INACTIVE, UE always maintains/stores MCG Pcell and Scell configuration before RRC Resume, and last serving cell index equals to 0.

Current Positioning SRS Resource Set's Cell ID is defined as below:

Positioning SRS Resource Set's Cell ID: This field indicates the identity of the Serving Cell, which contains activated/deactivated SP Positioning SRS Resource Set. If the C field is set to 0, this field also indicates the identity of the Serving Cell which contains all resources indicated by the Spatial Relation for Resource IDi fields, if present. The length of the field is 5 bits;

Since UE stores serving cell configuration from RRC_CONNECTED, so if the cell ID is not interpreted, UE may activate/deactivate wrong SRS configuration according to the indicated serving cell ID.

In Rel-17, the SRS configuration in RRC INACTIVE is only provided in the last serving cell, so the cell ID should be re-interpreted as last serving cell ID for the case the DL MAC CE is used in RRC_INACTIVE. The last serving cell index is 0.

Additionally for the Resource Serving Cell ID field(s) and Resource BWP ID field(s) in Spatial Relation for Resource IDi:

- If C is set to 1, spatial relation field should contain cell ID/BWP ID. If UE is in RRC INACTIVE, UE still remembers the Pcell/Scell configuration and their associated cell index/BWP index. So the cell ID/BWP ID in spatial relation field actually indicates the cell ID/BWP ID that UE used in RRC_CONNECTED and UE stored when entering RRC_INACTIVE, which should be valid.

- If C is set to 0, spatial relation field does contain cell ID/BWP ID and take Positioning SRS Resource Set's Cell ID as reference. According to the above, gNB should set the Positioning SRS Resource Set's Cell ID to be last serving cell ID and UE should ignore BWP ID. If last serving cell ID does not configure such CSI-RS/SRS for spatial relation or UE does not store the configuration, gNB can always set C as 1, i.e., explicitly indicate which serving cell/BWP contains the indicated RS for spatial relation reference.

The reason why UE should not directly ignore the Positioning SRS Resource Set's Cell ID is that, when C is set to 0, the cell of the RS for spatial relation should also refer to the Positioning SRS Resource Set's Cell ID. If the field is just ignored by UE, the UE cannot use the corresponding spatial relation info in the DL MAC CE.

Therefore, R17 CR should say the Positioning SRS Resource Set's Cell ID is the last serving cell ID. The CR is in R2-2406788.

Proposal 2: For Rel-17 SRS in RRC_INACTIVE, Positioning SRS Resource Set's Cell ID is the last serving cell ID. RAN2 to agree Rel-17 CR in R2-2406788.

Reuse DL MAC CE in Rel-18 INACTIVE SP-SRS

RAN2#126 has additionally achieved the following agreement:

RAN2#126 agreement: For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig (non-aggregated cases), the transmission of SP SRS is activated by MAC CE as legacy.

For Rel-18 non-pre-configured SRS and pre-configured SRS in RRC_INACTIVE:

- When last serving cell configures UE with one validity area and corresponding one SP SRS configuration, the UE needs to receive DL activation MAC CE in one of the configured cell, then the UE can start transmit SP SRS in the validity area.

- When last serving cell configures UE with several validity areas and corresponding several preconfigured SP SRS configurations, the UE needs to send SRS activation request, and the UE needs to receive DL activation MAC CE in one of the configured cell in a validity area, then then UE can start to send preconfigured SP SRS in the corresponding validity area.

When the UE is in RRC_INACTIVE, the UE can only receive the DL MAC CE in its camping cell via SDT. UE can only access to one single camping cell at a time, and CA is not supported. So when this MAC CE is reused for inactive SRS transmission across cells, the DL MAC CE can only be sent by the current camping cell.

Issue of Positioning SRS Resource Set's BWP ID

When the DL MAC CE is used to activate/deactivate the Rel-18 SRS in RRC_INACTIVE, the BWP ID is invalid since the Rel-18 SRS configuration also does not provide any BWP ID, only BWP parameters (for outside initial BWP case):

SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE { srs-PosConfigValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellsInVA-r18)) OF CellIdentity, srs-PosConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S areaValidityTA-Config-r18 AreaValidityTA-Config-r18 OPTIONAL, -- Need R ... }

Proposal 3: When the DL MAC CE is used to activate/deactivate the Rel-18 SRS in RRC_INACTIVE, UE should ignore the BWP ID field in the DL MAC CE when SRS is configured outside initial BWP. Agree the CR in R2-2406789.

Issue of Positioning SRS Resource Set's Cell ID

The legacy Positioning SRS Resource Set's Cell ID in the DL MAC CE denotes ‘This field indicates the identity of the Serving Cell, which contains activated/deactivated SP Positioning SRS Resource Set’. Since the configured SRS in multiple cells in RRC_INACTIVE are not ‘serving cell’, and UE stores serving cell configuration from RRC_CONNECTED (similar like the analysis in section 2.1.2), the Positioning SRS Resource Set's Cell ID should also be re-interpreted for Rel-18.

From our point of view, there are different potential ways of re-interpreting Positioning SRS Resource Set's Cell ID for Rel-18 SRS across multiple cells:

- Last serving cell: where the UE got the Rel-18 SRS (pre-)configuration, last serving cell ID equals to 0.

- VA where the camping cell is in: the current RRC has maximum VA number as 16, it can be denoted by the existing 5 bits. However it is not that necessary since UE already stores the cell&SRS relationship. Once UE knows the current camping cell’s PCI/NCGI, UE knows the corresponding SRS configuration.

- Cell identity of the camping cell: however since the Positioning SRS Resource Set's Cell ID field size is only 5 bits, it cannot embed a PCI or NCGI of the current camping cell.

- gNB can randomly set the cell ID, and UE should ignore it when receiving: same objection with Rel-17 discussion above that, when C is set to 0, the cell of the RS for spatial relation should also refer to the Positioning SRS Resource Set's Cell ID. If the field is just ignored by UE, the UE cannot use the corresponding spatial relation info in the DL MAC CE.

Based on above, bullet 2/3/4 are either not workable or unnecessary. So for simplicity and consistency between Rel-17 and Rel-18, the Positioning SRS Resource Set's Cell ID can also be last serving cell ID. In this way, the Rel-17 CR and Rel-18 CR have exactly same changes. The Rel-18 CR is provided in R2-2406789.

Proposal 4: When the DL MAC CE is used to activate/deactivate the Rel-18 SRS in RRC_INACTIVE, support the cell ID to be the last serving cell where the UE gets the Rel-18 SRS configuration. RAN2 to agree the CR R2-2406789.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: When UE is in RRC_INACTIVE, UE always maintains/stores MCG Pcell and Scell configuration before RRC Resume, and last serving cell index equals to 0.

Rel-17 CR

Proposal 1: For Rel-17 SRS in RRC_INACTIVE, UE should ignore the BWP ID field when the SRS is configured outside initial BWP. RAN2 to agree Rel-17 CR in R2-2406788.

Proposal 2: For Rel-17 SRS in RRC_INACTIVE, Positioning SRS Resource Set's Cell ID is the last serving cell ID. RAN2 to agree Rel-17 CR in R2-2406788.

Rel-18 CR

Proposal 3: When the DL MAC CE is used to activate/deactivate the Rel-18 SRS in RRC_INACTIVE, UE should ignore the BWP ID field in the DL MAC CE when SRS is configured outside initial BWP. Agree the CR in R2-2406789.

Proposal 4: When the DL MAC CE is used to activate/deactivate the Rel-18 SRS in RRC_INACTIVE, support the cell ID to be the last serving cell where the UE gets the Rel-18 SRS configuration. RAN2 to agree the CR R2-2406789.

- Reference

- R2-2404625	Correction on SP SRS activation/deactivation MAC CE	ZTE Corporation	CR	Rel-17	38.321	17.8.0	1840	-	F	NR_pos_enh-Core   2024-05

- R2-2404626	Correction on SP SRS activation deactivation MAC CE(R18)	ZTE Corporation	CR	Rel-18	38.321	18.1.0	1841	-	F	NR_pos_enh-Core  2024-05

- R2-2405704	Report from session on positioning and sidelink relay	Session chair (MediaTek)	discussion 2024-05