3GPP TSG-RAN WG2 Meeting #127                                       R2-2406794

Maastricht, Netherlands, Aug 19th – 23th, 2024

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID[1] of Rel-19 SBFD related to random access is highlighted as follows:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

This contribution provides our view on random access procedures in SBFD technique.

- Discussion

Supported RA scenarios in SBFD

There is a list of RA triggering events (specified in TS38.300[2]) summarized in the below table:

| RA scenario | RRC state | CBRA/CFRA | initiator |
| --- | --- | --- | --- |
| Initial access from RRC_IDLE | IDLE | CBRA | UE |
| RRC Connection Resume procedure from RRC_INACTIVE | INACTIVE | CBRA | UE |
| Request for Other SI | IDLE/INACTIVE/CONNECTED | CBRA（Msg3-based）/CFRA (Msg1-based) | UE |
| SDT in RRC_INACTIVE | INACTIVE | CBRA | UE |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | INACTIVE | CBRA | UE |
| RRC Connection Re-establishment procedure | CONNECTED | CBRA | UE |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | CONNECTED | CBRA/CFRA(PDCCH order) | gNB |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | CONNECTED | CBRA | UE |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; | CONNECTED | CBRA | UE |
| Handover, except for when RACH-less HO is configured; | CONNECTED | CFRA | gNB |
| SR failure | CONNECTED | CBRA | UE |
| Explicit request by RRC upon synchronous reconfiguration; | CONNECTED | CFRA | gNB |
| Beam failure recovery | CONNECTED | CBRA/CFRA | UE |
| To establish time alignment for a primary or a secondary TAG | CONNECTED | CFRA (PDCCH order) | gNB |
| Consistent UL LBT failure on SpCell | CONNECTED | CBRA | UE |
| Early UL synchronization with an LTM candidate cell | CONNECTED | CFRA(PDCCH order) | gNB |
| RACH-based LTM cell switch | CONNECTED | CBRA(RRC based)/CFRA(MAC CE based, RRC based) | UE |

In addition, the WID indicates SBFD-aware UE should be able to perform RA in SBFD resources under all of RRC states (RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode). Also in RAN1#116bis meeting[3], RAN1 has agreed to support both CBRA and CFRA for SBFD-aware UE to perform RA:

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

So from the RAN1’s agreement and RAN2’s trigger events, technically it is feasible to support all kinds of RA in SBFD resources for SBFD UE.

From motivation perspective, the benefit of supporting RA in SBFD resources can be:

- Reduce latency of RA procedure: supporting RA in SBFD resources provides more RA chances or earlier RA chances for SBFD-aware UE, since the number of D slot/symbol is usually larger than U slot/symbol in a TDD system, more ROs in legacy D slot/symbol can be well utilized by SBFD-aware UE;

- Increase UL coverage: Separate RACH configuration may provide a whole different RACH parameters for SBFD UE, e.g., longer preamble format. This helps to increase the UL coverage of SBFD-aware UE.

We do not observe any RA scenarios that does not need to reduce latency or increase UL coverage, for example, SBFD-aware UE wants to perform BFR, or SBFD-aware UE wants to perform re-synchronization. Therefore, there should be no restriction on the supported RA scenarios for RA in SBFD resource.

Proposal 1: RAN2 to support random access in SBFD symbols for all the RA scenarios.

RACH configuration dedicated for SBFD-aware UE

RAN1#117 meeting[4] has made the following agreement:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

It can be seen that there will be at least one newly introduced RACH configuration(no matter option 1 or option 2) which contains ROs in SBFD resource.

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP is allocated on NUL carrier and/or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid since NUL is not available in D slot/symbol in TDD system; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE should perform random access in SBFD symbols, the ROs should occupy SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than DL carrier/NUL to provide larger UL coverage, in order to cover resources in UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 1. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Proposal 2: RAN2 confirm that RACH configuration for SBFD-aware UE should only be configured in NUL carrier.

Legacy RACH configuration is configured in each UL BWP, and the frequency range of the ROs provided by legacy RACH configuration is within the corresponding UL BWP, and the ROs provided by legacy RACH configuration should be invalid in DL slot/symbols. Different from legacy RACH configuration, SBFD RACH configuration should provide ROs in SBFD resource (e.g., in legacy DL slot/symbols and in the UL subband). Since UE can only transmit Msg1 inside UL subband in SBFD symbol, the ROs are valid if the frequency range of the ROs provided by SBFD RACH configuration is within configured UL subband.

To achieve such frequency overlapping, the SBFD RACH configuration can be provided directly within the UL subband configuration. However in such case, the ROs can be only within UL subband, not within normal BWP, so it restricts SBFD-aware UE from initiating RA in non-SBFD resources, which reduces lots of RA chances. In addition, UL subband configuration is not fully settled by RAN1 and the detail is unknown.

Another way is to configure SBFD RACH configuration within UL BWP, where the legacy RACH configuration is provided. It is a common understanding and a more simple solution for RAN2. In this case, the frequency start of the RO may be re-interpreted to ensure the frequency overlapping between ROs and UL subband. Since RAN1 is discussing this, RAN2 should wait for RAN1’s decision on the details.

Observation 1: Since UE can only transmit Msg1 inside UL subband in SBFD symbol, the ROs are valid if the frequency range of the ROs provided by SBFD RACH configuration is within configured UL subband.

Proposal 3: RAN2 confirm that RACH configuration for SBFD-aware UE should be configured within UL BWP. Detailed re-interpretation of RACH parameters should wait for RAN1’s decision.

SBFD-aware UE can use the newly introduced SBFD RACH configuration and the legacy RACH configuration to initiate RA, legacy UE can only use the legacy RACH configuration to initiate RA. So when gNB does not know the UE type in the cell, gNB needs to broadcast both SBFD RACH configuration and legacy RACH configuration to suit the needs for SBFD-aware UE and legacy UE. At this case, which RACH configuration SBFD-aware UE should firstly choose to perform RA is worth to be discussed and settled by RAN2. The most simple way is that, when SBFD-aware UE is provided with SBFD RACH configuration, SBFD RACH configuration should be firstly choosen.

Observation 2: When gNB does not know the UE type in the cell, gNB needs to broadcast both SBFD RACH configuration and legacy RACH configuration to suit the needs for SBFD-aware UE and legacy UE.

Proposal 4: If RACH configuration for SBFD-aware UE is provided by gNB, SBFD-aware UE should firstly choose the RACH configuration to perform RA.

SBFD RACH configuration is optionally provided, and legacy RACH configuration is always provided. When UE use the RACH resource from SBFD RACH configuration to initiate RA, the UE should be able to fallback to legacy SBFD RACH configuration to acquire more RA chances/attempts, similar like 2-step RACH configuration fallback to 4-step RACH configuration.

Proposal 5: Support SBFD-aware UE to fallback from SBFD RACH configuration to legacy RACH configuration.

Single/separate SBFD RACH configuration

Single RACH configuration(i.e., option 1) and separate RACH configuration (i.e., option 2) share similar principle that the RACH configuration contains ROs in SBFD resource, which is dedicated for SBFD-aware UE to use. From specification perspective, single RACH configuration(i.e., option 1) and separate RACH configuration (i.e., option 2) are two different ways of configuration/signaling. The main difference is:

- Single RACH configuration means SBFD-aware UE reuse the legacy RACH configuration to determine RO location/SSB mapping/power control/etc., but the valid RO of SBFD-aware UE may be different from the valid RO of legacy UE. If RAN1 defines different validation rules in single RACH configuration, SBFD-aware UE will have valid ROs in SBFD symbol while legacy UE cannot have such valid ROs.

- Separate RACH configuration provides independent basic RACH parameters compared with the legacy RACH configuration. So the valid RO of derived from separate RACH configuration and legacy RACH configuration can be totally in different pattern.

See Figure 2, blue ROs are valid ROs of corresponding RACH configuration.

Figure 2. Example difference between legacy RACH configuration/single RACH configuration/separate RACH configuration

RAN1’s agreement indicates ‘UE is not required to support both options’, which means from UE capability perspective, UE can support only option 1, or only option 2, or support both. So before gNB knows UE’s capability, gNB is better to provide both single RACH configuration and separate RACH configuration to suit different UE’s capabilities. However this should not be a strict configuration restriction for gNB, since gNB will always provide legacy basic RACH configuration for all UEs to initiate RA. Therefore, RAN2 should specify both options in RRC signaling with OPTIONAL mark.

Proposal 6：RAN2 should specify both single RACH configuration (i.e., option 1) and separate RACH configuration (i.e., option 2) in RRC signaling with OPTIONAL mark.

Based on above, if gNB provides both single RACH configuration and separate RACH configuration, UE should know which set of RACH configuration to use. One simple way is that gNB configures the priority of the two sets RACH configurations.

Proposal 7: RAN2 should specify criteria for UE to choose between single SBFD RACH configuration and separate SBFD RACH configuration, if both configured.

gNB configures different RACH configuration for different RA scenarios. To be specific:

- For UE performing CBRA in RRC_IDLE/INACTIVE, the legacy RACH configuration is provided in SIB1 - ServingCellConfigCommonSIB - UplinkConfigCommonSIB - initialUplinkBWP-BWP - UplinkCommon - RACH-ConfigCommon;

- For UE performing CBRA in RRC_CONNECTED, the legacy RACH configuration is provided in ServingCellConfig - UplinkConfig - BWP-Uplink - BWP-UplinkCommon - RACH-ConfigCommon;

- For UE performing CFRA for SI request, the legacy RACH configuration is provided in SIB1 - SI-SchedulingInfo - SI-RequestConfig;

- For UE performing CFRA for BFR, the legacy RACH configuration is provided in BWP-UplinkDedicated - BeamFailureRecoveryConfig;

- For UE performing CFRA for ReconfigurationWithSync, the legacy RACH configuration is provided in ReconfigurationWithSync - rach-ConfigDedicated;

- For UE performing CFRA for LTM, the legacy RACH configuration is provided in RRCReconfiguration - LTM-Config -  LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig;

Single RACH configuration will just reuse some of the legacy ROs and does not change RO pattern/grid/SSB-RO mapping rules of the legacy RACH configuration, however separate RACH configuration provides a totally new/independent RACH configuration (e.g., new RACH-ConfigGeneric and SSB-RO mapping rule) parallel to the legacy RACH configuration. Therefore, separate RACH configuration requires more spec effort compared to single RACH configuration.

If RA in SBFD resources is supported for all RA scenarios, RAN2 needs to consider the necessity and the spec effort of introducing both options together with every legacy RACH configuration, as mentioned above. E.g., single RACH configuration can be supported in every RA scenarios, however separate RACH configuration may not be needed in some of the RA scenarios due to the larger spec impact and unclear motivation.

Proposal 8: RAN2 to discuss whether all the RA scenarios should support both single RACH configuration and separate RACH configuration.

Early identification of SBFD-aware UE

RAN1#116[5] agreed the following:

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: Msg2[/Msg4 PDSCH] reception in DL subband(s) Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping Msg3 repetition Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

Legacy Msg3 should only be scheduled in U slots if RA is initiated in NUL. This agreement indicates whether Msg3 can be scheduled/transmitted in SBFD resources needs further discussion. From RAN2 perspective, using SBFD resource to transmit Msg3 can increase the UL transmission chance compared to only using legacy U symbol, more importantly, it can also reduce latency of the whole RA procedure, which is also the motivation of sending Msg1 in SBFD resources. Therefore, sending Msg3 in SBFD resource should be supported.

Observation 3: Using SBFD resource to transmit Msg3 can reduce latency and increase the UL transmission chance compared to only using legacy U symbol, more importantly, it can also reduce latency of the whole RA procedure.

Proposal 9: RAN2 supports that SBFD-aware UE is allowed to send Msg3 in SBFD resource.

The system consists of non-SBFD-aware UEs and SBFD-aware UEs. When in CBRA, both kinds of UEs will send Msg1 according to the broadcast RACH configuration. If gNB wants to send the Msg2 RAR containing the Msg3 UL grant on SBFD resources to SBFD-aware UE, gNB needs to recognize SBFD-aware UE in the first place, i.e., during Msg1 transmission.

Figure 3. SBFD-aware UE early identification in CBRA

The above early identification can be realized via several ways in the current specification, e.g., RACH partition, or explicitly configure separate preamble indexes, etc. RAN2 should further discuss the method to achieve early identification.

Proposal 10: RAN2 to support early identification of SBFD aware-UE during Msg1 transmission.

- Conclusion

In this contribution, we propose the following observation and proposals:

Supported RA scenarios in SBFD

Proposal 1: RAN2 to support random access in SBFD symbols for all the RA scenarios.

SBFD RACH configuration

Observation 1: Since UE can only transmit Msg1 inside UL subband in SBFD symbol, the ROs are valid if the frequency range of the ROs provided by SBFD RACH configuration is within configured UL subband.

Observation 2: When gNB does not know the UE type in the cell, gNB needs to broadcast both SBFD RACH configuration and legacy RACH configuration to suit the needs for SBFD-aware UE and legacy UE.

Proposal 2: RAN2 confirm that RACH configuration for SBFD-aware UE should only be configured in NUL carrier.

Proposal 3: RAN2 confirm that RACH configuration for SBFD-aware UE should be configured within UL BWP. Detailed re-interpretation of RACH parameters should wait for RAN1’s decision.

Proposal 4: If SBFD RACH configuration is provided by gNB, SBFD-aware UE should firstly choose SBFD RACH configuration to perform RA.

Proposal 5: Support SBFD-aware UE to fallback from SBFD RACH configuration to legacy RACH configuration.

Single/separate SBFD RACH configuration

Proposal 6：RAN2 should specify both single RACH configuration (i.e., option 1) and separate RACH configuration (i.e., option 2) in RRC signaling with OPTIONAL mark.

Proposal 7: RAN2 should specify criteria for UE to choose between single SBFD RACH configuration and separate SBFD RACH configuration, if both configured.

Proposal 8: RAN2 to discuss whether all the RA scenarios should support both single RACH configuration and separate RACH configuration.

Early identification of SBFD-aware UE

Observation 3: Using SBFD resource to transmit Msg3 can reduce latency and increase the UL transmission chance compared to only using legacy U symbol, more importantly, it can also reduce latency of the whole RA procedure.

Proposal 9: RAN2 supports that SBFD-aware UE is allowed to send Msg3 in SBFD resource.

Proposal 10: RAN2 to support early identification of SBFD aware-UE during Msg1 transmission.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06

- TS38.300-i20, ‘NR and NG-RAN Overall Description; Stage 2’, Release 18, 2024-06

- R1-2403789, Final Summary on SBFD random access operation, CMCC, 2024-04

- R1-2405656, Final Summary on SBFD random access operation, CMCC, 2024-05

- R1-2401678, Final Summary on SBFD random access operation, CMCC, 2024-02