3GPP TSG-RAN WG2 Meeting #127                                       R2-2406795

Maastricht, Netherlands, Aug 19th – 23th, 2024

Source:        ZTE Corporation

Title:           Discussion on CLI measurement in SBFD

Agenda item:   8.11.3

Document for:  Discussion and Decision

- Introduction

The WID[1] of Rel-19 SBFD related to CLI is highlighted as follows:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

This contribution provides our view on UE-to-UE/gNB-to-gNB CLI measurements caused by SBFD, which may have some RAN2 impact.

- Discussion

UE-to-UE CLI measurement

In the latest WID and RAN1#117 meeting, it is agreed to use L1 CLI structure to derive UE-to-UE interference caused by SBFD in order to achieve more frequent measurement and reporting.

RAN1#116 meeting’s[2] agreement indicates that:

Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.

That is to say the L1-SRS-RSRP or L1-CLI-RSSI measurement should be made within DL active BWP, so the measurement gap is not needed for L1 CLI measurement.

Proposal 1: For UE-to-UE L1 CLI measurement, RAN2 confirms that L1 CLI measurement does not need measurement gap.

Another issue is that, the agreement does not say ‘performed within the active DL BWP’ should be a gNB configuration restriction or measurement restriction, i.e., whether the spec restricts that gNB should allocate the L1 CLI resource to be within victim UE’s DL BWP, or spec does not restrict gNB’s configuration, it is the UE that should further determine whether/which frequency part of the L1 CLI resource should be measured.

From our perspective, it is better to let UE to determine the measurement behaviour rather than restricting gNB’s configuration, since the L1 CLI resource is configured per serving cell, however UE should make measurement per DL active BWP. Furthermore, the most severe interference occurs within the overlapping part of frequency, so we think it is enough for UE to measure the L1 CLI resource in the frequency overlapping part with the DL active BWP.

Proposal 2: For UE-to-UE L1 CLI measurement, RAN2 supports victim UE to measure the L1 CLI resource in the frequency overlapping part with the DL active BWP.

The L1 CLI measurement can be configured per serving cell in the legacy L1 CSI framework. If the victim UE supports DC, it is natural to support MN and SN to both configure L1 CLI measurement to the UE. For L3 CLI measurement, TS37.340[3] has the following restriction:

TS37.340: CLI measurements can be configured for NR cells in all MR-DC options. In EN-DC and NGEN-DC, only the SN can configure CLI measurements. In NE-DC, only the MN can configure CLI measurements. In NR-DC, both the MN and the SN can configure CLI measurements, and the MN informs the SN about the maximum number of CLI measurement resources that can be configured by the SN to ensure that the total number of CLI measurement resources does not exceed the UE capabilities.

Since L1 CLI measurement is also a NR feature, we think the same principle can be applied here, i.e., in EN-DC and NGEN-DC, only the SN can configure L1 CLI measurements. In NE-DC, only the MN can configure L1 CLI measurements. In NR-DC, both the MN and the SN can configure L1 CLI measurements.

Proposal 3: RAN2 to support that, in EN-DC and NGEN-DC, only the SN can configure L1 CLI measurements. In NE-DC, only the MN can configure L1 CLI measurements. In NR-DC, both the MN and the SN can configure L1 CLI measurements.

gNB-to-gNB CLI measurement

gNB-to-gNB CLI measurement is mainly RAN3 scope, with the following potential enhancements:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request

The above information can be exchanged between gNBs via XnAP message. However we think for the second bullet, measurement resource configuration should be embedded in RAN2 inter-node message MeasurementTimingConfiguration (where MeasurementTimingConfiguration is a RRC container in XnAP IE Served Cell Information NR) since the IE is used to convey measurement resource configuration between nodes, for example, CSI-RS resource for mobility, SSB index to be measured:

- TS38.331, section 11.2.2 - – MeasurementTimingConfiguration The MeasurementTimingConfiguration message is used to convey assistance information for measurement timing. Direction: en-gNB to eNB, eNB to en-gNB, gNB to gNB, ng-eNB to gNB, gNB to ng-eNB, ng-eNB to ng-eNB, gNB DU to gNB CU, and gNB CU to gNB DU. MeasurementTimingConfiguration message -- ASN1START -- TAG-MEASUREMENT-TIMING-CONFIGURATION-START MeasurementTimingConfiguration ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ measTimingConf MeasurementTimingConfiguration-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } MeasurementTimingConfiguration-IEs ::= SEQUENCE { measTiming MeasTimingList OPTIONAL, nonCriticalExtension MeasurementTimingConfiguration-v1550-IEs OPTIONAL } MeasurementTimingConfiguration-v1550-IEs ::= SEQUENCE { campOnFirstSSB BOOLEAN, psCellOnlyOnFirstSSB BOOLEAN, nonCriticalExtension MeasurementTimingConfiguration-v1610-IEs OPTIONAL } MeasurementTimingConfiguration-v1610-IEs ::= SEQUENCE { csi-RS-Config-r16 SEQUENCE { csi-RS-SubcarrierSpacing-r16 SubcarrierSpacing, csi-RS-CellMobility-r16 CSI-RS-CellMobility, refSSBFreq-r16 ARFCN-ValueNR }, nonCriticalExtension SEQUENCE {} OPTIONAL } MeasTimingList ::= SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF MeasTiming MeasTiming ::= SEQUENCE { frequencyAndTiming SEQUENCE { carrierFreq ARFCN-ValueNR, ssbSubcarrierSpacing SubcarrierSpacing, ssb-MeasurementTimingConfiguration SSB-MTC, ss-RSSI-Measurement SS-RSSI-Measurement OPTIONAL } OPTIONAL, ..., [[ ssb-ToMeasure SSB-ToMeasure OPTIONAL, physCellId PhysCellId OPTIONAL ]] } -- TAG-MEASUREMENT-TIMING-CONFIGURATION-STOP -- ASN1STOP

Proposal 4: For gNB-to-gNB CLI measurement, support to include SSB and/or periodic NZP CSI-RS configuration into RAN2 INM MeasurementTimingConfiguration.

- Conclusion

In this contribution, we propose the following observation and proposals:

UE-to-UE CLI measurement

Proposal 1: For UE-to-UE L1 CLI measurement, RAN2 confirms that L1 CLI measurement does not need measurement gap.

Proposal 2: For UE-to-UE L1 CLI measurement, RAN2 supports victim UE to measure the L1 CLI resource in the frequency overlapping part with the DL active BWP.

Proposal 3: RAN2 to support that, in EN-DC and NGEN-DC, only the SN can configure L1 CLI measurements. In NE-DC, only the MN can configure L1 CLI measurements. In NR-DC, both the MN and the SN can configure L1 CLI measurements.

gNB-to-gNB CLI measurement

Proposal 4: For gNB-to-gNB CLI measurement, support to include SSB and/or periodic NZP CSI-RS configuration into RAN2 INM MeasurementTimingConfiguration.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06

- R1-2401678, RAN1#116, Final Summary on SBFD random access operation, CMCC, 2024-02

- TS37.340-i20, ‘Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage 2’, Release 18, 2024-06