3GPP TSG-RAN WG2 Meeting #127	 					     R2-2406796

Maastricht, Netherlands, August 19-23, 2024

Source:	Continental Automotive

Title:                       Further Discussion on AI based Measurement Event Prediction

Agenda Item:         	8.3.3

Document for:	Discussion and Decision

# Introduction

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell beam-level measurement prediction for L3 mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement event prediction (UE sided model [RAN2].

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity trade-offs [RAN2]

NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2.

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

In RAN #103 meeting, Study on AI/ML for mobility was approved. The objectives of the study were to evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects from RAN2 perspective [1]:

- For cell level measurement prediction model, at least consider the following cases:

  - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

  - Case 2: To directly predict cell level results based on cell level results.

  - Case 3: To directly predict cell level results based on beam level results

- We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements.

- For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level.   As baseline we will focus on pure temporal prediction.

- The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction：

  - Spatial-domain prediction： RSRP difference to the actual measurement

  - Temporal prediction：RSRP difference to the actual measurement reduction rate as one KPI

- As a first step we will focus on measurement prediction accuracy.  FFS whether and what system level performance evaluation is needed.

Further in the RAN2 #125-bis the following agreement related to RRM Measurement prediction were made:

In this contribution, we will discuss in detail the use-cases for AI based RRM event prediction which is currently being discussed in the study item.

# Discussion

###### 2.1 Motivation

In NR (5G), beamforming is the foundational technology. The UE assesses multiple beams within a cell, and the average measurement results determine the cell quality. The NR measurement model is outlined as follows (extracted from [2]):

Fig: High-level measurement model

As stated in the SID justification, managing proper thresholds for the current L3 mobility scheme can be problematic in situations with high UE mobility, high-density areas, or future critical services. This challenge can cause unintended handover failures, disrupting data transmission and affecting user experience. Currently, L3 mobility relies on event-based measurement reports for handovers. By utilizing AI/ML algorithms, the UE could predict measurement events directly, providing the network with advance guidance. This approach would improve handover performance and reduce the measurement burden on the UE.

###### 2.2 Use-cases for cell-level measurement event prediction

According to TS38.331, the current events A1-A5 are utilized to evaluate the measurements of serving and intra/inter frequency neighbour cells, triggering the corresponding measurement reports. During the RAN2#125 bis meeting, it was agreed to use the A3 event as a baseline. Since different measurement events allow the network to configure the UE in various deployment scenarios, supporting the prediction of all events is reasonable.

If the UE can predict measurement events in advance and report them to the gNB, it will enable the network to prepare and execute handovers at more appropriate times. Therefore, supporting temporal domain measurement events A1-A5 is crucial.

In addition to temporal-domain measurement event prediction, considering spatial-domain and frequency-domain measurement event prediction is also important. By predicting some extra intra-frequency or inter-frequency neighbour cells, the corresponding measurement event can be triggered with minimal actual measurements performed by the UE. This reduced measurement can save UE power consumption and minimize potential measurement gaps that might impact UE throughput.

Proposal 1: RAN2 shall support all measurement events which are A1-A5 prediction for frequency-domain, temporal-domain, and spatial-domain.

Additionally, the study item primarily focuses on enhancing handover (HO) performance. Compared to current measurements, the performance enhancement from AI/ML mainly arises from predictions in the temporal domain. Thus, we recommend prioritizing temporal-domain predictions.

#### 2.3 Cell-level measurement configuration

For measurement events prediction, it was agreed to study at least the evaluation of measurement events based on RRM measurement prediction results. Given this consensus, further discussion is needed to determine the detailed model inputs. These inputs could include the measured L1/L3 RSRP of SSB/CSI-RS and prediction configurations for various use cases, such as time windows and pattern information like RRM measurement prediction.

It is yet to be decided whether event parameters, including thresholds, hysteresis, and time-to-trigger, should be fixed inputs or dynamically chosen by the AI/ML model. If the latter approach is adopted, different measurement results with various configured event-related parameters will need to be collected as training data, increasing the complexity of data collection for model training. Alternatively, thresholds and hysteresis could be fixed, with the model predicting how the radio quality of the serving and neighboring cells would change, allowing it to calculate the appropriate TTT.

Input

For study evaluation, it is suggested to start with fixed event-related parameters. The model output for measurement event prediction could include the predicted triggered event, the triggering cells with predicted L3 RSRP, and the predicted event occurrence time.

Output

Prediction criteria

AI model

Events

RRC signaling can be utilized to configure the model inputs and outputs mentioned above. The predicted neighbor cells that trigger measurement events may differ from the measured cells. Consequently, both the measured and predicted sets of cells need to be configured from the network to the UE. The UE may report the confidence/probability information to the network.

The existing L3 signaling measurement report can serve as a baseline. Additionally, the gNB should be explicitly informed whether the measurement event was triggered by a prediction or by actual measurement.

Proposal 2: The following input and output for measurement event prediction should be considered:

Input: Measured RSRP/RSRQ for reference signals (i.e., CSI-RS/SSB), L3 measurements.

Prediction criteria: Events to be predicted, time instances, cell level, beam level, etc.

Events: Threshold, hysteresis, time to trigger, etc.

Output: Predicted triggered event, L3 RSRP, event triggering time, probability of an event, etc.

Proposal 3: RRC signaling should be considered for configuration of model input and output. Measurement report should support the prediction reporting.

# Conclusion

In this contribution, we discussed a few items that could be considered for further discussion related to AI/ML for mobility.

Proposal 1: RAN2 shall support all measurement events which are A1-A5 prediction for frequency-domain, temporal-domain, and spatial-domain.

Proposal 2: The following input and output for measurement event prediction should be considered:

Input: Measured RSRP/RSRQ for reference signals (i.e., CSI-RS/SSB), L3 measurements.

Prediction criteria: Events to be predicted, time instances, cell level, beam level, etc.

Events: Threshold, hysteresis, time to trigger, etc.

Output: Predicted triggered event, L3 RSRP, event triggering time, probability of an event, etc.

Proposal 3: RRC signaling should be considered for configuration of model input and output. Measurement report should support the prediction reporting.

# References

[1] RP-240082 Revised SID on AIML for mobility in NR.

[2] 3GPP TS 38.300

[3] R2-2404002 Report of 3GPP TSG RAN WG2 meeting #125-bis