3GPP TSG-RAN WG2 Meeting #127	R2-2406797

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda item:	8.7.4

Source: 	Huawei, HiSilicon

Title: 	Delay-aware scheduling enhancements

Document for:	Discussion and Decision

# 1. Introduction

In RAN2#126, RAN2 discussed the UL scheduling enhancements using delay/deadline information to improve XR capacity for UL and agreed the following:

- LCP enhancements: Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). - DSR enhancements: Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In this contribution, we will further discuss UL scheduling enhancements issues with respect of LCP enhancements and DSR enhancements.

# 2. Discussion

## 2.1 LCP enhancements

One issue identified in the last meeting is that even if the gNB is aware of the delay-critical data and allocates a UL grant to the UE, as long as there is another selected LCH whose LCH priority is higher than the LCH containing the delay-critical data as shown in Fig. 1, the allocated grant may be occupied by the data from the higher priority LCH, making the delay-critical data further delayed.

Fig.1 Example of data status for multiple LCHs.

To address the above issue, RAN2#126 agreed the following:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

In the subsequent sections, we discuss further on the enhanced LCP restriction and also the FFS related to LCP prioritisation.

### 2.1.1 Enhanced LCP restrictions/LCH selection

It has already been agreed to enhance the LCH prioritization to prioritize the transmission of the delay-critical data. More specifically, the agreement enables the UE to use an additional configured LCH priority which is higher than the default configured one for a LCH with delay-critical data, so that the LCH may be allocated with more resource during LCP. However, the transmission of the delay-critical data may still be obstructed due to the LCP restriction mechanism. For a given UL grant, only the LCHs satisfying the restrictions can be selected for resource allocation. The LCHs with delay-critical data may not satisfy the LCH restriction resulting in the delay critical data being further delayed.

The legacy LCP restrictions can be enhanced so that when the gNB allocates a UL grant to the UE and the UE has delay-critical data in the buffer to transmit, the UE can also be configured to select the LCHs with delay-critical data during LCH selection in LCP.

To realize the LCP restrictions/LCH selection enhancement, the following two approaches can be considered:

- Approach (i) introduce a new LCP restriction:

In this approach, a new LCP restriction rule is introduced, which includes restriction specially for delay-critical data. For example, NW can configure a separate restriction related to delay-critical data on top of the legacy restriction(s), the separate restriction limits that UE should first select LCH with delay-critical data upon reception of an UL grant. Specifically, if UE is configured to apply the new LCP restrictions rule, then upon reception of an UL grant, if there is pending delay-critical data, UE should first select LCH(s) with delay critical data that satisfies the current LCP restrictions.

RAN2#126 agreed that “The solutions should not disallow non-delay critical data from using an UL grant.” For approach (i), to avoid the case where LCHs with only non-delay critical data cannot use the UL grant and also LCHs with delay critical data that do not fulfil the LCP restriction cannot use the UL grant, a second stage of resource allocation can be performed if there are any remaining UL resources after the usage by the data from LCHs with delay-critical data. Specifically, upon reception of an UL grant, UE should first perform a first stage of resource allocation: only LCH(s) with delay-critical data are selected based on the above enhanced LCP restrictions, and UE performs resource allocation for the selected LCH(s) with delay-critical data. After the first stage of resource allocation, if there is any remaining resource, UE performs a second stage of resource allocation: same as in legacy, UE performs LCH(s) selection and resource allocation for the selected LCH(s).

- Approach (ii) relax some legacy LCP restrictions to allow LCH(s) with delay-critical data to be selected:

One way to relax such mapping restriction is to perform an RRC reconfiguration by the network to remove such restriction when there is delay critical data.  However, RRC reconfiguration may not be dynamic enough and thus will not be useful in such scenario.  Another way is to always allow all the LCHs that may have delay critical data to be configured with no LCP restriction.  But this may not be desired when there is no delay critical data in those LCHs while some of the CG and DG are allocated for certain services.

In this approach, the legacy LCP restrictions can be relaxed if there is pending delay-critical data in an LCH. More specifically, to ensure LCH(s) with delay-critical data will be selected for resource allocation, UE can be configured via RRC to relax some or all the legacy LCP restrictions on LCH(s) with delay-critical data. Based on such relaxation, the LCH(s) with delay-critical data can be selected and thus will be allocated with resources.

Observation 1: Delay critical data may be in an LCH which is not mapped to a particular DG and CG and this may result in delay critical data being delayed further if the available DG and CG are not available for the LCHs where the delay critical data is located.

Proposal 1: RAN2 supports enhancing LCP restriction to resolve the issue that delay critical data is not able to use current available DG or CG resource.

Observation 2: For LCP restriction enhancements, the following two semi-static approaches can be considered to solve the issue in Observation 1:

- Introduce a new LCP restriction on top of the legacy LCP restrictions to indicate the MAC entity to prioritize selection of the LCHs with delay-critical data for LCP. Other LCHs should be allowed to use the remaining resource, if there is any.

- Relax the legacy LCP restrictions during LCH selection to allow LCH(s) with delay-critical data to be selected.

Proposal 2: To achieve LCP restrictions enhancements, the following two semi-static alternatives can be considered:

- Introduce a new LCP restriction on top of the legacy LCP restrictions to indicate the MAC entity to prioritize selection of the LCHs with delay-critical data for LCP. Other LCHs should be allowed to use the remaining resource, if there is any.

- Relax the legacy LCP restrictions during LCH selection to allow LCH(s) with delay-critical data to be selected.

### 2.1.2 FFSes in Enhanced LCH prioritization

To enhance LCP prioritisation, RAN2#126 agreed the following:

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

In existing LCP procedure, UE performs resource allocation at an LCH granularity. For resource allocation, RAN2#126 also agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions”. Therefore, it is obvious that all the data within the same LCH uses the same priority.

Observation 3: At a specific time, data within the same LCH should be treated using the same LCH priority since LCP prioritization within an LCH is not considered.

Considering that intra LCH resource allocation is not considered in R19, and the current LCP procedure is based on LCH granularity, it is more reasonable to apply the additional priority for the whole LCH.

Proposal 3: For LCP prioritisation enhancement, when the additional LCH priority is used, it should be applied to the whole LCH.

If the additional priority is applied to the whole LCH, then all the data within the LCH uses the same additional priority. It is reasonable to use the additional priority when there is delay-critical data in the LCH. However, if the additional priority is still used when there is no delay-critical data, it would be unfair with respect to other LCHs without delay-critical data whose priority level remains the same all the time. For example, default priority of LCH1 is 5, default priority of LCH2 is 3, UE adjusts the priority of LCH1 which has delay-critical data to the additional priority, e.g., 2. If additional priority “2” is still used after all the delay-critical data are assembled, e.g., during the remaining resource stage when the non-delay-critical data of LCH1 still uses the additional priority “2” even though all the delay-critical data are already assembled, it is not fair to LCH2 considering the default priority of LCH2 is higher than that of LCH1.

Observation 4: For LCP prioritisation enhancement, if the additional priority is still used for a LCH when there is no delay-critical data on the LCH at any remaining resource stage, it would be unfair to other LCHs without delay-critical data whose priority level remains the legacy one all the time.

In such case, the issue we need to consider is when to start and stop using the additional priority, e.g., when should the MAC entity adjust the priority for an LCH, and under which case the MAC entity should fall back to the legacy priority. From our understanding, the MAC entity should determine the LCH priority before the resource allocation procedure based on whether there is delay-critical data in the LCH. If there is, the additional priority is used, otherwise, the default priority is used. In addition, considering that the additional priority is used only when there is delay-critical data, it is obvious to fall back to the default priority if there is no delay-critical data in the LCH at deciding on remaining resource.

Proposal 4: For LCP prioritisation enhancement, the MAC entity should determine the LCH priority before resource allocation based on whether there is delay-critical data. If there is, the additional priority is used. Otherwise, the default priority is used.

Proposal 5: For LCP prioritisation enhancement, if there is no delay-critical data remained in the LCHs using the additional LCH priority, the LCH priority should fall back to the default one at deciding on any remaining resource.

With respect of impacts on traffic from SRBs, NW can configure a higher priority for the LCH associated with SRBs. For example, by NW implementation, NW can ennsure that the priority for LCH associated with SRBs is higher than that of delay-critical data, even when the priority of the LCH with delay-critical data has been adapted.

Proposal 6: For LCP prioritisation enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with delay-critical data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs of delay critical data.

### 2.1.3 LCP enhancement threshold

RAN2#126 agreed the following for LCP enhancement:

FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In existing DSR procedure, the legacy DSR threshold is used for UE to trigger and report the delay status for delay-critical data, based on which gNB can have a better knowledge of the delay information of UE’s pending data and determine its scheduling strategy. However, the delay-aware LCP enhancement is for UE to consider the delay information when data assembly, which is essentially different procedure. Therefore, it is better not to couple the two procedure, and a separate threshold should be used.

Proposal 7: DSR and delay-aware LCP enhancement should not be coupled. A separate remaining time threshold should be used for delay-aware LCP enhancement.

### 2.1.4 Intra-LCH prioritization in RLC layer

In [1], the following RLC-AM enhancement is proposed to solve the case where non-delay-critical data is before the delay-critical data. In such case, it is proposed to better prioritize the delay-critical data in RLC over other non-delay-critical data (e.g., retransmission data).

Proposal 1: To complement the delay-based LCP enhancements being pursued by RAN2 for Rel-19, RAN2 should also consider some new prioritization mechanisms for RLC-AM to make sure delay-critical packets can be prioritized over non-delay-critical packets if needed.

However, RAN2#126 agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions”. In addition, such enhancement also leads to much complexity to UE. Specifically, RLC entity determines which RLC PDU is delivered to MAC layer in a first-in and first-out manner. If RLC-AM is enhanced to first deliver delay-critical packets, then RLC entity needs to first determine the remaining time for each packet and then deliver those packets to MAC entity, which complicates the RLC process. Moreover, the case where non-delay-critical data is before the delay-critical data happens when PSI-based discarded is activated, e.g., the non-delay-critical data is not important packet with small remaining time. The PSI-based discard is only activated by NW when there is congestion which should be quite infrequent.

Considering that the intra-LCH scheduling is not in RAN2 scope, and the RLC-AM enhancement for this corner case leads to the RLC complexity, so we do not believe it is needed to perform such intra-LCH prioritization in RLC layer.

Proposal 8: There is no need to perform intra-LCH prioritization in RLC layer.

## 2.2 DSR enhancements

RAN2#126 discussed DSR enhancements related issues “whether to have multiple pairs of remaining time and buffer size in DSR” and agreed the following:

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In existing DSR procedure, DSR is triggered when the remaining time among the data buffered for an LCG becomes below the pre-configured threshold. For DSR enhancement, even though RAN2 agreed to report multiple pairs of remaining time and buffer size for an LCG, the UE can still apply the legacy DSR triggering condition. For example, DSR is triggered using the legacy threshold and condition, but DSR MAC CE is enhanced to include multiple pairs of remaining time and buffer size to provide gNB with finer granularity of UE’s pending data. Therefore, the DSR triggering is not impacted in DSR enhancement.

Proposal 9: For enhanced DSR, one remaining time threshold is enough, and the legacy DSR triggering procedure can be reused.

Another issue which needs to be discussed is what should be included in the enhanced DSR MAC CE. The legacy DSR mechanism only allows the UE to report the remaining time and buffer status of the delay-critical data. If there are also non-delay-critical data buffered for the LCG, it will cause problems if the non-delay-critical data information is not indicated. For example, it may lead to two cases:

- Case (1): non-delay-critical data consumes the resources scheduled for delay-critical data, and making the delay-critical data discarded;

- Case (2): most of the data become delay-critical before transmitting it, leaving less time for the gNB to schedule it and causing more DSRs to be triggered

For case (1): If the transmission of non-delay-critical data has impact on transmission of delay-critical data, but the information is not indicated through any other signaling, e.g., BSR, the gNB may not know the existence of the non-delay-critical data and will not schedule sufficient grant to the UE. The cases where non-delay-critical data has impact on transmission of delay-critical data include the following:

- Case (1.1): non-delay-critical data from other LCHs configured with higher priority impacts the transmission of delay-critical data. Currently, UE reports the delay-critical data information per LCG if DSR is triggered by an LCH, based on which gNB can schedule UL grant for the delay-critical data. However, if there are other non-delay-critical data on LCH(s) with higher priority, and the priority is higher than that of the LCH which triggered the DSR, the delay-critical data transmission may be impacted by these non-delay-critical data. In this case, the non-delay-critical data information from LCH(s) with higher priority can be reported to gNB through enhanced DSR MAC CE, so that gNB can allocate more UL grant and avoid the delay-critical data being discarded.

Observation 5: If the information of non-delay-critical data from other LCHs with higher LCH priority (higher than that of the LCH which triggers the DSR) is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

- Case (1.2): when PSI-based discard is activated, low importance data which is in front of the delay-critical data impacts the transmission of delay-critical data. Currently, when DSR is triggered and PSI-based discard is activated, the non-delay-critical data information in front of the delay-critical data is not included in DSR MAC CE. However, RAN2 agreed in the previous meeting that intra-LCH scheduling is not in RAN2 scope, this means that data within an LCH is assembled in a first-in and first-out manner. Therefore, there is no doubt that non-delay-critical data in front of the delay-critical data will be assembled firstly and consume some UL grant, under which case the delay-critical data transmission may be impacted and even be discarded when there is not enough UL grant. Therefore, the non-delay-critical data information which is in front of the delay-critical data should be reported to gNB through enhanced DSR MAC CE, so that UE can request more UL grant from gNB and avoid the delay-critical data being discarded.

Observation 6: When PSI-based discard is activated, if the information of non-delay-critical data which is in front of the delay-critical data is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

For case (2): Taking UL AR traffic as an example, whose periodicity is 16.67ms while the PSDB is 30ms according to TS 38.838, it is highly possible that the second data burst arrives before the first data burst is completely transmitted. In this case, the arrival of the second data burst will not trigger the regular BSR since it is from the same LCH of the same priority. When the first data burst becomes delay-critical, a DSR is triggered for the LCG. However, the status of the second data burst is not included in the DSR since it is not delay-critical. Upon receiving the DSR, the gNB allocates just sufficient resources to the UE to transmit the first data burst within its remaining time based on the remaining time and the data volume of the first data burst in the DSR. The second data burst will be delayed until its remaining time becomes below the threshold so that another DSR can be triggered to report the status of the second data burst. The above results in making most of the data become delay-critical before transmitting it, leaving less time for the gNB to schedule it and causing more DSRs to be triggered which can otherwise be avoided. If the radio link quality is not sufficiently good or there are no sufficient resources within the remaining time, it is hard to guarantee that the data can be transmitted on time. This case can be solved by UE providing multiple pairs of remaining time and buffer size for the LCG as agreed by RAN2.

Proposal 10: For DSR enhancement, the non-delay-critical data information which has impact on delay-critical data transmission should be included for each LCG.

Proposal 11: The non-delay-critical data information which has impact on delay-critical data transmission for each LCG includes:

- Non-delay-critical data information from other LCHs with higher LCH priority, the priority is higher than that of the LCH which triggers the DSR;

- When PSI-based discard is activated, the data volume of the low importance PDU sets. FFS whether to report the data volume of all the low importance data or only the part that will impact the transmission of delay-critical data.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for scheduling enhancements using the delay/deadline information. The following observations and proposals were made:

LCP restrictions enhancements:

Observation 1: Delay critical data may be in an LCH which is not mapped to a particular DG and CG and this may result in delay critical data being delayed further if the available DG and CG are not available for the LCHs where the delay critical data is located.

Proposal 1: RAN2 supports enhancing LCP restriction to resolve the issue that delay critical data is not able to use current available DG or CG resource.

Observation 2: For LCP restriction enhancements, the following two semi-static approaches can be considered to solve the issue in Observation 1:

- Introduce a new LCP restriction on top of the legacy LCP restrictions to indicate the MAC entity to prioritize selection of the LCHs with delay-critical data for LCP. Other LCHs should be allowed to use the remaining resource, if there is any.

- Relax the legacy LCP restrictions during LCH selection to allow LCH(s) with delay-critical data to be selected.

Proposal 2: To achieve LCP restrictions enhancements, the following two semi-static alternatives can be considered:

- Introduce a new LCP restriction on top of the legacy LCP restrictions to indicate the MAC entity to prioritize selection of the LCHs with delay-critical data for LCP. Other LCHs should be allowed to use the remaining resource, if there is any.

- Relax the legacy LCP restrictions during LCH selection to allow LCH(s) with delay-critical data to be selected.

LCP prioritization enhancements:

Observation 3: At a specific time, data within the same LCH should be treated using the same LCH priority since LCP prioritization within an LCH is not considered.

Proposal 3: For LCP prioritisation enhancement, when the additional LCH priority is used, it should be applied to the whole LCH.

Observation 4: For LCP prioritisation enhancement, if the additional priority is still used for a LCH when there is no delay-critical data on the LCH at any remaining resource stage, it would be unfair to other LCHs without delay-critical data whose priority level remains the legacy one all the time.

Proposal 4: For LCP prioritisation enhancement, the MAC entity should determine the LCH priority before resource allocation based on whether there is delay-critical data. If there is, the additional priority is used. Otherwise, the default priority is used.

Proposal 5: For LCP prioritisation enhancement, if there is no delay-critical data remained in the LCHs using the additional LCH priority, the LCH priority should fall back to the default one at deciding on any remaining resource.

Proposal 6: For LCP prioritisation enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with delay-critical data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs of delay critical data.

LCP enhancement threshold:

Proposal 7: DSR and delay-aware LCP enhancement should not be coupled. A separate remaining time threshold should be used for delay-aware LCP enhancement.

Intra-LCH prioritization in RLC layer

Proposal 8: There is no need to perform intra-LCH prioritization in RLC layer.

DSR enhancements:

Proposal 9: For enhanced DSR, one remaining time threshold is enough, and the legacy DSR triggering procedure can be reused.

Observation 5: If the information of non-delay-critical data from other LCHs with higher LCH priority (higher than that of the LCH which triggers the DSR) is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

Observation 6: When PSI-based discard is activated, if the information of non-delay-critical data which is in front of the delay-critical data is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

Proposal 10: For DSR enhancement, the non-delay-critical data information which has impact on delay-critical data transmission should be included for each LCG.

Proposal 11: The non-delay-critical data information which has impact on delay-critical data transmission for each LCG includes:

- Non-delay-critical data information from other LCHs with higher LCH priority, the priority is higher than that of the LCH which triggers the DSR;

- When PSI-based discard is activated, the data volume of the low importance PDU sets. FFS whether to report the data volume of all the low importance data or only the part that will impact the transmission of delay-critical data.

# 4. Reference

[1]. R2-2404650 Complementary RLC Mechanisms for LCP Enhancements Fukuoka, Japan, 20 – 24 May 2024