3GPP TSG-RAN WG2 #127	R2-2406806

Maastricht, Netherlands, 19th – 23th August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to per-LCH carrier set restriction in LCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current MAC spec version, the gNB provides a parameter sl-AllowedCarriers to UE via RRC to set the allowed sidelink carriers for a logical channel in case of CA PDCP duplication. - sl-AllowedCarriers which sets the allowed sidelink carrier(s) for sidelink transmission. How this parameter affects LCP procedure has been discussed in RAN2 and concluded in RAN2#124 that MAC details: - The per-LCH carrier set restriction is to be indicated from RRC-layer to MAC-layer, for LCP procedure. This means that this parameter will not be directly used by the MAC layer, instead, it is the RRC layer to use this parameter to formulate per-LCH carrier set restriction and indiacte to MAC layer, based on which, the MAC layer performs the LCP. However, this parameter sl-AllowedCarriers is being referred in clause 5.22.1.4.1.1, together with other legacy parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the LCP procedure texts in clause 5.22.1.4.1.1, to remove the reference of this parameter sl-AllowedCarriers. Impact Analysis Impacted 5G architecture options: NR SA This CR is mandatory to implement for UEs and networks supporting Sidelink carrier aggregation. Impacted functionality: Sidelink carrier aggregaration Inter-operability: if the network implements the change according to the CR and the UE does not, there are no inter-operability issues. If the UE implements the change according to the CR and the network does not, there are no inter-operability issues. If one UE implementes the change according to the CR, but not the other UE, there are inter-operability issues, i.e., one UE MAC layer may select the carriers according to this RRC parameter, while another UE MAC layer may just ignore this RRC parameter. Thus, UE behaviors in LCP procedure regarding how to select allowed carriers in case of CA PDCP duplication is inconsistent among UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the changes are not approved, the UE behaviors in LCP procedure regarding how to select allowed carriers in case of CA PDCP duplication is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

<<<<Skipped>>>>

###### 5.22.1.4.1.1	General

The sidelink Logical Channel Prioritization procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of sidelink data by signalling for each logical channel:

-	sl-Priority where an increasing priority value indicates a lower priority level;

-	sl-PrioritisedBitRate which sets the sidelink Prioritized Bit Rate (sPBR);

-	sl-BucketSizeDuration which sets the sidelink Bucket Size Duration (sBSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	sl-configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for sidelink transmission;

-	sl-AllowedCG-List which sets the allowed configured grant(s) for sidelink transmission;

-	sl-HARQ-FeedbackEnabled which sets whether the logical channel is allowed to be multiplexed with logical channel(s) with sl-HARQ-FeedbackEnabled set to enabled or disabled;

The following UE variable is used for the Logical channel prioritization procedure:

-	SBj which is maintained for each logical channel j.

The MAC entity shall initialize SBj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment SBj by the product sPBR × T before every instance of the LCP procedure, where T is the time elapsed since SBj was last incremented;

1>	if the value of SBj is greater than the sidelink bucket size (i.e. sPBR × sBSD):

2>	set SBj to the sidelink bucket size.

NOTE:	The exact moment(s) when the UE updates SBj between LCP procedures is up to UE implementation, as long as SBj is up to date at the time when a grant is processed by LCP.

<<<<Skipped>>>>

End of change