3GPP TSG-RAN WG2 Meeting #127	R2-2406808

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.20.2

Source:	Nokia

Title:	Clarification on the codebook type request in the UE capability enquiry

WID/SID:	NR_MIMO_evo_DL_UL-Core – Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#126, a contribution regarding UE capability enquiry with codebook type request was discussed [1]. In this contribution, we share our perspective on the necessary corrections to the RRC for Rel-18 MIMO in relation to this matter.

# 2	Discussion

The codebookTypeRequest-r16 is supported within UE capability enquiry parameters with the primary aim of reducing the signalling overhead of SupportedCSI-RS-Resource:

codebookTypeRequest-r16        SEQUENCE {

type1-SinglePanel-r16          ENUMERATED {true}                                    OPTIONAL,    -- Need N

type1-MultiPanel-r16           ENUMERATED {true}                                    OPTIONAL,    -- Need N

type2-r16                      ENUMERATED {true}                                    OPTIONAL,    -- Need N

type2-PortSelection-r16        ENUMERATED {true}                                    OPTIONAL     -- Need N

}                                                                                   OPTIONAL,    -- Need N

codebookTypeRequest Only if this field is present, the UE includes SupportedCSI-RS-Resource supported for the codebook type(s) requested within this field (i.e. type I single/multi-panel, type II and type II port selection) into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC. If this field is present and none of the codebook types is requested within this field (i.e. empty field), the UE includes SupportedCSI-RS-Resource supported for all codebook types into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC.

The field description specifies that the UE includes SupportedCSI-RS-Resource for indicated codebook types. However, there is an issue with the clarity regarding which exact codebook type is being referred to in the specification, due to the complex and evolving history of codebooks. This ambiguity can lead to confusion when interpreting the details. Nonetheless, given that this inquiry was introduced in Release 16 as part of TEI16, our understanding is that codebook types in question refer to the one introduced in Release 15.

Proposal 1: RAN2 clarifies that codebookTypeRequest solely refers to the codebook types introduced in Release 15.

Now the problem is that in the later releases from Release 16 –  Release 18, different codebook types are introduced, UE will be confused on how to handle the new codebook types in association with UE capability enquiry parameters, codebookTypeRequest.  In this scenario, our preference is that the UE ignore this codebook types request for codebook types introduced in in Release 16, Release 17, and Release 18. This implies that SupportedCSI-RS-Resource related to codebook types from Release 16/17/18 will always be included.

Proposal 2: UE ignore codebook types request for codebook types introduced in in Release 16, Release 17, and Release 18. This implies that SupportedCSI-RS-Resource related to codebook types from Release 16/17/18 will always be included.

# 3	Conclusion

Proposal 1: RAN2 clarifies that "codebookTypeRequest" solely refers to the codebook types introduced in Release 15.

Proposal 2: UE ignore codebook types request for codebook types introduced in in Release 16, Release 17, and Release 18. This implies that SupportedCSI-RS-Resource related to codebook types from Release 16/17/18 will always be included.

# References

[1]  R2-2405172, Clarification on UE capability enquiry with codebook type request, Samsung